3GPP TSG-RAN WG4 Meeting # 108								    		         R4-2313101

Toulouse, France, 21 – 25 August 2023

Agenda item:			8.26.2

Source:	Samsung

Title:	Simulation calibration assumptions and results for above 10GHz NTN co-existence study

Document for:	Approval

- Introduction

This document is for the comparing calibration assumptions between companies who provided calibration data.

Some issues discussed during RAN4#107 to RAN4#108, those issues should be discussed in this meeting:

- Whether additional FRF = 2 or 3 should be calibrated?

- FθdB=10*log10(2 J1uu2) has been confirmed, it should be double checked by every company who provided calibration results.

- New power control parameter of VSAT UE should be agreed by the meeting for both calibration and co-ex study.

- NTN UE antenna elevation angle is 90 degrees in current results should be confirmed during the meeting.

- Discussion

Calibration simulation scenarios

| No. | Combination | Frequency band | Samsung | Ericsson | Qualcomm | Thales | Huawei | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TN Downlink | 27 GHz | √ | √ | √ | √ | √ | √ |
| 2 | TN Uplink | 27 GHz | √ | √ | √ | √ | √ | √ |
| 3 | TN Downlink | 17 GHz |  | √ | √ | √ | √ | √ |
| 4 | TN Uplink | 17 GHz |  | √ | √ | √ | √ | √ |
| 5 | NTN GEO Downlink VSAT UE | 17 GHz | √ | √ | √ | √ | √ | √ |
| 6 | NTN GEO Uplink VSAT UE | 27 GHz | √ | √ | √ | √ | √ | √ |
| 7 | NTN LEO1200 Downlink VSAT UE | 17 GHz | √ | √ | √ | √ | √ | √ |
| 8 | NTN LEO1200 Uplink VSAT UE | 27 GHz | √ | √ | √ | √ | √ | √ |
| 9 | NTN LEO600 Downlink VSAT UE | 17 GHz | √ | √ | √ | √ | √ | √ |
| 10 | NTN LEO600 Uplink VSAT UE | 27 GHz | √ | √ | √ | √ | √ | √ |
| 11 | NTN GEO Downlink ESIM UE | 17 GHz |  |  |  |  |  |  |
| 12 | NTN GEO Uplink ESIM UE | 27 GHz |  |  |  |  |  |  |
| 13 | NTN LEO1200 Downlink ESIM UE | 17 GHz |  |  |  |  |  |  |
| 14 | NTN LEO1200 Uplink ESIM UE | 27 GHz |  |  |  |  |  |  |
| 15 | NTN LEO600 Downlink ESIM UE | 17 GHz |  |  |  |  |  |  |
| 16 | NTN LEO600 Uplink ESIM UE | 27 GHz |  |  |  |  |  |  |

TN BS and UE ACLR and ACS values

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

| Combination | Number | Samsung | Ericsson | Qualcomm | Thales | Huawei | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FRF | 1 | √ | √ | √ | √ | √ | √ |
| Additional FRF | 2 or 3 |  |  |  |  |  |  |
| Channel bandwidth | 200MHz | √ | √ | √ | √ | √ | √ |

Network layout model

Co-existence between NTN and TN

Cellular cell structure is considered for both NTN and TN network layout.

Coordination System

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z).

Simulation Methodology

Following simulation steps can be used for NTN-TN co-existence study.

- [Generate aggressor and victim networks.

- FRF = 1(Issue 1-3)

- Deployment of TN network (19 cells with wraparound) refers to Annex 2(Issue 2-4).]

- UE associations

- TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss.

- [Deployment of NTN UE refers to Annex 2(Issue 2-4).]

[To simplify the discussion on joint issues 2-3, 2-4 & 3-1, consider following approaches for the deployment of NTN UE and satellite - The Satellite should be generated in the visible area/sky of NTN UE; - NTN UEs point to the satellite accurately or with a deviation provided by satellite vendors; - To simplify the work in calibration phase, NTN UEs point to the satellite accurately without any deviations. - Further discuss the deviation for co-existence simulation, satellite vendors and operators are encouraged to provide a vertical angle value and a deviation value. - The position of the satellite should guarantee that NTN UE vertical angle towards the satellite is with certain value(s) or within the range of [10, 20, 30 or other numbers] degree to 90 degree; - For calibration, use 30 degree and 90 degree instead of the range. - Further discuss the range and/or the value for co-existence simulation. - Horizontal angle of NTN UEs should be calculated based on the satellite position - ]

- Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

- Throughput is computed in the victim systems without considering ACI as below:

- ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821 section 6.1.3 equations. Thus those equations should be used for SINR calculation.

- Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

- RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

[To simplify the simulation of interference from TN to NTN UL in Case 2 and 6, following method can be used. Consider the active TN cells from central NTN beam for the ACI evaluation from TN to NTN UL. The scaling factor is to be discussed and determined if any in next meeting. There is a view that simplifying such coexistence simulation work for Case 2 may even not be required.

- Step 1: to drop NTN UE per beamprint randomly;

- Step 2: to drop N clusters consisting of 57 sectors per beamprint randomly

- Step 3: to calculate the total ACI per beam to NTN UL by following scaling factor:

where:

active_TN = active_factor*round (the area per beam/the area of 57 sectors)

active_factor = 20% (or lower, particularly for urban scenarios)]

- Step 4: to calculate the total ACI from all beams (e.g. M=7) for NTN:

Simulation parameters

NTN parameters

Satellite parameters

Set-1 satellite parameters are shown in Table 2.3.1-2 according to TR 38.821.

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 2.3.1-1 NRB configuration per BandWidth size and SCS

| Configuration FR2 Ka-band | NRB (200MHz BW) |
| --- | --- |
| SCS 120 kHz | 132 |

Table 2.3.1-2 Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | √ | √ | √ | √ | √ | √ |
| Satellite EIRP density |  | Ka-band (i.e. 20 GHz for DL) | 40 dBW/MHz | √ | √ | √ | √ | √ | √ |
| Satellite max TX power in dBm | BW (MHz) |  | 200 | √ | √ | √ | √ | √ | √ |
|  | SCS kHz |  | 120 | √ | √ | √ | √ | √ | √ |
|  | NRB |  | 132 | √ | √ | √ | √ | √ | √ |
| Satellite Tx max Gain |  |  | 58.5 dBi | √ | √ | √ | √ | √ | √ |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.1765 deg | √ | √ | √ | √ | √ | √ |
| Satellite beam diameter |  |  | 110 km | √ | √ | √ | √ | √ | √ |
| G/T |  | Ka-band (i.e. 30 GHz for UL) | 28 dB K-1 | √ | √ | √ | √ | √ | Using equation with gain and NF |
| Satellite Rx max Gain |  |  | 58.5 dBi | √ | √ | √ |  | √ | √ |
| G_Rx (dBi) |  |  | 58.5 | √ | √ | √ | √ | √ | √ |
| NF (dB) |  |  | 5.9 | √ | √ | √ However, 3.5 dB should be used | √ | √ | √ |

| Satellite orbit |  |  | LEO-1200 |  | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 1200 km |  | √ | √ | √ | √ | √ | √ |
| Satellite EIRP density |  | Ka-band (i.e. 20 GHz for DL) | 10 dBW/MHz |  | √ | √ | √ | √ | √ | √ |
| Satellite max TX power in dBm | BW (MHz) |  | 200 |  | √ | √ | √ | √ | √ | √ |
|  | SCS kHz |  | 120 |  | √ | √ | √ | √ | √ | √ |
|  | NRB |  | 132 |  | √ | √ | √ | √ | √ | √ |
| Satellite Tx max Gain |  |  | 38.5 dBi |  | √ | √ | √ | √ | √ | √ |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 1.7647 deg |  | √ | √ | √ | √ | √ | √ |
| Satellite beam diameter |  |  | 40 km |  | √ | √ | √ | √ | √ | √ |
| G/T |  | Ka-band (i.e. 30 GHz for UL) | 13 dB K-1 |  | √ | √ | G/T recomputed from 7.976 = 5.9 dB NF! | √ | G/T recomputed from 7.976 = 5.9 dB NF | Using equation with gain and NF |
| Satellite Rx max Gain |  |  | 38.5 dBi |  | √ | √ | √ | √ | √ | √ |
| G_Rx (dBi) |  |  | 38.5 |  | √ | √ | √ | √ | √ | √ |
| NF (dB) |  |  | 5.9 |  | √ | √ | √ However, 3.5 dB should be used | √ | √ | √ |

| Satellite orbit |  |  | LEO-600 |  | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 600 km |  | √ | √ | √ | √ | √ | √ |
| Satellite EIRP density |  | Ka-band (i.e. 20 GHz for DL) | 4 dBW/MHz |  | √ | √ | √ | √ | √ | √ |
| Satellite max TX power in dBm | BW (MHz) |  | 200 |  | √ | √ | √ | √ | √ | √ |
|  | SCS kHz |  | 120 |  | √ | √ | √ | √ | √ | √ |
|  | NRB |  | 132 |  | √ | √ | √ | √ | √ | √ |
| Satellite Tx max Gain |  |  | 38.5 dBi |  | √ | √ | √ | √ | √ | √ |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 1.7647 deg |  | √ | √ | √ | √ | √ | √ |
| Satellite beam diameter |  |  | 20 km |  | √ | √ | √ | √ | √ | √ |
| G/T |  | Ka-band (i.e. 30 GHz for UL) | 13 dB K-1 |  | √ | √ | G/T recomputed from 7.976 = 5.9 dB NF! | √ | G/T recomputed from 7.976 = 5.9 dB NF | Using equation with gain and NF |
| Satellite Rx max Gain |  |  | 38.5 dBi |  | √ | √ | √ | √ | √ | √ |
| G_Rx (dBi) |  |  | 38.5 |  | √ | √ | √ | √ | √ | √ |
| NF (dB) |  |  | 5.9 |  | √ | √ | √ However, 3.5 dB should be used | √ | √ | √ |

Table 2.3-2 Other parameters for NTN

| Parameters | NTN | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The number of active UE (UL) | 10 UEs | √ | √ | √ | √ | √ | √ |
| The number of active UE (DL) | 1 UEs | √ | √ | √ | √ | √ | √ |
| Traffic model | Full buffer | √ | √ | √ | √ | √ | √ |
| DL power control | NO | √ | √ | √ | √ | √ | √ |
| UL power control | See Session 2.7.2 | √ | √ | √ | √ | √ | √ |
| NTN satellite Noise figure in dB | See Table 2.3.1-3 | √ | √ | √ | √ | √ | √ |
| Handover margin | 3dB | √ | √ | √ | √ | √ | √ |

UE parameters

UE parameters are shown in Table 2.3.1-4

[Table 2.3.1-4 UE characteristics for system level simulations

| Characteristics | fixed VSAT | L-ESIM |
| --- | --- | --- |
| Frequency band | 27GHz | 27GHz |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | circular |
| Rx Antenna gain | 39.7 dBi | TBD dBi per element |
| Antenna temperature | 150 K | TBD K |
| Noise figure | 4dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | TBD dBi per element |
| - For calibration, the number of active NTN UE (UL) is 10 UEs with divided RBs per UE by channel bandwidth. - For calibration, NTN UE antenna elevation angle is 30 and 90 degree. - For calibration, NTN UE NF is 4dB basic on RAN4#109 agreements. |  |  |
| NOTE 1: FFS whether and how to consider phased array antenna pattern, not preclude to consider phased array antenna pattern in the future if needed. |  |  |

]

TN parameters

Table 2.3.2-1 Simulation assumptions of TN NR

|  |  | Samsung | Ericsson | Qualcomm | Thales | Huawei | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency in GHz | 27 | √ | √ | √ | 17&27 | 17&27 | 17&27 |
| Size of each nominal channel BW in MHz | 200 | √ | √ | √ | √ | √ | √ |
| Transmission bandwidth in MHz | 190.08 | √ | √ | √ | √ | √ | √ |
| Environment | Urban macro | √ | √ | √ | √ | √ | √ |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | √ | √ | √ | √ | √ | √ |
| Inter-site distance in meter | 300m | √ | √ | √ | √ | √ | √ |
| UE number per cell | Same as the number of BS beam | √ | √ | √ | 1 |  | √ |
| System loading and activity | 20% 100% | √ | √ | 100% Why 20%?, the 20% is considered in active number of clusters inside the NTN beam | 100 % | √ | √ |
| Network location | See Annex 2 | √ | √ | √ | √ | √ | √ |
| DL subcarrier spacing | 120kHz | √ | √ | √ | √ | √ | √ |
| UL | OFDMA | √ | √ | √ | √ | √ | √ |
| DL power control | No | √ | √ | √ | √ | √ | √ |
| UL power control | See Section 2.7.1 | √ | √ | √ | √ | √ | √ |
| Frequency reuse | 1 | √ | √ | √ | √ | √ | √ |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam | √ | √ | √ | 1 | √ | √ |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam | √ | √ | √ | 1 | √ | √ |
| BS height | 25 | √ | √ | √ | √ | √ | √ |
| UE antenna height in meter | 1.5 | √ | √ | √ | √ | √ | √ |
| UE TX power in dBm | -40 to 23 | √ | √ | √ | √ | √ | √ |
| UE peak EIRP | 22.4 dBm(13.4 tx power +9dBi gain) | √ | √ | √ I think you mean Tx power not EIRP? | 23 | √ | 23 Tx power, not used |
| UE antenna element gain in dBi | 0 -> 5 | √ | √ | Should be 5 dBi (why 0, below in 2.4.2.2, 5 dBi is used with BF 2x2) | Should it be 0 or 5 dBi per element? | Max element gain 5dbi M2101 | 5 |
| Building penetration loss | In path loss model, TR 38.901 | √ | √ | √ | √ | √ | √ |
| Outdoor UE percentage | 100% |  |  |  |  |  | √ |
| Cell selection margin in dB | 3 | √ | √ | √ | √ | √ | √ |
| BS-MS min distance in meters | 35 | √ | √ | √ | 0 | √ | √ |
| BS noise figure in dB | 11 -> 10 | √ | 10 | 10 | 10 | √ | 10 |
| UE noise figure in dB | 11 -> 10 | √ | 10 | 10 | 10 | √ | 10 |
| BS-UE path-loss model | TR 38.901 | √ | √ |  | √ | √ | √ |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. | √ | √ | √ | √ | √ | √ |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 | √ | √ | √ | √ | √ | √ |
| Link-level performance model | See Section 2.10 | √ | √ | √ | √ | √ | √ |
| UE distribution | Uniform | √ | √ | √ | √ | √ | √ |
| Evaluation metrics | See Section 2.9 | √ | √ | √ | √ | √ | √ |

Table 2.3.2-3 ACLR/ACS for TN (30GHz)

|  |  | NR | Samsung | Ericsson | Qualcomm | Thales | Huawei | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BS | ACLR | 28dB | √ | √ | √ | √ | √ | √ |
|  | ACS | 24dB | √ | √ | √ | √ | √ | √ |
| UE | ACLR | 17dB | √ | √ | √ | √ | √ | √ |
|  | ACS | 22dB | √ | √ | √ | √ | √ | √ |

Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.4.1 in TR 38.811.

Satellite antenna pattern

Parabolic antenna pattern for SAN:

  - Use the following parabolic antenna model for SAN

Option 1: FθdB=10*log10(2 J1uu)

Option 2: FθdB=10*log10(2 J1uu2)

With:

- Jix  is the Bessel function of first type and ith order with argument x

-  is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

-  : Wavelength

- FFS whether and how to consider phased array antenna pattern

- Not preclude to consider phased array antenna pattern in the future if needed

Parabolic antenna pattern for NTN UE:

  - Use the following parabolic antenna model for NTN UE

Option 1: FθdB=10*log10(2 J1uu)

Option 2: FθdB=10*log10(2 J1uu2)

With:

- Jix  is the Bessel function of first type and ith order with argument x

-  is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

-  : Wavelength

| Samsung | Ericsson | Thales | CATT |  |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked Fθ is amplitude |  |

107 agreement:

ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2)

- with ABS [degree]=180/pi x ABS[rad] and

- with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern.

Satellite and UE beam forming pattern

The following table is agreed for the beam layout definition for a single satellite simulation in S-Band.

Table 2.4.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |  |  |
| --- | --- | --- | --- |
| Number of beams | Baseline: 7-beam layout (i.e. 6 co-frequency beams surrounding the central beam) |  |  |
| UV plane illustration (extracted from [19]) |  |  |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |  |  |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern. |  |  |
| Central beam bore sight direction definition | Central beam center is considered at nadir point Interested companies can bring analysis and results for other values. |  |  |
|  | Option 1: FRF=1 | Option 2: FRF=3 | Option 3: FRF=2 |
| Polarization re-use | Option 1: Disable Option 2: Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |  |  |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |  |  |
| UE distribution | The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |  |  |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |  |  |
| UE orientation | Ideal Tracking serving beam |  |  |
| UE attachment | RSRP |  |  |
| NOTE 1: Typical impairment values (additional frequency error, SNR loss) due to the feeder link except for delay can be considered to be negligible. When available, specific values can be considered in the evaluation and should be reported. NOTE 2: For the calibration purpose, the ionospheric scintillation loss shall be considered equal to zero (i.e., the UEs are located between 20 and 60 degrees of latitude). |  |  |  |

| Samsung | Ericsson | Thales | Qualcomm | CATT |
| --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked |

TN BS and UE antenna and beam forming pattern modelling

BS antenna

|  | Macro urban | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna pattern | TR 38.803 | √ | √ | √ | √ | √ | √ |
| Element gain GE,max (dBi) | 5.5 | √ | √ | √ | √ | √ | √ |
| Horizontal j3dB /vertical q3dB 3 dB beam width of single element (degree) | 90º for H 90º for V | √ | √ | √ | √ | √ | √ |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V | √ | √ | √ | √ | √ | √ |
| Side lobe suppression SLAv (dB) | 30 | √ | √ | √ | √ | √ | √ |
| Antenna polarization | Linear ±45º | √ | √ | √ | √ | √ | √ |
| Antenna array configuration (Row × Column) | 16 × 8 elements | √ | √ | √ | √ | √ | √ |
| Horizontal/Vertical radiating element spacing | dh = 0.5 dv = 0.5 | √ | √ | √ | √ | √ | √ |
| Array Ohmic loss LE (dB) NOTE 1 | 2 | √ | √ | √ | √ | (how it is implemented, the value of 5.5 dBi element should already consider this 2 dB, so we don’t need to consider it) | Not used |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 | √ | √ | √ | √ | √ | √ |
| Mechanical downtilt (degrees) | 10 | √ | √ | √ | √ | √ | √ |
| NOTE 1: The value of 5.5 dBi element should already consider this 2 dB, we don’t need to consider it in the simulation. |  |  |  |  |  |  |  |

Table 2.4.2-1 BS antenna modelling for Urban macro scenario

| Parameter | Values | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=30dB | checked | Checked | Checked | Checked | Checked | Checked |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=30dB |  |  |  |  |  |  |
| Combining method for 3D antenna element pattern (dB) |  |  |  |  |  |  |  |
| Maximum directional gain of an antenna element GE,max | 5.5 |  |  |  |  |  |  |
| (Mg, Ng, M, N, P) note | For 30GHz: (1, 1, 8, 16, 2) |  |  |  |  |  |  |
| (dv, dh) | (0.5λ, 0.5λ) |  |  |  |  |  |  |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |  |  |  |  |  |  |

UE antenna

Refer to TR 38.803.

Table 2.4.2-2 Antenna parameters for 30GHz

| Parameter | Values | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Antenna element vertical radiation pattern (dB) |  | Checked | Checked | Checked. Note the 5 dBi UE element gain here vs 0 dBi earlier. | Checked. Note the 5 dBi UE max element gain , | Checked | Checked |
| Antenna element horizontal radiation pattern (dB) |  |  |  |  |  |  |  |
| Combining method for 3D antenna element pattern (dB) |  |  |  |  |  |  |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |  |  |  |  |  |  |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |  |  |  |  |  |  |
| (dv, dh) | (0.5λ, 0.5λ) |  |  |  |  |  |  |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |  |  |  |  |  |  |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |  |  |  |  |  |  |

ACIR model

- Flat ACIR modelling used [for co-existence for one UE]

- To use 1 UE for both TN and NTN in the DL and 10 UEs for NTN and 1 UEs for TN in the UL.

  - [10 UE should use the ACIR modelling step 3 in 36.942]

Propagation model

Following table is to be used for calibration.

| Link |  | Propagation model | Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TN BS to TN UE |  | UMa as in 3GPP TR 38.803 | √ | √ | √ | √ | √ | √ |
| Satellite to VSAT/ESIM |  | 3GPP TR 38.811(Urban scenario) | √ | √ | √ | √ | √ (Urban is used) | √ |
|  | Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. [atmospheric losses = 0dB for calibration] [atmospheric losses use 1dB or 2dB for RAN4 study?] |  |  |  |  |  |  |  |

Transmission power control model

TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

| Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- |
| Checked | Checked | Checked. | Checked | Checked | Checked |

NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

| Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked | Checked |

DL TPC

For downlink scenario, no power control scheme is applied.

Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

| Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked | Checked |

Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

| Samsung | Ericsson | Thales | Huawei | Qualcomm | CATT |
| --- | --- | --- | --- | --- | --- |
| Checked | Checked | Checked | Checked | Checked | Checked |