3GPP TSG-RAN WG4 Meeting #108	R4-2313109

Toulouse, France, 21 – 25 Aug, 2023

Source:	ZTE Corporation

Title:	Discussion on Case 2 RRM requirements

Agenda Item:	8.9.2.3

Document for:	Approval

- Introduction

In recent RAN plenary 100 meeting, a revised WID was approved in [1] for Rel-18 to revise the objectives of enhancement of pre-configured MGs, multiple concurrent MGs and NCSG as follows:

The following objectives are considered in this WI: - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Define requirements only for the following two cases for UE configured with: - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: This WID does not include any inter-working with MUSIM gaps - Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc. - Inter-RAT measurements without gaps [RAN4] - Inter-RAT NR measurements - Inter-RAT LTE measurement

Based on the revised WID achieved in RAN plenary 100, it seems that the scope for this WI is more clear and limited, the following points should be noticed:

- NTN MG is not considered in this WI.

- NCSG is not considered in Case 1, and pre-configured MG is not considered in Case 2. This means the big joint between pre-configured MG, NCSG and concurrent MGs is not considered in this WI.

- MUSIM gap is not considered in this WI.

So for the future discussion, all these points should be seemed as premise. So that RAN4 can put efforts on the issues within the revised scope.

- Around Case 2 in the WI, the following agreements were achieved in [2] during107 meeting:

Sub-topic 4-1: Gap combinations Issue 4-1-1: [Case 2] Detail measurement gaps combinations for UE supporting per-FR gap < Agreement >: - This issue is related to open issues 2-1-1 and 2-1-2. If the outcome of the related issues is to support Type-1 then the notes of Type-1 can be discussed directly in the CR. Sub-topic 4-2: Parallel measurements for NCSG + NCSG Issue 4-2-1: [Case 2] Whether to consider parallel measurements upon gap collision < Agreements from online session >: - Do not support parallel measurements upon NCSGs collision in Rel-18 MG Enhancements WI. Issue 4-2-2: [Case 2] Whether to support parallel measurements in the following scenarios for two NCSG < Agreement >: - Follow agreement from issue 4-2-1. Issue 4-2-3: [Case 2] Whether to support parallel measurements in a general way for two NCSGs < Agreement >: - Follow agreement from issue 4-2-1. Sub-topic 4-3: collision handling Issue 4-3-1: [Case 2] Potential changes to UE behaviour upon gap collision < Agreement >: - Upon gap collision, RAN4 not to consider additional enhancements on top of the existing agreements. Sub-topic 4-4: Other Rel-17 rules to be revisited Issue 4-4-1: [Case 2] Potential changes for NCSG upon SCell activation < Agreements from meeting RAN4#106-bis-e >: - UE behavior for deactivated SCell measurements with NCSG in Case 2 is FFS - Option 1: Legacy UE behavior (i.e. UE measures the deactivated SCell outside of MG) - Option 2: When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped. < Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only. < Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG?

- After the previous meetings, around Case 2, multiple typical issues such as parallel measurements for 2 NCSGs, collision handling have been concluded. Now only the Rel-17 related rules revision and the requirements need to be further discussed. In this document, we provide some further analysis on these remaining issues.

- Other Rel-17 rules to be revisited

- Requirements

- Discussion

2.1 Other R17 rules to be revisited

Potential changes for NCSG upon SCell activation

In Rel-17, the deactivated SCell is agreed to be measured within NCSG instead of outside gap if NW configures NCSG. However, when NW triggers SCell activation, the related SSB of the deactivated SCell’s MO to be measured may be outside active BWP. In such case, how to address the measurement of the deactivated SCell’s MO, which should be identified.

After some discussion during last meeting, the following two options were agreed as candidates in [2]:

< Agreements from online session >: - Option 1: - UE measures the deactivated SCell outside of MG - Option 2: - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG. - FFS whether a new indication shall be introduced enable support of NCSG for deactivated SCell only.

Before exploring the solution in R18, it is important to reach consensus on the R17 UE behaviors. Fortunately the following alignment between companies are achieved in last meeting:

< Agreement >: - Align the understanding of Rel-17 UE behaviours - Only up to 1 NCSG can be configured. All activated Scell MOs are implicitly associated to the NCSG - In the dynamic UE capability signalling, there is no separate indication for activated/deactivated serving cells. This implies UE only indicate the capability if it supports all scenarios, including - deactivated Scell - activated Scell but SSB not in active BWP - Understanding to be clarified: - Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG?

Firstly, regarding the content to be clarified with respect to R17 UE behaviour “Will all deactivated Scell be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG”, we would provide our view on this.

Based on current R17 spec, once NCSG is configured, UE would apply NCSG to perform the following types of measurement highlighted in yellow given that all or part of SMTC occasions are overlapped with NCSG.

The carrier-specific scaling factor CSSFwithin_ncsg,i for a measurement object i derived in this clause is applied to following measurement types: ... - SSB-based intra-frequency measurement object corresponding to a deactivated serving cell or to an activated serving cell in dormancy, when all or part of the SMTC occasions of this intra-frequency measurement object are overlapped by the NCSG; - ...

Furthermore, For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for intra-frequency measurement, regarding the applicability of NCSG, R17 spec has the following description, i.e. without any specific condition.

The UE can perform intra-frequency SSB based measurement corresponding to a deactivated SCell or dormant SCell with NCSG.

As a result, no matter what status UE report in intraFreq-needForNCSG, all deactivated SCells should be measured via NCSG unless no overlapping between SMTC and NCSG. So it can be concluded that

- Understanding to be clarified: All deactivated Scell should be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG given that all or part of the SMTC occasions of the deactivated SCell are overlapped with the NCSG.

Observation 1: Regarding the R17 UE behavior alignment, based on current R17 spec, it can be clarified that: All deactivated Scell should be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG given that all or part of the SMTC occasions of the deactivated SCell are overlapped with the NCSG.

After the clarification on R17, look at the progress of this R18 WI, to our understanding, still two aspects should be determined:

- The association between the MO of deactivated SCell and concurrent MG(including at least one NCSG)

- The UE measurement behavior(including the deactivated and activated status)

With respect to Issue 1, the following options were suggested in [2]:

- Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG

To our understanding, Option 1 is aligned with R17 NCSG, and Option 2 is consistent with R17 Concurrent MGs. Regarding Option 1, it is hard to decide which NCSG to apply for the deactivated SCell MO under the gap combination of NCSG + NCSG. Actually such gap combination is allowed in previous meeting. So under such gap combination, some additional rule is still needed. While at the same time, Option 1 can relax the UE responsibility when the deactivated SCell switch to activated status. When the deactivated SCell is activated, UE can determine the measurement behavior within the candidates of via MG 1, via MG 2 or without MG, based on the comprehensive consideration of gap association, NeedForGapNCSG-InfoNR report and active BWP.

Observation 2: If skipping the gap association, all deactivated SCells are measured within NCSG, it is hard to decide which NCSG to apply for one deactivated SCell MO under the gap combination of NCSG + NCSG.

It seems that Option 2 is the most straightforward solution. No matter the gap combination is NCSG + NCSG or NCSG + Type-2 MG, gap association configuration determines the UE behaviour, i.e. the deactivated SCell MO is measured with one NCSG if the MO is associated with the NCSG. Such principle can handle all types of gap combinations uniformly, RAN4 does not need to determine the principle case by case, therefore which is the most straightforward and simple principle. As a result, the deactivated SCell MO associated with NCSG is measured within NCSG, while the deactivated SCell MO not associated with NCSG is measured outside NCSG.

Proposal 1: Prefer to reuse the gap association rule to determine in which MG the deactivated SCell MO would be performed, this is the most straightforward and uniform for any gap combination.

If follow Option 2, considering the SCell status switching from deactivated to activated, which means after the deactivated switches to activated, the gap association should still be applied, i.e. this activated SCell MO is still associated with the NCSG, however whether UE should measure this activated SCell MO(intra-frequency) within NCSG or not, to our understanding, reuse the conditions defined in R17 NCSG as below is one solution. It is aligned with the understanding of R17 UE behaviors achieved in last meeting as above.

For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for intra-frequency measurement, An intra-frequency SSB measurement is defined as measurement with NCSG if - - the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoNR for the intra-frequency measurement, and - - the SSB is not completely contained in the active BWP of the UE, and - - the active downlink BWP is not an initial BWP

Proposal 2: Based on the principle of reusing the gap association rule to determine in which MG the deactivated SCell MO would be performed, when the deactivated SCell switches to be activated, still reuse the R17 conditions to decide whether this SCell can be measured with the NCSG. That is, keep alignment with the understanding of R17 UE behaviors.

2.2 Requirements

Regarding the requirement of interruption, the following way forward is summarized in [2].

< Way forward >: - Over optimization on the interruption length requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps is not necessary.

We agree with it, i.e. no need to over optimization the interruption length for Case 2, reuse the legacy interruption requirement for Case 2.

Proposal 3: Over optimization on the interruption length requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps is not necessary.

- Conclusion

In this contribution, we have the following observation and proposals for joint consideration around Case 2:

Observation 1: Regarding the R17 UE behavior alignment, based on current R17 spec, it can be clarified that: All deactivated Scell should be measured via NCSG regardless the UE capability report of intraFreq-needForNCSG given that all or part of the SMTC occasions of the deactivated SCell are overlapped with the NCSG.

Observation 2: If skipping the gap association, all deactivated SCells are measured within NCSG, it is hard to decide which NCSG to apply for one deactivated SCell MO under the gap combination of NCSG + NCSG.

Proposal 1: Prefer to reuse the gap association rule to determine in which MG the deactivated SCell MO would be performed, this is the most straightforward and uniform for any gap combination.

Proposal 2: Based on the principle of reusing the gap association rule to determine in which MG the deactivated SCell MO would be performed, when the deactivated SCell switches to be activated, still reuse the R17 conditions to decide whether this SCell can be measured with the NCSG. That is, keep alignment with the understanding of R17 UE behaviors.

Proposal 3: Over optimization on the interruption length requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps is not necessary.

- Reference

- [1] RP-231477 “Revised WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps”, Intel Corporation, MediaTek Inc.

- [2] R4-2310175 “WF on NR MG enhancements”, MediaTek Inc