3GPP TSG-RAN WG4 Meeting #108	R4-2313116

Toulouse, France, 21 – 25 Aug, 2023

Source:	ZTE Corporation

Title:	Discussion on Receive timing difference between different directions

Agenda Item:	8.7.3.6

Document for:	Approval

- Introduction

In recent RAN plenary 100 meeting, a revised WID[1] for Rel-18 was approved to further clarify the scope of the  simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier. The main update is to remove the NOTEs related with L3 measurement delay. The objective in the revised WID is as follows:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4 layer DL MIMO - Enhanced RF requirements: - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs - The legacy spherical coverage requirement for reception from a single direction will be kept - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized - Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier - Enhanced RRM requirements: - The following requirements should be studied and specified if necessary: - L1-RSRP measurement delay - RLM and BFD/CBD requirements - Scheduling/measurement restrictions - TCI state switching delay with dual TCI - Receive timing difference between different directions (different QCL Type D RSs) NOTEs: - The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited.

In RAN4 106 meeting, RAN4 has discussed the MRTD issue for multi-panel/multi-beam reception. The following agreements were achieved in [2].

- Agreements in online session: - Define RRM requirements for MRTD < CP - FFS whether to support MRTD > CP and the discussion can take place after RAN4 #107 - Note: the MRTD above is the maximum receive time difference between the signals coming from two directions on the same carrier

In RAN4 106bis and 107 meeting, no progress achieved on this topic. So in this paper, we provide some further discussion on the remaining issues.

- Discussion

After the discussion in last meeting, the remaining controversy is whether to support the case of receiving timing difference beyond CP in this WI. In our opinion, which is closely related with the supported scenario. If inter-cell mTRP is decided to be included in this WI, then the possibility of the receiving timing difference larger than CP exists. From the perspective of UE architecture, based on the assumption of independent RF chain and BB for multi-panel case, not applying any restriction on receiving timing difference is feasible.

- Proposal 1: If inter-cell mTRP is decided to be included in this WI, then the possibility of the receiving timing difference larger than CP exists.

- Proposal 2: From the perspective of UE architecture, based on the assumption of independent RF chain and BB for multi-panel case, not applying any restriction on receiving timing difference is feasible.

- Further more, considering the determination of receiving timing difference, which includes BS TAE and propagation delay difference. So the total receiving timing difference can be seen as a trade-off between BS TAE and propagation delay difference. If we aim to control the total value within CP and support the inter-cell scenario at the same time, then smaller BS TAE should be guaranteed.

- Proposal 3: The total receiving timing difference can be seen as a trade-off between BS TAE and propagation delay difference. So as to control the total value, then smaller BS TAE should be guaranteed if not restrict the deployment of inter-cell scenario.

- Conclusion

In this contribution, we have the following proposals around the receiving time difference between multiple panels for simultaneous DL reception from different directions:

- Proposal 1: If inter-cell mTRP is decided to be included in this WI, then the possibility of the receiving timing difference larger than CP exists.

- Proposal 2: From the perspective of UE architecture, based on the assumption of independent RF chain and BB for multi-panel case, not applying any restriction on receiving timing difference is feasible.

- Proposal 3: The total receiving timing difference can be seen as a trade-off between BS TAE and propagation delay difference. So as to control the total value, then smaller BS TAE should be guaranteed if not restrict the deployment of inter-cell scenario.

- Reference

- RP-223527 “Revised WID: Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception”, Qualcomm Incorporated.

- R4-2303317 “WF on NR FR2 multi-Rx chain DL reception RRM requirements (part 2)”, Qualcomm Incorporated.