3GPP TSG-RAN WG4 Meeting #108	R4-2313120

Toulouse, France, 21 – 25 Aug, 2023

Source:	ZTE Corporation

Title:	Discussion on RRM impacts of Option B-1-1

Agenda Item:	8.10.2.2

Document for:	Approval

- Introduction

In recent RAN plenary 100 meeting, a revised WID was approved in [1] for Rel-18 to revise the objectives of completion of specification support for BWP without restriction in NR. The main update is to add the RRM measurement into the consideration. Therefore, until now, both L1 measurements and RRM measurement should be considered to support the BWP without restriction operation. The revised scope for core part is summarized as below:

### The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based onSSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to Interoperability Device Testing (IoDT) availability). - The interruption related requirements will be decided and specified in RAN4. 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. - Note: further check in RAN #101 on whether any work is required to support L3 intra-frequency measurements without gaps for Option B-1-2 - The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

According to the discussion in RAN, RAN4 and RAN1, the following potential solutions were identified in [2]:

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP - Option B) Perform BM/RLM/BFD based on SSB outside active BWP - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps (MG) - Option B-2-2) Dedicated MG or Network Controlled Small Gap (NCSG) for RLM/BFD/BM measurements - Option C) Non Cell Defining-SSB (NCD-SSB) approach which would work with existing UE hardware architectures (FG 6-1 BWP Operation With Restriction) and be compatible with existing RAN4 specifications for BM/RLM/BFD

Around Option B-1-1, after the discussion in RAN4#107 meeting, RAN4 achieved the following agreements:

Applicable of existing RLM/BFD/BM requirements for supporting Option B-1-1: <Agreement >: - When CD-SSB is outside the active BWP, the existing requirements for SSB-based RLM/BFD/BM are applicable for UE supporting option B-1-1. - FFS details of applicable condition L1 measurement requirements to be made applicable for supporting option B-1-1: <Agreement >: - Existing SSB based RLM/BFD/CBD/BM measurement requirements in clause 8.1, 8.5, 9.5 and 9.8 are to be made applicable for UE supporting option B-1-1. Assumption on RF/BB bandwidth for UE supporting option B-1-1: <Agreement >: - Do not specify assumptions on RF BW and BB BW for UE supporting option B-1-1. <Agreement >: - Do not introduce new UE and gNB behaviour for option B-1-1 in RAN4 spec. It is up to UE and gNB implementation how option B-1-1 is supported. - Do not introduce a new section for RRM requirements for option B-1-1. <Agreement >: - When CD-SSB is outside the active BWP, the requirements for SSB-based RLM/BFD/BM for UE supporting option B-1-2 with interruptions are FFS.

In this document, we provide some of our views on the impact of Option B-1-1 from the following aspects:

- Applicability of existing RLM/BFD/BM requirements

- Impact on UE timing requirements

- Supporting of L3 intra-frequency measurements without gaps

- Discussion

- 2.1 Applicability of existing RLM/BFD/BM requirements

Since it has been approved that for Option B-1-1, the existing requirements for SSB-based RLM/BFD/BM applicable for the UE supporting Option B-1-1. It is still FFS the details for the applicable condition. From RAN4 perspective, even though the CD-SSB is outside the active BWP, it is still within the larger BW, so applying such CD-SSB to perform RLM/BFD/BM is aligned with the request of performing RLM/BFD/BM based on the SSB within active BWP. We can not see additional condition on the applicability of applying existing RLM/BFD/BM requirements for such case as long as the UE supporting Option B-1-1.

In RAN1#113 meeting, the relevant UE feature has been approved in [3] as below:

| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on CD-SSB without interruptions, where the CD-SSB is outside active DL BWP but is within the bandwidth of [the corresponding carrier(s) to be measured.] 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for SCell 3. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on CD-SSB outside active BWP without interruptions | [Per band] | No | No | n/a | [UE indicates at most one of FG 53-1 and FG 53-2.] Note: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig [Note: If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations.] FFS: candidate component values This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

It has been shown in the Note that “The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig”, so in details, the BW of the UE can be further clarified as the bandwidth of the carrier.

Proposal 1: The applicable condition of reusing the existing RLM/BFD/BM requirements is that the CD-SSB is within the BW of the UE. The BW of the UE can be further clarified as the bandwidth of the carrier.

- 2.2 Impact on UE timing requirements

This issue was discussed in recent meetings. The following options were discussed in 106bis and 107 meetings:

- Option 1: - It is clarified in the spec that existing timing requirements for non-RedCap UE are applicable regardless of whether SSB is within active BWP or not. - Option 2: - No clarification on existing timing requirements

In current UE transmit timing requirement in section 7.1 of [4], there is no condition that the SSB should be within the active DL BWP. This means the UE transmit timing requirement applies regardless of whether the SSB is within or outside the active DL BWP. Therefore, applying same logic, here we can not see the necessity to add additional clarification on existing timing requirements used for the UE supporting Option B-1-1.

Proposal 2: No clarification on existing timing requirements.

- 2.3 Supporting of L3 intra-frequency measurements without gaps

The latest revised WID has explicitly included the L3 intra-frequency measurements without gaps for Option B-1-1 and Option C.

- To our understanding, for the UE capable of Option B-1-1, once UE can perform RLM/BFD/BM base on the SSB outside active BWP, which means the BW of UE covers the SSB, so UE can perform such L1 measurement without any additional interruption. Besides L1 measurement, for intra-frequency RRM measurement, from the perspective of UE behavior, it is no doubt that such UE is able to perform intra-frequency RRM measurement without gap. The only concern is that in current spec, intra-frequency RRM measurement without gap in only allowed when the following conditions are met:

The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3].

- So it is beneficial to support the intra-frequency SSB based measurement without MG through the SSB outside the active BWP but within the BW of the carrier. As a result, the condition of intra-frequency SSB based measurements without gap can be extended as below:

The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3], or - the SSB is outside the active BWP if the UE supports FG 53-1.

Proposal 3: For the UE supporting Option B-1-1, the UE is allowed to perform intra-frequency SSB based measurement without MG through the SSB outside active BWP but within the BW of the carrier.

- Conclusion

In this contribution, we have the following proposals for the RRM impact of Option B-1-1:

Proposal 1: The applicable condition of reusing the existing RLM/BFD/BM requirements is that the CD-SSB is within the BW of the UE. The BW of the UE can be further clarified as the bandwidth of the carrier.

Proposal 2: No clarification on existing timing requirements.

Proposal 3: For the UE supporting Option B-1-1, the UE is allowed to perform intra-frequency SSB based measurement without MG through the SSB outside active BWP but within the BW of the carrier.

- Reference

- RP-231486 “Revised WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo.

- RP-230766 “Summary of offline discussion about BWP without restriction”, Vodafone.

- R1-2306165 “Session Notes of AI 9.16.4”, Ad-Hoc Chair(AT&T).

- TS 38.133-i30.