3GPP TSG-RAN WG4 Meeting #108	R4-2313121

Toulouse, France, 21 – 25 Aug, 2023

Source:	ZTE Corporation

Title:	Discussion on RRM impacts of Option B-1-1

Agenda Item:	8.10.2.3

Document for:	Approval

- Introduction

In recent RAN plenary 100 meeting, a revised WID was approved in [1] for Rel-18 to revise the objectives of completion of specification support for BWP without restriction in NR. The main update is to add the RRM measurement into the consideration. Therefore, until now, both L1 measurements and RRM measurement should be considered to support the BWP without restriction operation. The revised scope for core part is summarized as below:

### The work item includes following objectives for non-Redcap UE: 1) Specify support for BM/RLM/BFD: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based onSSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD-SSB and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to Interoperability Device Testing (IoDT) availability). - The interruption related requirements will be decided and specified in RAN4. 2) Specify the necessary requirements to support L3 intra-frequency measurements without gaps (RAN4) - For Option B-1-1 - Specify applicability of legacy intra-frequency L3 measurements without gap and corresponding UE capabilities, if needed. - Note: UEs capable of Option B-1-1 BWP operation can optionally support L3 intra-frequency measurements with gaps. - For Option C - Specify requirements for L3 intra-frequency measurements based on existing RedCap NCD-SSB requirements. - Note: further check in RAN #101 on whether any work is required to support L3 intra-frequency measurements without gaps for Option B-1-2 - The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

According to the discussion in RAN, RAN4 and RAN1, the following potential solutions were identified in [2]:

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP - Option B) Perform BM/RLM/BFD based on SSB outside active BWP - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps (MG) - Option B-2-2) Dedicated MG or Network Controlled Small Gap (NCSG) for RLM/BFD/BM measurements - Option C) Non Cell Defining-SSB (NCD-SSB) approach which would work with existing UE hardware architectures (FG 6-1 BWP Operation With Restriction) and be compatible with existing RAN4 specifications for BM/RLM/BFD

Around Option C, after the discussion in RAN4#107 meeting, RAN4 achieved the following agreements:

Applicable of existing RLM/BFD/BM requirements for supporting Option C: <Agreement >: - The existing requirements for SSB-based RLM/BFD/BM for non-RedCap UE are applicable to option C. - FFS details or wording of applicable condition. L1 measurement requirements to be made applicable for supporting option C: <Agreement >: - Existing SSB based RLM/BFD/CBD/BM measurement requirements in clause 8.1, 8.5, 9.5 and 9.8 are to be made applicable for supporting option C. - Impact to intra-frequency measurement requirements for supporting option C: <Agreement >: - It is a common RAN4 understanding that support of L3 measurements based on NCD-SSB for Option C can be beneficial, but it is not explicitly included in the WI objectives. Whether to support L3 measurements based on NCD-SSB is up to RAN decision. - RAN4 work will focus on enabling L1 measurements based on NCD-SSB in Q2’2023. Whether to define requirements for transition between CD-SSB and NCD-SSB for supporting option C: <Agreement >: - Re-use the requirements for transition between CD-SSB and NCD-SSB for RLM and BFD from RedCap.

In this document, we provide some of our views on the impact of Option B-1-1 from the following aspects:

- RLM/BFD/BM requirements

- Timing requirements

- Supporting of L3 intra-frequency measurements without gaps

- Discussion

- 2.1 RLM/BFD/BM requirements

Since it has been decided that the existing requirements for RLM/BFD/BM are applicable for UE supporting Option C. From the perspective of applying RLM/BFD/BM requirements, no difference between CD-SSB and NCD-SSB. So it is natural for the UE supporting Option C to reuse the existing SSB-based RLM/BFD/BM.

Regarding the applicable condition, based on the relevant UE feature approved in [3] In RAN1#113 meeting,

| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured) and bandwidth of the UE-specific RRC configured BWP may not include CD-SSB for Scell 3. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. |  | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per band | No | No | n/a | This FG is not applicable to RedCap UEs. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Therefore the UE supporting Option C should meet the RLM/BFD/BM requirements provided that the NCD-SSB is within the active BWP.

Proposal 1: The applicable condition of reusing the existing RLM/BFD/BM requirements is that the NCD-SSB is within active BWP of the UE.

The clarification similar as that for RedCap UE can be added into clauses 8.1, 8.5, 9.5 and 9.8. i.e. the SSB and SMTC in the relevant clause applies for both CD-SSB and NCD-SSB.

Proposal 2: In RLM/BFD/BM measurement requirements in clauses 8.1, 8.5, 9.5 and 9.8, it is clarified that the SSB and SMTC applies for CD-SSB and NCD-SSB if the UE supporting FG 53-3.

- 2.2 Timing requirements

This issue was discussed in recent meetings. The following options were discussed in previous meeting:

- Option 1: - The current SSB-based timing requirement can be also applied to non-redCap UE with NCD-SSB support. - Option 2: - It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can also be NCD-SSB within active BWP if UE supports option C.

In fact companies acheved consensus that the UE capable of Option C shall meet the transmit timing error (Te) requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last X ms. Where X=160 ms for UL SCS ≤ 120 kHz or X=80 ms for UL SCS = 480 kHz or X=40 ms for UL SCS = 960 kHz. Right now, the conflict lies in whether some clarification is needed or not. In our view, the description for RepCap UE can be set as reference as below.

- 7.1A.2 Requirements The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured.

Therefore CD-SSB or NCD-SSB can be explicitly mentioned in the timing requriements for non-Redcap UE.

Proposal 3: It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can be CD-SSB or NCD-SSB if UE supports FG 53-3.

- 2.3 Supporting of L3 intra-frequency measurements without gaps

The latest revised WID has explicitly included the L3 intra-frequency measurements without gaps for Option B-1-1 and Option C.

It has been approved that supporting L3 measurements based on NCD-SSB for Option 3 is beneficial, the only obstacle of the NCD-SSB based L3 measurement is the WID scope in previous discussion. However until now, this issue has been included in the revised WID explicitly.

- To our understanding, for the UE capable of Option C, no matter CD-SSB or NCS-SSB, provided the SSB is within the active BWP, UE can perform such L1 measurement without any additional interruption. Besides L1 measurement, for intra-frequency RRM measurement, from the perspective of UE behavior, it is no doubt that such UE is able to perform intra-frequency RRM measurement without gap. Which is beneficial to aviod unnecessary interruption cased by gap. Furthermore, such enhancement has already been supported for Redcap UE.

- Regarding the spec impact, to our understand, the condition of performing intra-frequency SSB based measurement without gap can be extended as below:

The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the CD-SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3] or - the NCD-SSB is completely contained in the active BWP of the UE if the UE supports FG 53-3.

Proposal 4: For the UE supporting Option C, the UE is allowed to perform intra-frequency SSB based measurement without MG through the CD-SSB or NCD-SSB within the active BWP.

- Conclusion

In this contribution, we have the following proposals for the RRM impact of Option B-1-2:

Proposal 1: The applicable condition of reusing the existing RLM/BFD/BM requirements is that the NCD-SSB is within active BWP of the UE.

Proposal 2: In RLM/BFD/BM measurement requirements in clauses 8.1, 8.5, 9.5 and 9.8, it is clarified that the SSB and SMTC applies for CD-SSB and NCD-SSB if the UE supporting FG 53-3.

Proposal 3: It is clarified in the spec that existing timing requirements for non-RedCap UE shall be met provided at least one SSB is available during 160ms, which can be CD-SSB or NCD-SSB if UE supports FG 53-3.

Proposal 4: For the UE supporting Option C, the UE is allowed to perform intra-frequency SSB based measurement without MG through the CD-SSB or NCD-SSB within the active BWP.

- Reference

- RP-231486 “Revised WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo.

- RP-230766 “Summary of offline discussion about BWP without restriction”, Vodafone.

- R1-2306165 “Session Notes of AI 9.16.4”, Ad-Hoc Chair(AT&T).