3GPP TSG-RAN WG4 Meeting #108	R4-2313141

Toulouse, France, 21– 25 August 2023

Source:	Ericsson, Rogers

Title:	TP for TR 37.718-11-21: to include band combinations DC_7A_n12A-n78A

Agenda item:			7.6.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-11-21 to add DC_7A_n12A-n78A configurations as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 6.x	DC_7_n12-n78

### 6.x.1	Operating bands for DC

Table 6.x.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n12-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.x.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A_n12A-n78A | DC_7A_n12A DC_7A_n78A |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n12A-n78A |  |  | 7 |  | 15 | 5 | 10 | 151 | 201,3 |  |  |  |  |  |  |  |  |  |  |  | 135 |
|  |  |  | n12 |  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

### 6.x.3	Co-existence studies

Based on co-existence studies of DC_7_n78 and DC_7_n12, there is no own Rx impact of the 3rd band for this combination.

### 6.x.4	∆TIB and ∆RIB values

For DC_7_n12-n78, the TIB,c and RIB,c values are reused from DC_7_n12-n77 and  are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_7_n12-n78 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_7_n12-n78 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.x.5	MSD requirements

Based on the co-existence studies in section 6.x.3, there is no need to define MSD requirements for DC_7_n12-n78.

---End of changes---

# Reference

[1]		RP-231059, Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)), LG Electronics