3GPP TSG-RAN WG4 Meeting #108	R4-2313157

Toulouse, France, 21 – 25 August 2023

Source:	Ericsson, Rogers

Title:	TP for TR 37.718-21-11: to include band combinations DC_12A-71A_n77A

Agenda item:			7.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.718-21-11 to add DC_12A-71A_n77A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.x	DC_12-71_n77

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_12A-71A_n77A | DC_12A_n77A DC_71A_n77A |

### 5.x.2	Co-existence studies

Based on co-existence studies of DC_12_n77 and DC_71_n77, own Rx impact of the 3rd band is the followings.

-	5th order IMD products generated by DC_71_n77 uplink may fall into own Rx of band 12.

-	5th order IMD products generated by DC_12_n77 uplink may fall into own Rx of band 71.

### 5.x.3	∆TIB and ∆RIB values

For DC_12-71_n77, the TIB,c and RIB,c values are reused from CA_n5-n12-n77 are given in the tables below.

Table 5.X.3-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_12-71_n77 | 0.4 | 0.8 | 0.5 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 5.X.3-2: ΔRIB

| nter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_12-71_n77 | 0.3 | 0.5 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 5.x.4	Reference sensitivity exceptions

Table 5.x.4-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n5A-n12A-n77A are reused.

Table 5.x.4-1: MSD for the DC configuration

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_12A-71A_n77A | 12 | 702 | 5 | 25 | 732 | 14.9 | IMD5 |
|  | 71 | 667 | 5 | 25 | 621 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 12 | 701 | 5 | 25 | 731 | N/A | N/A |
|  | 71 | 692 | 5 | 25 | 646 | 14.0 | IMD5 |
|  | n77 | 3450 | 10 | 50 | 3450 | N/A | N/A |

---End of changes---

# Reference

[1]		RP-231132, Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei