3GPP TSG-RAN WG4 Meeting #108	R4-2313162

Toulouse, France, 21 – 25 August 2023

Source:	Ericsson, T-Mobile USA

Title:	TP for TR 38.718-03-01: to include band combination CA_n41A-n71A-n85A

Agenda item:			7.11.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 38.718-03-01to add CA_n41A-n71A-n85A configuration as specified in WID [1].

# 2. Text Proposal

---Start of changes---

## 5.x	CA_n41A-n71A-n85A

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n85 | 698MHz | – | 716MHz | 728MHz | – | 746 MHz | FDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n41+n71+n85

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n41A-n71A-n85A | CA_n41A-n71A CA_n41A-n85A CA_n71A-n85A | n41 | n41 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n85 | n85 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n41-n71-n85, the TIB,c and RIB,c values are reused from CA_n3-n20-n28 and are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n41-n71-n85 | 0.3 | 0.5 | 0.5 |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n41-n71-n85 | - | 0.1 | 0.1 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

UE co-existence has been already studied for 2DL/1UL fallback combinations: CA n41-n71, CA_n41-n85 and CA_n71-n85 and the impact of harmonic interference has been clarified. The own Rx impact of the 3rd band is shown as the followings:

–   4th order IMD generated by dual uplink of Band n71 + Band n85 may fall into own Rx of Band n41.

#### 5.x.2.2	REFSENS requirements

Table 5.x.2.2-1 lists the MSD required for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD value for CA_n3-n20-n28 is reused.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n41-n71-n85 | n41 | 2687.5 | 5 | 25 | 2687.5 | 9.4 | TDD | IMD4 |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |
|  | n85 | 700.5 | 5 | 25 | 730.5 | N/A | FDD | N/A |

---End of changes---

# Reference

[1]		RP-231100, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2), ZTE