3GPP TSG-RAN WG4 Meeting # 108															R4-2313164

Toulouse, France, August 21 – August 25, 2023

Agenda item:			8.30.2.1.2 Tx requirements

Source:					LG Electronics

Title:	TP for TR 38.786 on the SL-U MPR and A-MPR

Document for:	Approval

# Background

This contribution provides text proposal on the PSSCH/PSCCH MPR/A-MPR for SL-U operation in unlicensed band based on the proposals in [1].

# Text Proposal

##### <<<<<<<<<<< Start of changes in section 6 >>>>>>>>>>

# 6	Transmitter characteristics for NR SL evolution

## 6.1	Tx requirements for NR SL single carrier operation in unlicensed band

### 6.1.1	Maximum output power for NR SL-U

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of shared spectrum channel access carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.1.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  | [23] | [+2/-3] | 20 | +2/-3 |
| n96 |  |  |  |  | [23] | [+2/-3] | 20 | +2/-3 |
| n102 |  |  |  |  | [23] | [+2/-3] | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

The UE operating shall meet the following additional requirements for maximum mean transmission power density specified in Table 6.1.1-2 when NS is signaled and when transmission overlaps with any portion of the specified frequency range.  In case transmission overlaps multiple frequency ranges, the lowest power density requirement applies.

Table 6.1.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_28 | 20, 40, 60, 80 | 5150 – 5350 | 10 |
|  |  |  | 5470 – 5725 |  |
|  | NS_29 | 20 | 5170 – 5330 | 10 |
|  |  |  | 5490 – 5730 |  |
|  |  | 40 | 5170 – 5330 | 7 |
|  |  |  | 5490 – 5730 |  |
|  |  | 60, 80 | 5170 – 5330 | 4 |
|  |  |  | 5490 – 5730 |  |
|  | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
|  | NS_31 | 20 | 5150 - 5230 | 10 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 40 | 5150 - 5230 | 7 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 | 4 |
|  |  | 60, 80 | 5150 - 5230 | 4 |
|  |  |  | 5250 – 5350 |  |
|  |  |  | 5470 – 5725 |  |
|  |  |  | 5725 - 5850 |  |
|  |  |  | 5230 – 5250 |  |
| n96 | NS_53 | 20, 40, 60, 80, 100 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |
|  | NS_59 | 20, 40, 60, 80, 100 | 5925 – 7125 | 5 |
|  | NS_60 | 20, 40, 60, 80, 100 | 5925 – 7125 | 2 |
|  | NS_61 | 20, 40, 60, 80 | 5925 - 6425 | 1 |
|  | NS_66 | 20, 40, 60, 80, 100 | 5925 - 7125 | -8 |
|  | NS_67 | 20, 40, 60, 80, 100 | 5925 - 7125 | -5 |
| n102 | NS_58 | 20, 40, 60, 80, 100 | 5945 – 6425 | 10 |
|  | NS_64 | 20, 40, 60, 80, 100 | 5945 – 6425 | 1 |
|  | NS_65 | 20, 40, 60, 80, 100 | 5945 – 6425 | 11 |
|  | NS_68 | 20, 40, 60, 80, 100 | 5945 – 6425 | 1 |

### 6.1.2	UE maximum output power reduction for NR SL-U

The following assumption can serve as a starting point for MPR simulation assumptions that are considered to derive MPR requirements for SL-U operation.

-	Basic RF parameters for MPR of single carrier SL-U UE

-	Waveforms: CP-OFDM for SL-U

-	Supported CBW: 20/40/60/80/100MHz.

-	Modulation: QPSK/16-QAM/64-QAM/256-QAM

-	SCS: 15/30kHz

-	C-IMD: 45dBc or 60dBc

-	EVM and impairments

| Modulation | SystemEVM | PA only | Image | PA+image |
| --- | --- | --- | --- | --- |
| QPSK | 17.5% | 10% | 28dB | 10.0% |
| 16QAM | 12.5% | 8% | 28dB | 8.0% |
| 64QAM | 8% | 4% | 28dB | 5.65% |
| 256QAM | 3.5% | 1.8% | 34dB | 2.69% |

-	IBE and impairment exceptions are same as NR-U

-	Reuse IBE mask from TS38.10101 Table 6.4F.2.3-1

-	Image/carrier exceptions position and specific test according to interlaces.

-	ACLR: 27dBc for both PC3/PC5 UE with NR MBW

-	SEM according to 38.101-1 clause 6.5F.2.2

-	PA calibration & PA configuration:

-	Power class 3 (both single PA and dual PAs with 20+20): 1dB MPR and DFT-s-OFDM QPSK with 100RB0 20MHz waveform with 30dB ACLR

-	Power class 5 (single PA): 1dB MPR and DFT-s-OFDM QPSK 100RB3 20MHz waveform with 27dB ACLR

-	MPR table format can be reused the MPR format of NR-U according to each transmitted channel e.g.PSSCH/PSCCH transmission, PSFCH transmission and PSBCH transmission if any issue is not seen.

-	The MPR will be applied to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel.

-	Follow the RB interlaced allocation based on Table 6.2F.2-1 and Table 6.2F.2-2 in TS38.101-1. Also refer the interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. FFS on the inter-laced RB allocation for PSFCH, and on the repeated RB allocation for S-SSB by RAN1

For SL-U in single CC operation, the simultaneous transmission of PSCCH and PSSCH in the same sub-frame is supported and the following constraints in Table 6.1.2-1 can be assumed.

Table 6.1.2-1: SL-U operation’s MPR simulation assumptions

| Items | Assumption |
| --- | --- |
| Allowed sub-channel sizes | Support {10, 12, 15, 20, 25, 50, 75, 100} PRBs for possible sub-channel size. |
| Allowed LCRB allocation | 10,12,15,20,24,25,30,36,40,45,48,50,60,70,72,75,80,84,90,96,100,105,108,110,120,125,130,132,135,140,144,150,156,160,165,168,170,175,180,190,192,195,200,204,210,216,220,225,228,230,240,250,252,255,260,264,270 The Allowed LCRB will be decided by the supported SCS. |
| Regarding PSCCH / PSSCH multiplexing |  |
| PSCCH size | 10RB*3 Symbols |
| PSD offset of X dB between PSCCH and PSSCH | 0dB |

For simultaneous transmission of PSFCH transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-2: SL-U UE’s MPR simulation assumptions for PSFCH transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSSCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is follow RAN1 agreements |
| RB allocation | - Power per RB is same in PSFCH for all users - Total power is 20dBm - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB-set, RAN4 consider interlacing RBs for PSFCH For multiple RB-sets, RAN4 considers both contiguous RB sets and non-contiguous RB sets. - N gap from RBstart to RBend should meet at least 80% of channel bandwidth.in a single RB-set(Ngap = RBend – RBstart ) |

For S-SSB transmission for SL-U operation, RAN4 assumed as follow

Table 6.1.2-3e: SL-U UE’s MPR simulation assumptions for S-SSB transmission

| Items | Assumption |
| --- | --- |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | - Single RB-set and multiple RB-sets will be considered based on RAN1 decision. For single RB set, the 11RB will be repeated N time in a RB set. For the multiple RB-sets, RAN4 consider both contiguous RB sets and non-contiguous RB sets. |

#### 6.1.2.1	MPR for SL-U operation in single CC

##### 6.1.2.1.1	MPR for simultaneous PSSCH/PSCCH transmission

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for PSCCH and PSSCH is as specified in Table 6.1.2.1.1-1 for Power class 5 UE.

The UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations for power class 5 according to Table 6.1.2.1.1-1 and Table 6.1.2.1.1-2.

For wideband operation only sub-bands which are contiguously transmitted are considered in the current version of the specification as defined in clause 6.1F in TS 38.101-1.

The Outer and Inner sub-band configurations for SL-U wideband operation are shown in table Table 6.1.2.1.1-2, which includes both contiguous and non-contiguous sub-band configurations. Non-contiguous sub-band configurations can be  considered for PSFCH and S-SSB transmission.

Table 6.1.2.1.1-1 Maximum power reduction (MPR) for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |  |
|  |  |  |  | Full2 (dB) | Partial3 (dB) | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full6 (dB) |  |
| CP-OFDM |  | QPSK |  | ≤ 3.5 | ≤ 3.5 | ≤ 1.5 | ≤ 1.5 | ≤ 4.5 |  |
|  |  | 16 QAM |  | ≤ 4.0 | ≤ 4.0 | ≤ 3.0 | ≤ 3.0 | ≤ 4.5 |  |
|  |  | 64 QAM |  | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |  |  |
|  |  | 256 QAM |  | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2 of TS 38.101-1. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2 of TS 38.101-1. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1-2 apply. NOTE 6: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2-2. |  |  |  |  |  |  |

Table 6.1.2.1.1-2: Outer/Inner sub-band configuration for SL-U wideband operation

| Wideband operation channel bandwidth (MHz) | Contiguous sub-band configuration |  | Non-contiguous sub-band configuration |  |
| --- | --- | --- | --- | --- |
|  | Outer | Inner | Outer | Inner |
| 40 | 11, 10, 01 | N/A | N/A | N/A |
| 60 | 111, 110, 011, 100, 001 | 010 | 101 | N/A |
| 80 | 1111, 1110, 0111, 1100, 0011, 1000, 0001 | 0110, 0100, 0010 | 1101, 1011, 1010, 0101, 1001 | N/A |
| 100 | 11111, 11110, 01111, 11100, 00111, 11000, 00011, 10000, 00001 | 01110, 01100, 00110, 01000, 00010, 00100 | 11011, 11010, 01011, 11001, 10011, 10101, 10110, 01101, 10100, 00101, 10010, 01001, 01010, 11101, 10111, 10001 | N/A |
| NOTE 1: The sub-band configuration is represented as a bitmap where ‘1’ indicates that a sub-band is transmitted and ‘0’ indicates a sub-band is not transmitted. The bitmap is ordered with MSB mapped to the lowest frequency sub-band and LSB mapped to highest frequency sub-band within the wideband channel. |  |  |  |  |

##### 6.1.2.1.2	MPR for PSFCH transmission

##### 6.1.2.1.3	MPR for S-SSB transmission

### 6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

#### 6.1.3.1	A-MPR for SL-U with NS_28

##### 6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.1.2	A-MPR for S-SSB transmission

##### 6.1.3.1.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.2	A-MPR for SL-U with NS_29

##### 6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.2.2	A-MPR for S-SSB transmission

##### 6.1.3.2.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.3	A-MPR for SL-U with NS_30

##### 6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.3.2	A-MPR for S-SSB transmission

##### 6.1.3.3.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.4	A-MPR for SL-U with NS_31

##### 6.1.3.4.1	A-MPR for simultaneous PSSCH/PSCCH transmission

Table 6.1.3.4.1-1. A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | Follow SL-U MPR table |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1-2 apply.. |  |  |  |  |  |  |

##### 6.1.3.4.2	A-MPR for S-SSB transmission

##### 6.1.3.4.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.5	A-MPR for SL-U with NS_53

##### 6.1.3.5.1	A-MPR for simultaneous PSSCH/PSCCH transmission

Table 6.1.3.5.1-1. A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.5.2	A-MPR for S-SSB transmission

##### 6.1.3.5.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.6	A-MPR for SL-U with NS_54

##### 6.1.3.6.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.6.2	A-MPR for S-SSB transmission

##### 6.1.3.6.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.7	A-MPR for SL-U with NS_58

##### 6.1.3.7.1	A-MPR for simultaneous PSSCH/PSCCH transmission

Table 6.1.3.7.1. A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |
|  |  | Full (dB)2 | Partial (dB)3 | Full (dB) 2 | Partial (dB) 3 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 4.0 | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The A-MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The A-MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: The A-MPR applies instead of MPR for 20 MHz channel centered at the nearest NR-ARFCN corresponding to 5955 MHz, 40 MHz channel at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channel at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channel at the nearest NR-ARFCN corresponding to 5985 MHz. For all other channels, A-MPR is zero and MPR as specified in Table 6.2F.2-1 applies. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1-2 apply. |  |  |  |  |  |

##### 6.1.3.7.2	A-MPR for S-SSB transmission

##### 6.1.3.7.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.8	A-MPR for SL-U with NS_59

##### 6.1.3.8.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.8.2	A-MPR for S-SSB transmission

##### 6.1.3.8.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.9	A-MPR for SL-U with NS_60

##### 6.1.3.9.1	A-MPR for simultaneous PSSCH/PSCCH transmission

Table 6.1.3.9.1-1. A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.0 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 6.0 | ≤ 8.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.9.2	A-MPR for S-SSB transmission

##### 6.1.3.9.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

#### 6.1.3.10	A-MPR for SL-U with NS_61

##### 6.1.3.10.1	A-MPR for simultaneous PSSCH/PSCCH transmission

Table 6.1.3.10.1-1. A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz |  | 40MHz |  | 60MHz |  | 80MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | ≤ 7.5 | ≤ 10.0 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 16 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 64 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 6.5 | ≤ 6.5 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 | ≤ 6.0 |
|  | 256 QAM | ≤ 7.5 | ≤ 10.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.10.2	A-MPR for S-SSB transmission

##### 6.1.3.10.3	A-MPR for PSFCH transmission

<Editor Note> A-MPR requirements will be added

##### <<<<<<<<<<< End of changes in section 6 >>>>>>>>>>

# References

[1] R4-2313102, On Tx requirements for NR sidelink evolution, LG Electronics

[2] TR 38.786 v0.2.0