3GPP TSG-RAN WG4 Meeting#108                            R4-2313165

Toulouse ,France, Aug 21 – Aug 25,2023

Agenda item:	8.13.2.2

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Tx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached some initial progress for ATG UE in [6], however there are still lots of open issues left for further discussions. For the ACLR/SEM and ACS requirement of ATG UE, it should be dependent on the outcome of ATG coexistence study as following. In this contribution, we want to share further views on the remaining issues for ATG UE Tx RF requirements.

Discussion

## 2.1. Other Tx RF requirements for ATG UE

In addition, in the following table 1, we share some further views on the ATG UE RF requirement and its details could be found as following:

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Transmitter Characteristics |  |  |
| General | No | Generic requirements from TN should apply. |
| Tx power | Yes | Staus: complete Agreement: - The lower limit of conductive MOP or TRP of ATG UE is 23dBm Agreement: - New capability can be introduced for ATG UE to indicate [the minimum of maximum output power] at maximum modulation order and full PRB configurations. - The capability report granularity is 1dB. Agreement: - Upper limit for new capability for maximum conductive output power or TRP - [40]dBm |
| MPR | No | Staus: Completed without requirement |
| A-MPR | Yes | Status: not discussed yet. No A-MPR requirement defined similar IAB-MT; |
| Configured Tx power | No | Status: not discussed yet. Agreement: - Introduce the configured output power for ATG UE The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, Reported UE capability– ∆TC,c } PCMAX_H,f,c = MIN {PEMAX,c, Reported UE capability } |
| Output Power Dynamics | No | Status: ongoing Agreement: - ATG UE minimum output power can be specified as -25dBm+X/20MHz for 2GHz and -20dbm+Y/100MHz for 4GHz. - FFS on X and Y pending on the co-existence study. Based on the latest calibration results for ATG coexistence study obtained so far, if the minimum uplink SINR could be around -8dB, then minimum output power should be -174dBm/Hz+10*log10(100*10^6)+5-8+100dB=3dBm Where 100dB refer to the minimum coupling loss extracted from the following cdf curve of coupling loss; Considering the some implementation margins, the minimum output power could be around -3dB if 6dB implementation margin is considered. In the last RAN4 meeting, there are some proposals based on the minimum distance as 3km which is much smaller than the minimum distance between ATG CPE and ATG BS and also not aligned without coupling loss CDF curve as depicted as above. Proposal 1: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW. |
| Transmit signal quality |  |  |
| - Frequency error | No | Status :completed |
| - Transmit modulation quality | No | Status : complete Agreement: - UL 256QAM should be supported for ATG UE as optional feature for ATG. |
| Transmit OFF power and Transmit ON/OFF time mask |  | Status :completed |
| Output RF spectrum emissions |  |  |
| - Occupied bandwidth | No | Status :completed |
| - Out of band emission |  |  |
| - SEM | No | Status : ongoing. Based on the simulation results and agreement reached for ATG CPE ACLR requirement in last RAN4 meeting, it is reasonable to reuse the same FR1 SEM requirement requirement for ATG CPE Proposal 2: to reuse the FR1 UE SEM requirement for ATG CPE. |
| - Additional SEM | Yes | Status : complete Agreement: - Agree on Option 1. - Option 1: propose not to define it unless there are any regulatory requirement if necessary |
| - ACLR | No | Status : complete. Agreement: - Agree on Option 1. - Option 1: to reuse FR1 PC3 ACLR requirement for ATG CPE. |
| - Spurious emission |  |  |
| - General | No | Status: completed |
| - For UE coexistence | Yes | Status: completed Agreement: - Agree on Option 1. |
| Transmit intermodulation | No | Status: completed |

Conclusions

In this contribution, we shared some further views for ATG UE Tx RF requirements and proposals are made as following:

Proposal 1: the minimum output power for ATG UE could be [-3dBm/100MHz] and scale with other BW.

Proposal 2: to reuse the FR1 UE SEM requirement for ATG CPE.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,	Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738	WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.

- R4-2220543	WF on ATG UE RF requirements, Huawei, approved.

- R4-2220542,WF on ATG co-existence evaluation, CMCC, Approved.

- R4-2220540,TP for TR 38.876 to capture scenarios and network layout, Ericsson, approved.

- https://aviation.stackexchange.com/questions/48077/which-antenna-on-an-commercial-airline-airplane-is-used-for-the-transponder-ssr

[10] R4-2306609	WF on ATG UE RF requirements, Huawei, approved.

[11] R4-2310400	WF on UE RF requirements for ATG, Huawei, approved.