3GPP TSG-RAN WG4 Meeting#107                              R4-2313166

Incheon , KR, May 22 – May 27,2023

Agenda item:	8.13.2.3

Source: 	ZTE Corporation

Title: 	Further discussion on ATG UE Rx RF requirements

Document for:	Approval

Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package. During the previous RAN4 meetings, we have reached some initial progress for ATG UE in [6], however there are still lots of open issues left for further discussions. For the ACLR/SEM and ACS requirement of ATG UE, it should be dependent on the outcome of ATG coexistence study as following. In this contribution, we want to share further views on the remaining issues for ATG UE Rx RF requirements.

Discussion

## 2.1. Other Rx RF requirements for ATG UE

In addition, in the following table 1, we share some further views on the ATG UE Rx RF requirement and its details could be found as following:

Table 1. Summary of the proposals for ATG UE requirements

| Requirement | Band-specific | Applicability |
| --- | --- | --- |
| Receiver characteristics |  |  |
| General | No |  |
| Diversity characteristics | No | Status: completed |
| Reference sensitivity | Yes | - Status: completed |
| Maximum input level | No | Status: ongoing This depend on the simulation results input to verify whether this requirement could be further relaxed similar as NR over NTN in Rel-17. Based on the coupling loss shown as following, the maximum input power could be further relaxed around 30dB compared with MCL 70dBc for Wide area BS class. Proposal 1: the maximum input power could be relaxed around 30dBc. |
| ACS | No | Status: ongoing Issue 3-2-1: ATG UE ACLR / ACS Agreement: 30dBc ACLR and [33dBc] ACS for ATG UE.. Proposal 2: to confirm 33dBc ACS requirement for ATG CPE. |
| Blocking characteristics |  |  |
| - In-band | No | Status: ongoing To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Out-of-band | Partially Yes | Status: ongoing To use the existing requirements in TS 38.101-1 as baseline and further discuss whether it could be relaxed further since the expected blocking signal should be lower compared with the handheld smartphone |
| - Narrow band | No | Status: completed |
| Spurious response | No | Status: ongoing Expect the same as TN and its power level of interfering signal could be further investigated similar as in-band blocking and out of band blocking. |
| Intermodulation | No | Status: ongoing Not applicable |
| Spurious emissions | No | Status: completed |

Conclusions

In this contribution, we shared some further views for ATG UE Rx RF requirements and proposals are made as following:

Proposal 1: the maximum input power could be relaxed around 30dBc.

Proposal 2: to confirm 33dBc ACS requirement for ATG CPE.

References

- RP-221369, Core part: Air-to-ground network for NR, CMCC, Approved.

- R4-2214461,WF on BS requirements for ATG, ZTE, Approved.

- R4-2215147,WF on UE requirements for ATG, Apple, Approved.

- R4-2214247,	Email Discussion Summary for [104-e][136] NR_ATG_UERF, CMCC, Noted

- R4-2217738	WF on ATG terminal RF requirement, Huawei, HiSilicon, Approved.

- R4-2306609	WF on ATG UE RF requirements, Huawei, approved.

- [11] R4-2310400	WF on UE RF requirements for ATG, Huawei, approved.