3GPP TSG-RAN WG4 Meeting#108                              R4-2313170

Toulouse, France, Aug 21 – Aug 25, 2023

Agenda item:	8.19.2.2.1

Source: 	ZTE Corporation

Title: 	Further discussion on full duplex from FR1 BS perspective

Document for:	Approval

Introduction

In RAN#94e meeting, the work item [RP-221352] on study on evolution of NR duplex operation was approved as one of Rel-18 RAN1 package. In the last RAN4 meeting, there were further discussions on the feasibility for SBFD BS and companies are encouraged to provide the details of self interference analysis for SBFD BS. In this contribution, we want to share some inputs for FR1 SBFD BS.

Discussion

## 2.1. FR1 full duplex BS

In the following section, we provided the link budget table for MR and LA BS we got so far and further study for WA SBFD since the coexistence study for SBFD is still under the discussions.

|  |  | MR | LA |
| --- | --- | --- | --- |
|  | BS TX Power dB(PTX ) = ① dBm | 30 | 23 |
| Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc | 51 | 51 |
|  | Frequency isolation techniques used | DPD utilized | DPD utilized |
| Spatial isolation | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc | 60 | 60 |
|  | Spatial isolation techniques used | TX/RX panel separation | TX/RX panel separation |
| TX Beam nulling /isolation | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc | 0 | 0 |
|  | DL EIRP impact due to beam nulling in TX sub-band | N/A | N/A |
| Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dB(P_(SI_leakage) ) (Note 1) |  | -101 | -108 |
| RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dB(α_(SI-RFIC) ) = ⑤ dBc | 20 | 0 |
|  | RF IC capability and other tech. in RX sub-band dB(α_(SI-RFIC) ) = ⑧ dBc | 0 | 0 |
|  | RF IC techniques and other tech. (before LNA) | subband filtering | N/A |
|  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  |
| Self-Interference signal in gNB TX subband, measured at the input of LNA dB(P_(SI_TXSB) ) |  | -50 | -37 |
| Blocker Suppression at RX | Frequency isolation capability dB(α_(SI-frequency,RX) )= ⑥ dBc | 60 | 60 |
|  | Frequency isolation techniques | digital Filtering | digital Filtering |
|  | Rx IIP3 capability (dBm) | -5 | -5 |
|  | Rx IM3 contribution (dBm) | -140 | -101 |
|  | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Marginal | Marginal |
| Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dB(P_(SI_Selectivity) |  | -110 | -97 |
| RX Beam nulling /isolation | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc | 0 | 0 |
|  | RX sensitivity degradation caused by RX beam nulling | N/A | N/A |
| Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  | 20 | 20 |
| Overall RSIC capability dB(RSIC) |  | 130.5 | 118.3 |
| Noise | Noise Figure (dB) | 10 | 13 |
|  | UL bandwidth (MHz) | 20 | 20 |
|  | Noise Floor ⑩dBm | -91.0 | -88.0 |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  | -97.0 | -94.0 |
| Required RSIC budget (①-⑪dBc) |  | 127.0 | 117.0 |

## 2.2. Spatial isolation between different sector

In the last RAN4 meeting, there were some initial discussions for the spatial isolation among different sectors in the different carriers especially considering the sectors are installed separately in the vertical domain on the mast. Based on the following measurement results, it seems that it might be quite difficult to get the more than 100dB antenna isolation as discussed in the previous meeting.

Conclusions

In this contribution, we want to share further feasibility analysis on the MR SBFD and LA SBFD BS and proposals are made as following:

- Proposal 1: to confirm that MR and LA SBFD with 1dB sensitivity degradation is feasible.

- Observation 1: FR1 antenna isolation among different sectors separated in the vertical domain on the mast are expected to be around 60dBc which is much less than 100dBc.

References

- RP-221352, Study on evolution of NR duplex operation, CMCC, Samsung, Approved

- TR38.828,Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16)

- R4-2214377,WF on feasibility study from RF perspective, Samsung, approved.

- R4-2214376,Reply LS on interference modelling for duplex evolution, Samsung, CMCC, Qualcomm, approved.

- R4-2217464, WF on SBFD feasibility study and RF impact: BS aspects, Samsung, approved.

- R4-2220244 WF for the feasibility from BS aspect, Samsung, approved.

- R4-2302883, WF for Self-interference analysis from BS aspect for SBFD operation, Approved.

TP

- <Start of the TP>

## 10.3 Feasibility of FR1 Medium range BS aspects

### 10.3.1	Self-interference analysis

|  |  | ZTE MR |  |
| --- | --- | --- | --- |
|  | BS TX Power dB(PTX ) = ① dBm | 30 |  |
| Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc | 51 |  |
|  | Frequency isolation techniques used | DPD utilized |  |
| Spatial isolation | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc | 60 |  |
|  | Spatial isolation techniques used | TX/RX panel separation |  |
| TX Beam nulling /isolation | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc | 0 |  |
|  | DL EIRP impact due to beam nulling in TX sub-band | N/A |  |
| Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dB(P_(SI_leakage) ) (Note 1) |  | -101 |  |
| RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dB(α_(SI-RFIC) ) = ⑤ dBc | 20 |  |
|  | RF IC capability and other tech. in RX sub-band dB(α_(SI-RFIC) ) = ⑧ dBc | 0 |  |
|  | RF IC techniques and other tech. (before LNA) | subband filtering |  |
|  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  |
| Self-Interference signal in gNB TX subband, measured at the input of LNA dB(P_(SI_TXSB) ) (Note 1) |  | -50 |  |
| Blocker Suppression at RX | Frequency isolation capability dB(α_(SI-frequency,RX) )= ⑥ dBc | 60 |  |
|  | Frequency isolation techniques | digital Filtering |  |
|  | Rx IIP3 capability (dBm) | -5 |  |
|  | Rx IM3 contribution (dBm) | -140 |  |
|  | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Marginal |  |
| Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dB(P_(SI_Selectivity) )(Note 1, 2) |  | -110 |  |
| RX Beam nulling /isolation | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc | 0 |  |
|  | RX sensitivity degradation caused by RX beam nulling | N/A |  |
| Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  | 20 |  |
| Overall RSIC capability dB(RSIC) (Note 1) |  | 130.5 |  |
| Noise | Noise Figure (dB) | 10 |  |
|  | UL bandwidth (MHz) | 20 |  |
|  | Noise Floor ⑩dBm | -91.0 |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  | -97.0 |  |
| Required RSIC budget (①-⑪dBc) |  | 127.0 |  |

## 10.4 Feasibility of FR1 Local Area BS aspects

### 10.4.1	Self-interference analysis

|  |  | ZTE LA |  |
| --- | --- | --- | --- |
|  | BS TX Power dB(PTX ) = ① dBm | 23 |  |
| Frequency isolation at TX | Frequency isolation capability dB(α_(SI-frequency,TX) ) = ② dBc | 51 |  |
|  | Frequency isolation techniques used | DPD utilized |  |
| Spatial isolation | Spatial isolation capability dB(α_(SI-spatial) ) = ③ dBc | 60 |  |
|  | Spatial isolation techniques used | TX/RX panel separation |  |
| TX Beam nulling /isolation | TX Beam nulling /isolation in TX sub-band dB(α_(SI-beam) ) = ④ dBc | 0 |  |
|  | DL EIRP impact due to beam nulling in TX sub-band | N/A |  |
| Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dB(P_(SI_leakage) ) (Note 1) |  | -108 |  |
| RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dB(α_(SI-RFIC) ) = ⑤ dBc | 0 |  |
|  | RF IC capability and other tech. in RX sub-band dB(α_(SI-RFIC) ) = ⑧ dBc | 0 |  |
|  | RF IC techniques and other tech. (before LNA) | N/A |  |
|  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  |  |
| Self-Interference signal in gNB TX subband, measured at the input of LNA dB(P_(SI_TXSB) ) (Note 1) |  | -37 |  |
| Blocker Suppression at RX | Frequency isolation capability dB(α_(SI-frequency,RX) )= ⑥ dBc | 60 |  |
|  | Frequency isolation techniques | digital Filtering |  |
|  | Rx IIP3 capability (dBm) | -5 |  |
|  | Rx IM3 contribution (dBm) | -101 |  |
|  | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Marginal |  |
| Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dB(P_(SI_Selectivity) )(Note 1, 2) |  | -97 |  |
| RX Beam nulling /isolation | RX Beam nulling /isolation in RX sub-band dB(α_(SI-beam) ) = ⑨ dBc | 0 |  |
|  | RX sensitivity degradation caused by RX beam nulling | N/A |  |
| Digital IC dB(α_(SI-digtial) ) = ⑦ dBc |  | 20 |  |
| Overall RSIC capability dB(RSIC) (Note 1) |  | 118.3 |  |
| Noise | Noise Figure (dB) | 13 |  |
|  | UL bandwidth (MHz) | 20 |  |
|  | Noise Floor ⑩dBm | -88.0 |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  | -94.0 |  |
| Required RSIC budget (①-⑪dBc) |  | 117.0 |  |

- <End of the TP>