3GPP TSG-RAN WG4 Meeting#108                              R4-2313172

Toulouse , France, Aug 21 – Aug 25, 2023

Agenda item:	8.26.1.1

Source: 	ZTE Corporation

Title: 	Further discussion on system parameter for NTN in Ka band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In last RAN4 meeting, we reached some progress for system parameter of NTN K-band, however there are only one remaining issue for system parameter of NTN Ka-band.

Agreement:

  - Option 1: Use following GSCN for Ka-band as following

Table 2: Applicable SS raster entries per operating band (FR2-NTN)

| NR operating band | SS Block SCS | SS Block pattern (note 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| NOTE 1: SS Block pattern is defined in section 4.1 in TS 38.213. |  |  |  |

  - Option 3: Remove [] from agreed WF at RAN4#106-bis-e

Table 2: Applicable SS raster entries per operating band

| NR operating band | SS Block SCS | SS Block pattern (note 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17444 – <12> – 19424 |
|  | 240 kHz | Case E | 17456 – <24> – 19400 |
| n511 | 120 kHz | Case D | 17444 – <12> – 19424 |
|  | 240 kHz | Case E | 17456 – <24> – 19400 |
| n510 | 120 kHz | Case D | 17444 – <12> – 19424 |
|  | 240 kHz | Case E | 17456 – <24> – 19400 |
| NOTE 1: SS Block pattern is defined in section 4.1 in TS 38.213. |  |  |  |

Discussion

## 2.1. sync raster

In the previous RAN4 meeting, we agreed on the step size for sync raster and freq range, however the details need to be further checked. Please check the attached excel sheet for further discussion.

Proposal 1: to use following GSCN for NTN Ka-band as

| NR operating band | SS Block SCS | SS Block pattern (note 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| NOTE 1: SS Block pattern is defined in section 4.1 in TS 38.213. |  |  |  |

Conclusions

In this contribution, we want to share some further views from system parameter perspective for NTN Ka-band.and proposals are made as following:

Proposal 1: to use following GSCN for Ka-band as

Table 4: Applicable SS raster entries per operating band (FR2-NTN)

| NR operating band | SS Block SCS | SS Block pattern (note 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472– <24> – 19416 |
| NOTE 1: SS Block pattern is defined in section 4.1 in TS 38.213. |  |  |  |

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2217496, Email discussion summary for [104-bis-e][312] NR_NTN_enh_Part1

- R4-2217467, WF on NR NTN Enhancements Part1, Thales, approved.

- R4-1709181,WF on NR band Numbering, approved.

- R4-2220224 Ad-hoc minutes for NTN above 10GHz band introduction

- R4-2220239 WF for above 10GHz band definition and system parameters,Thales, Approved.

- R4-2302879, Ad-hoc minutes for system parameters of above 10GHz NTN band introduction, Qualcomm, Approved.

- R4-2305925	WF on NTN enhancement - [106bis-e][309] NR_NTN_enh_Part1,Approved.

- R4-2309766	WF for system parameters on Ka band, Thales, Approved.

- R4-2309182	Further discussion on system parameter for NTN in Ka band, ZTE, Noted