3GPP TSG-RAN WG4 Meeting#108                              R4-230xxxxx

Toulouse , France, Aug 21 – Aug 25, 2023

Agenda item:	8.26.2

Source: 	ZTE Corporation

Title: 	Further discussion on simulation assumption and calibration data for NTN in Ka band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In last RAN4 meeting, we reached good progress for Ka-band coexistence study, however there are still some open issues left for further discussions.

Discussion

In the previous RAN4 meeting, we reached the consensus for ACLR and ACS requirement for TN side, however based on the discussion for SBFD coexistence study, FR2 UE ACLR requirement 17dBc could be replaced by the OBW requirement (23dBc) since OBW requirement is more stringent than ACLR requirement.

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | [19] | [25] | [23.8] | [18.2] |
| 27 GHz | 28 | 24 | 17 | 23 | 21.8 | 16.2 |

Proposal 1: to update the ACLR requirement for TN as following:

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | 23 | [25] | [23.8] | [21.2] |
| 27 GHz | 28 | 24 | 23 | 23 | 21.8 | 20.5 |

In addition, the conducted power for FR2 TN BS is not correct which is based on the 43dBm which is much higher than the practical value. we suggest to follow the FR2 SBFD coexistence study which is 30dBm for total power.

In addition, mechanical downtilt should be 6 degree instead of 10degree which is used for the Micro scenario.

Proposal 2: to update the BS antenna configuration as following:

BS antenna

|  | Macro urban |
| --- | --- |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal j3dB /vertical q3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 dv = 0.5 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Transmitter power (Total conducted power) (dBm) | 30 |
| Mechanical downtilt (degrees) | 6 |

In the last RAN4 meeting, we reached good progress for VSAT antenna pattern, however from the antenna pattern in dBc is not agreed yet, we prefer to go with option 2 similar as Rel-17 assumption.

Parabolic antenna pattern for NTN UE:

  - Use the following parabolic antenna model for NTN UE

Fθ=2 J1uu

With:

- Jix  is the Bessel function of first type and ith order with argument x

- is the angle in a θ;φ spherical coordinates system,

- u=πDλsin⁡(θ)

- D : Antenna diameter

- : Wavelength

Proposal 3: for antenna pattern calculation for NTN VSAT and NTN SAN, propose to follow the option 2.

Option 2: FθdB=10*log10(2 J1uu2)

For the ISD in the following table, it was agreed as 300m, however for all other coexistence study in FR2, its default assumption should be 200m instead of 300m.

Proposal 4: to update the ISD in FR2 as 200m

| Carrier frequency in GHz | 27 |
| --- | --- |
| Size of each nominal channel BW in MHz | 200 |
| Transmission bandwidth in MHz | 190.08 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 200m |
| UE number per cell | Same as the number of BS beam |

For the uplink power control of NTN UE or TN UE, we propose to follow the same approach as agreed for SBFD coexistence study.

Proposal 5: for the uplink power control of TN and NTN UE as following:

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2 are 15 dB.

Conclusions

In this contribution, we want to share some further views from system parameter perspective for NTN Ka-band.and proposals are made as following:

Proposal 1: to update the ACLR requirement for TN as following

| Frequency band | BS |  | UE |  | ACIR |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ACLR | ACS | ACLR | ACS | BS ACLR UE ACS | UE ACLR BS ACS |
| 17 GHz | [30] | [26] | 23 | [25] | [23.8] | [21.2] |
| 27 GHz | 28 | 24 | 23 | 23 | 21.8 | 20.5 |

Proposal 2: to update the BS antenna configuration as following:

BS antenna

|  | Macro urban |
| --- | --- |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal j3dB /vertical q3dB 3 dB beam width of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 dv = 0.5 |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Transmitter power (Total conducted power) (dBm) | 30 |
| Mechanical downtilt (degrees) | 6 |

Proposal 3: for antenna pattern calculation for NTN VSAT and NTN SAN, propose to follow the option 2.

Option 2: FθdB=10*log10(2 J1uu2)

Proposal 4: to update the ISD in FR2 as 200m

| Carrier frequency in GHz | 27 |
| --- | --- |
| Size of each nominal channel BW in MHz | 200 |
| Transmission bandwidth in MHz | 190.08 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 200m |
| UE number per cell | Same as the number of BS beam |

Proposal 5: for the uplink power control of TN and NTN UE as following:

For uplink scenario, TPC model specified in TR 36.942, subclause 9.1 is applied with following parameters.

-	CLx-ile = –SNR_target + UE_max_eirp– ThermalNoise – BS_NoiseFigure - 10*log10(BW)

-	γ = 1

Where, SNR_target for FR1 and FR2 are 15 dB.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- R4-2309771	Simulation assumption for co-existence study of Above 10GHz NTN band, Samsung, Approved.

- R4-2309767	WF for co-existence study of Above 10GHz NTN band, Samsung, Approved.