3GPP TSG-RAN WG4 Meeting#108                              R4-2313176

Toulouse, France, Aug 21 – Aug 25,2023

Agenda item:	8.34.2

Source: 	ZTE Corporation

Title: 	Discussion on UE RF requirement impacts from NES perspective

Document for:	Approval

Introduction

In RAN#98e meeting, the network power saving WID was approved in [1]. In the last RAN4 meeting, there were some initial discussions on RF requirement impacts and RF feasibility for NES, however there are no much progress reached in last RAN4 meeting. Therefore in this contribution, we want to share some further views on it.

Issue 2-1-1: Priorities for feasibility study - Agreement： - RAN4 prioritize the discussion on SSB-less Scell operation for inter-band CA for FR1 and co-located cell. Issue 2-2-6: How to define the RF requirements for inter-band CA for SSB-less Scell operation - Agreement: - No impact to the legacy requirements of inter-band CA TAE defined in TS38.104. Whether or not to define feature-specific TAE requirements for SSB-less operation (NW enabling energy savings) for FR1 co-located inter-band CA is FFS. Issue 2-3-1: RF requirements for Cell DTX - Agreement: - FFS on the necessity to define ON-OFF power requirement waiting for more inputs from RAN1/2. Issue 2-4-1: RF requirements for spatial domain techniques - Agreement: - FFS on the necessity to the updates in manufacturer declarations, switching period and DL EVM requirement waiting for more inputs from RAN1/2. Issue 1-1: TAE - Further discuss the following values for TAE to guarantee the SSB-less feature performance - Option 1: 2.x us, i.e., CP size for 30kHz SCS - Option 2: 260ns - Option 3: 65ns - Option 4: 3us - In the next meeting, encourage companies to provide the achievable values.

Discussion

- In the last RAN4 meeting, there were some initial discussions on the feasibility of achieving much better TAE requirement for inter-band CA in co-located scenario, however there are no consensus reached. First of all, we want to highlight that this requirement is only applicable for co-located inter-band CA scenario. For BS capable to achieve the tight TAE requirement as proposed in the following section, then it could declare to support the SSB-less operation; however for BS not capable to achieve the tight TAE requirement, then it could declare not to support the SSB-less operation.

- In the following section, we mainly focus on the feasibility study on the achievable TAE requirement for inter-band CA in co-located scenario.

- 2.1. Inter-band CA with SSB-less operation

- The existing requirement was defined as 3us for NR which was also inherited from LTE inter-band CA scenario and this requirement is supposed to be based on the worst assumption that the transceivers in different bands are implemented with individual/separate sync source and both sync source has been lost (e.g. 2 downlink bands), then transceivers per band need to maintain its sync performance by its internal maintenance or backup timing module, therefore its achievable timing difference is defined as 3us.

- However as shown in the following Figure 1 for typical inter-band CA BS architecture in the co-located scenario, then practical timing difference among different bands should be optimized down to 10ns-65ns which is much less than the normal CP of the supported subcarrier spacing in FR1. Indeed even for the low boundary of 10ns TAE performance, this could be further optimized with prec-compensation algorithm and higher granularity sampling clock in the practice.

- In addition, if the “adjacent” bands which could be supported by single RF transceiver chain (e.g. FR1 multi-band operation), then TAE between different bands would be mainly dominated by the group delay due to the analog RF component without any residual transceiver calibration error, then the TAE performance is also quite limited even much lower than 10ns. Indeed for single RF chain case to support the multi-bands which is quite similar as intra-band contiguous CA RF architecture to support the BW aggregated positioning, it was agreed that TAE could be ignored otherwise the positioning performance cannot been guaranteed at all.

- It should be known that timing requirement for intra-band contiguous CA with ScellwithoutSSB is 260ns and 6dB power imbalance between active Scell and to-be-activated Scell. From the network perspective, the BS TAE requirement as mentioned before could fulfill the 260ns requirement for intra-band contiguous CA scenario.

- Figure 1. the link budget for inter-band CA TAE performance in co-located scenario

- Observation 1: inter-band CA TAE requirement could be optimized down to 260ns;

- From UE RF perspective, RTD is mainly contributed by BS inter-band CA TAE requirement, therefore from this perspective, at least RTD from UE could be achieved within 260ns considering the BS TAE capability.

- Proposal 1: RTD for co-located inter-band CA could be achieved within 260ns.

- In addition, regarding the power imbalance between different bands, from our understanding, 6dB power balance for intra-band contiguous CA scenario is mainly based on the assumption of single Rx chain reception, however for inter-band CA, usually different Rx chain could be assumed for different bands, therefore from this perspective, 6dB power imbalance for inter-band CA might be not so necessary.

- Proposal 2: propose to consider further relaxation of the 6dB power imbalance for inter-band CA scenario.

Conclusions

In this contribution, we want to share some initial views on NES from UE RF requirement perspective and proposals are made as following:

- Observation 1: inter-band CA TAE requirement could be optimized down to 260ns;

- Proposal 1: RTD for co-located inter-band CA could be achieved within 260ns.

- Proposal 2: propose to consider further relaxation of the 6dB power imbalance for inter-band CA scenario.

References

- RP-223540 “New WID: Network energy savings for NR”, Huawei

- RP-222821, 3GPP TR 38.864 V1.0.0, Study on network energy savings for NR.

- [3] R4-2306639,WF on RF feasibility study of NR Network Energy Saving, Huawei, Approved.

- [4] R4-2306640,Work plan on network energy savings for NR, Huawei, Approved.