3GPP TSG-RAN WG4 Meeting#108                              R4-2313180

Toulouse , France, Aug 21 – Aug 25, 2023

Agenda item:	8.28.2.2

Source: 	ZTE Corporation

Title: 	Discussion on RF requirements for NCR-MT

Document for:	Approval

Introduction

In the latest RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In last RAN4 meeting, we have reached initial consensus for power control for NCR-MT, however there are lots of other issue left for further discussions. In this contribution, we want to share further analysis on RF requirement for NCR-MT.

Discussion

## 2.1. RF requirements for NCR-MT transmitter

First of all, in the following table, we summarized the progress of discussion of NCR-MT Tx RF requirement so far, therefore in this following section, we aim to resolve the all the remaining issues for NCR-MT Tx part.

Table 2.1-1. the summary of the progress of NCR-MT RF requirement discussion

|  | Status/Progress |
| --- | --- |
| General for all Tx requirements | R4-2220255@RAN4#105 meeting - Agreement: For NCR-MT transmitter: - Baseline assumption: Legacy uplink transmitter requirements from Rel-17 RF repeater still be applicable - Further discuss the power control requirements for NCR-MT - Update on other requirements not precluded - For NCR Tx direction requirements, reuse the concept of OTA coverage range which also include OTA peak directions sets. |
| NCR-MT output power | R4-2302903： For NCR-MT, set requirements on TRP and EIRP accuracy using the BS approach (i.e., same as for IAB-MT) [Ericsson,R4-2301883] |
| NCR-MT output power dynamics | R4-2309893: Issue 3-1-1 Output power dynamics and power control - Agreement: - For local area NCR-MT: reuse legacy UE requirements (relative power tolerance and aggregated power tolerance requirements) - For wide area NCR-MT:[reuse the output dynamic range requirement, relative power tolerance and aggregated power tolerance requirements from wide area IAB-MT] |
| NCR-MT power control | Ongoing, the power control of NCR-MT to be confirmed with RAN1 R4-2302903： Agreement: Further clarify WI scope on power control functionality for MT part. R4-2305931 Agreement - Option 1: to support the NCR-MT power control; |
| NCR-MT ON-OFF power | R4-2309893: Issue 3-1-2 Transmitter ON-OFF power and transition period - Agreement: Further discuss below two options: - Option 1: IAB-MT approach - Option 2: Legacy UE approach |
| NCR-MT transition period | R4-2309893: Issue 3-1-2 Transmitter ON-OFF power and transition period - Agreement: Further discuss below two options: - Option 1: IAB-MT approach - Option 2: Legacy UE approach |
| NCR-MT frequency quality | R4-2302903： For the NCR-MT, set requirements on frequency accuracy and EVM using the IAB-MT specification as a reference. |
| NCR-MT unwanted emission | R4-2309893: Issue 3-1-3 OBW - Agreement - Proposal 1 agreed - Proposal 1: for NCR-MT occupied channel bandwidth, to follow the regulation ITU-R SM.328; Issue 3-1-4 ACLR - Agreement: - For wide area NCR-MT: BS requirements applied - For local area NCR-MT: UE requirements (PC2) applied Issue 3-1-5 OBUE/SEM - Agreement: - For wide area NCR-MT: BS requirements applied - For local area NCR-MT: FFS - Proposal 3 agreed - Proposal 3 If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD. Issue 3-1-6 transmitter spurious emission - Agreement: - Proposal 1 agreed - Proposal 1: for NCR-MT transmitter spurious emission requirements, propose to follow Rel-17 repeater transmitter spurious emission requirements. - If the NCR supports simultaneous MT and FWD transmission, then the UL (i.e. BS side) TX emissions requirements should be defined on the total emissions from MT and FWD. |
| NCR-MT transmitter intermodulation | R4-2309893: Issue 3-1-7 Transmitter transmitter intermodulation - Agreement: - Further discuss below two options for NCR-MT: - Option 1: IAB-MT approach - Option 2: Legacy UE approach |

### 2.1.1 Transmitter ON-OFF power and transition period

In the following table, we show some comparison between normal UE and IAB-MT requirement for the further discussions.

From the transmitter ON-OFF requirement perspective, the purpose of defining the corresponding requirement for IAB-MT (same as BS) is different from the normal UE. One is to ensure the minimum impact on the surrounding victim gNB’s receiver, other one is to ensure the minimum impact on the surrounding victim UE’s receiver, therefore the existing ON-OFF requirement are different between BS and UE perspective.

From the transition period requirement perspective, in FR1, the transition period requirement of BS is the same as that of UE; in FR2, the transition period of BS is slightly tighten than UE side due to its better RF capability.

For NCR-MT, at least for Wide area NCR-MT, it should follow the BS requirement or wide area IAB-MT requirement since its deployment scenario and victim system to be protected are supposed to be the same, therefore we propose to reuse the Wide area IAB-MT requirement for NCR-MT which is also aligned with other agreement for it.

Proposal 1: for Wide area NCR-MT transmitter ON-OFF power and transition period requirement, propose to reuse the Wide area IAB-MT requirement.

For Local area NCR-MT, similar approach as other RF agreement, it might be more reasonable to follow the legacy UE requirement instead of to following IAB-MT requirement since IAB-MT could reuse the same hardware as IAB-DU in TDM manner to achieve the same RF requirement, however NCR-MT is supposed to be an additional RF component which is different from the legacy IAB architecture.

From transition period requirement perspective, for FR1, BS and UE requirements are the same and there are no difference. For FR2 local area NCR-MT, if it is to follow the legacy UE requirement which is 5us which will be slightly larger than 3us of local area repeater transition period requirement which might cause some potential timing misaligment from repeater forward backhual. However considering that RF component for NCR-MT and repeater uplink direction is individual, therefore it should be okay to define the requirement separately.

Proposal 2: for Local area NCR-MT transmitter ON-OFF power and transition period requirement, propose to reuse the legacy UE requirement.

| FR1 ON-OFF power 6.4.2.2 Minimum requirement for IAB-DU type 1-H The BS requirements specified in clause 6.4.2.2 in TS 38.104 [2] apply to IAB-DU type 1-H. 6.4.2.3 Minimum requirement for IAB-MT type 1-H The BS requirements specified in clause 6.4.2.2 in TS 38.104 [2] apply to IAB-MT type 1-H. [-85 dBm/MHz per antenna connector.] | TS38.101-1 Table 6.3.2-1: Transmit OFF power Channel bandwidth; (MHz); 5,10,15,20,25,30,35,40,45,50; 60,70,80,90,100. REF_SCS; (kHz); 15; 30. Transmit OFF power; (dBm); -50. Measurement bandwidth; (MHz); MBW=REF_SCS*(12*NRB+1)/1000. NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1.. |
| --- | --- |
| FR1 Transition period 6.4.2.2 Minimum requirement for IAB-DU type 1-H The BS requirements specified in clause 6.4.2.2 in TS 38.104 [2] apply to IAB-DU type 1-H. 6.4.2.3 Minimum requirement for IAB-MT type 1-H The BS requirements specified in clause 6.4.2.2 in TS 38.104 [2] apply to IAB-MT type 1-H. Table 6.4.2.2-1: Minimum requirement for the transmitter transient period for BS type 1-C and BS type 1-H Transition; Transient period length (µs). OFF to ON; 10. ON to OFF; 10. | Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR1 |
| FR2 ON-OFF power 9.5.2.4 Minimum requirement for IAB-MT type 1-O The BS requirements specified in 9.5.2.2 in TS 38.104 [2] apply to IAB-MT type 1-O. 9.5.2.5 Minimum requirement for IAB-MT type 2-O The BS requirements specified in 9.5.2.3 in TS 38.104 [2] apply to IAB-DU type 1-O. The OTA transmitter OFF TRP spectral density for BS type 2-O shall be less than -36 dBm/MHz. | The transmit OFF power shall not exceed the values specified in Tables 6.3.2-1 and 6.3.2-2 for each operating band supported. The requirement is verified with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid). Table 6.3.2-1: Transmit OFF power for FR2-1 Operating band; Channel bandwidth / Transmit OFF power (dBm) / measurement bandwidth. 50 MHz; 100 MHz; 200 MHz; 400 MHz. n257, n258, n259, n260, n261, n262; -35; -35; -35; -35. 47.58 MHz; 95.16 MHz; 190.20 MHz; 380.28 MHz. |
| FR2 Transition period 9.5.3.4 Minimum requirement for IAB-MT type 1-O The BS requirements specified in 9.5.3.2 in TS 38.104 [2] apply to IAB-MT type 1-O. 9.5.3.5 Minimum requirement for IAB-MT type 2-O The BS requirements specified in 9.5.3.3 in TS 38.104 [2] apply to IAB-MT type 2-O. Table 9.5.3.3-1: Minimum requirement for the OTA transmitter transient period for BS type 2-O Transition; Transient period length (µs). OFF to ON; 3. ON to OFF; 3. | Figure 6.3.3.2-1: General ON/OFF time mask for NR UL transmission in FR2 |

### 2.1.2 Transmitter unwanted emission requirement

In the last RAN4 meeting, we reached the consensus on the ACLR requirement for NCR-MT and OBUE requirement for Wide area NCR-MT, however for OBUE requirement of Local area NCR-MT is still FFS. It should be known that we have already agreed to reuse the PC2 UE ACLR requirement for local area NCR-MT, it is more reasonable to follow the same approach by reusing the PC2 UE SEM requirement for Local area NCR-MT instead of reusing local area BS OBUE requirement since Local area BS OBUE requirement is mainly based on BS 45dBc ACLR requirement instead of UE ACLR requirement. Based on the above reasoning, we propose to reuse the legacy UE SEM requirement for Local area NCR-MT.

Proposal 3: for Local area NCR-MT, propose to reuse legacy UE SEM requirement.

For transmitter spurious emission requirements of NCR-MT, since these spurious emission requirement are either to following ITU-R regulation or based on deterministic calculation for the protections of receiver in other spectrum than DL in co-located scenario or in the same geographical scenario, therefore these legacy requirement defined for Rel-17 repeater UL or Rel-16 IAB-MT could be also reused for NCR-MT. In addition, it should be known that there are one difference between Rel-17 repeater UL and Rel-16 IAB-MT.

For Rel-16 IAB-MT, Cat A spurious emission requirement reusing from BS side are applicable for both IAB-DU and IAB-MT. However for Rel-17 repeater UL, Cat A spurious emission are reusing from Cat A requirement in UE specification instead of Cat A requirement in BS specification.

In last RAN4 meeting, it was agreed to follow Rel-17 repeater transmitter spurious emission requirements, indeed this agreement should be further clarified for Wide area NCR-MT and Local area NCR-MT.

For Wide area NCR-MT, since its ACLR requirement, OBUE requirement was agreed to follow the legacy BS requirement, it’s more reasonable to follow the legacy BS transmitter spurious emission requirement for NCR-MT. It should be noted that Rel-16 repeater transmitter spurious emission requirement in the uplink direction is to follow the UE spurious emission regardless of repeater Class in FR1 and to follow the BS spurious emission requirement regardless of repeater class in FR2.

For Local area NCR-MT, since its ACLR was agreed to reuse the legacy UE ACLR requirement and OBUE requirement is also proposed to reuse the legacy UE SEM requirement as consistency. For its transmitter spurious emission requirement, we also proposed to reuse the legacy UE transmitter spurious emission requirements.

Based on the above analysis, we make the following proposals:

Proposal 4a: for Wide area NCR-MT transmitter spurious requirement, propose to reuse the legacy repeater uplink transmitter spurious emission requirement.

Proposal 4b: for local area NCR-MT transmitter spurious requirement, propose to reuse the legacy UE transmitter spurious emission requirement.

### 2.1.3 Transmitter transmitter intermodulation

For transmitter intermodulation requirements of NCR-MT, it might be not appropriate to follow the repeater output intermodulation requirement since the wanted signal for repeater output intermodulation requirement should fill up the whole pass-band as much as possible, however for NCR-MT might only occupy single carrier block or limited carriers if CA supported instead of the whole pass-band, therefore we think that to follow the IAB-MT transmitter intermodulation requirement might be more appropriate option.

Proposal 5a: for Wide area NCR-MT transmitter intermodulation requirement, propose to reuse the Rel-16 IAB-MT intermodulation requirements.

For Local area NCR-MT, to follow the similar logic as previous analysis, it might be more appropriate to follow the FR1 UE transmitter intermodulation requirement for it, however we don’t have strong opinions on it. However one cons of reusing FR1 UE transmitter intermodulation will result in requirement definition misalignment for Wide area NCR-MT and Local area NCR-MT. In addition, considering the interfering level of IAB-MT approach or BS approach is 10dB higher than that of UE side, it should be okay for us to follow the Rel-16 IAB-MT transmitter intermodulation requirement.

Proposal 5b: for Local area NCR-MT transmitter intermodulation requirement, propose to reuse the Rel-16 IAB-MT intermodulation requirements.

## 2.2. RF requirements for NCR-MT receiver

In the following table, we summarized the progress of discussion of NCR-MT Rx RF requirement so far, therefore in this following section, we aim to resolve the all the remaining issues for NCR-MT Rx part.

Table 2.2-1. the summary of the progress of NCR-MT RF requirement discussion

|  | Status/Progress |
| --- | --- |
| General for all Rx requirements | R4-2220255@RAN4#105 meeting - Agreement: - Proposal 1: For NCR-MT Rx direction requirements, it’s suggested to specify following OTA Rx direction declarations. These declarations apply for both input signal and interference signal. - A declared receiver target redirection range. This range is not needed if NCR don’t support redirection. - [five] declared sensitivity RoAoA comprising the conformance testing directions. If don’t support redirection, only one RoAoA is suggested - Receiver target reference direction - Min EIS may only need for MT part Proposal 3: for the RF requirement of NCR-MT receiver, to define the refense requirement for NCR-MT receiver. |
| NCR-MT reference sensitivity | R4-2302903： - To define the NCR-MT Refsens requirement based on IAB-MT approach; - To use the proposal 3 as baseline for calculation of power level of Refesens requirement; - Note: FFS for NF - For 30kHz and 60kHz FRC, to reuse the existing FRC from TS38.174 Annex A.1 - For 15kHz FRC, FFS R4-2309893 Issue 3-2-1 REFSENS requirement - Agreement: - NF is assumed as 5dB for FR1 WA NCR-MT, - FR1 LA NCR-MT: - Option1: 13dB - Option 2: legacy UE value - NF is assumed as 10dB for 30GHz and 12dB for 45GHz for FR2 NCR-MT |
| NCR-MT dynamic range | R4-2309893 Issue 3-2-2 Dynamic range requirement - Agreement: - Proposal 1 agreed - Proposal 1: propose not to define the dynamic range requirement for NCR-MT. |
| NCR-MT ACS/NCR-MT In-band blocking | R4-2309893 Issue 3-2-3 ACS/IBB - Agreement: - For wide area NCR-MT: reusing IAB-MT requirements - For local are NCR-MT: FFS |
| NCR-MT Out of band blocking | R4-2309893 Issue 3-2-4 OOBB requirement - Agreement: - For wide area NCR-MT: reusing IAB-MT requirements - For local are NCR-MT: FFS |
| NCR-MT receiver spurious emission | Not discussed yet |
| NCR-MT receiver intermodulation | R4-2309893 Issue 3-2-6 receiver intermodulation requirement - Agreement: - For wide area NCR-MT: reusing IAB-MT requirements - For local are NCR-MT: FFS |
| NCR-MT in-channel selectivity | R4-2309893 Issue 3-2-7 receiver in-channel selectivity requirement requirement - Agreement: - Proposal 1 agreed - Proposal 1: propose not to define the ICS requirement for NCR-MT |

### 2.2.1 REFSENS requirement

For REFSENS requirement of NCR-MT, it was agreed to follow IAB-MT approach in the past, however there are still lots of open issues left for further discussion.

Regarding the NF for NCR-MT, for FR1 LA NCR-MT, it is FFS. There are basically two options, either to follow the Local area BS NF or normal UE noise figure. Indeed, noise figure of local area BS 13dB is much higher than 9-10dB noise figure at UE side. In addition, considering the removal of form factor limitation at NCR-MT, it might be more reasonable to go with UE noise figure.

Proposal 6: for Local area NCR-MT, to follow the legacy UE noise figure.
Regarding the FRC for 15kHz for NCR-MT, we propose to follow IAB-MT approach or BS approach, the details could be found as following. To define 5MHz and 20MHz FRC with 15kHz and CP-OFDM symbols per slot is reduced from 12 to 9 due to the PDCCH transmission in the downlink.

Proposal 7: for 15kHz FRC of FR1 IAB-MT, propose to use the following FRC.

Table A1-1: FRC parameters for FR1 reference sensitivity level for NCR-MT.

| Reference channel | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-25 | G-FR1-A1-26 | G-FR1-A1-27 | G-FR1-A1-28 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 60 | 30 | 60 | 15 | 15 |
| Allocated resource blocks | 11 | 11 | 51 | 24 | 25 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

### 2.2.2 ACS and IBB requirement

In the previous meeting, in our contribution, we analyzed the combination of ACS requirement and ACRR requirement shown as following, however this scenario is not generic to all practical scenarios indeed, therefore we are also fine to define the ACS and IBB requirement separately with NCR-Fwd link.

Figure 2.2.3-1. the illustration of ACRR requirement with NCR-MT Rx requirements

Figure 2.2.3-2. the illustration of ACRR requirement with NCR-MT Rx requirements

Similar as Tx ACLR requirement, whether to follow the UE specification or BS specification ACS requirement, this needs more discussions.

In last RAN4 meeting, we reached consensus on ACS and IBB requirement for Wide area NCR-MT, however it’s still FFS for Local area NCR-MT.

For Rel-16 IAB-MT ACS and IBB requirement, it was agreed to reuse the BS ACS and IBB requirement for it, however as we mentioned in unwanted emission requirement, this is not valid for NCR-MT part, therefore we propose to use the UE ACS requirement as 33dBc for FR1 NCR-MT and 23,22dBc for FR2 NCR-MT update the IAB-MT or BS requirement accordingly as following.

Proposal 8: for Local area NCR-MT,  propose to use the UE ACS requirement as 33dBc for FR1 NCR-MT and 23,22dBc for FR2 NCR-MT update the IAB-MT or BS requirement accordingly.

Proposal 9: for Local area NCR-MT IBB requirement, propose to follow the legacy UE requirement.

### 2.2.3 OOBB requirement

For OOBB requirement for NCR-MT, we think that the existing IAB-MT requirement or BS approach requirement could be reused here even though we think that OOBB requirement could be further relaxed due to relaxed IBB requirement and the same OOBB requirement -15dBm/CW.

Proposal 10: for OOBB requirement for Local area NCR-MT, propose to follow the legacy UE requirement.

### 2.2.4 Receiver spurious emission requirements

For receiver spurious emission requirement for NCR-MT, we think that the existing IAB-MT requirement or BS approach requirement could be reused.

Proposal 11a: for receiver spurious emission requirement for Wide area NCR-MT, propose to reuse the IAB-MT requirement for it.

Proposal 11b: for receiver spurious emission requirement for Local area NCR-MT, propose to reuse the legacy UE requirement.

### 2.2.5 Receiver intermodulation requirements

For receiver intermodulation for NCR-MT, similar as ACS/IBB requirement for Local area NCR-MT, we propose to follow the legacy UE requirement.

Proposal 12: for receiver intermodulation requirement for Local areas NCR-MT, propose to legacy UE requirement.

Conclusions

In this contribution, we want to share further views on NCR-MT RF requirements and proposals are made as following:

Proposal 1: for Wide area NCR-MT transmitter ON-OFF power and transition period requirement, propose to reuse the Wide area IAB-MT requirement.

Proposal 2: for Local area NCR-MT transmitter ON-OFF power and transition period requirement, propose to reuse the legacy UE requirement.

Proposal 3: for Local area NCR-MT, propose to reuse legacy UE SEM requirement.

Proposal 4a: for Wide area NCR-MT transmitter spurious requirement, propose to reuse the legacy repeater uplink transmitter spurious emission requirement.

Proposal 4b: for local area NCR-MT transmitter spurious requirement, propose to reuse the legacy UE transmitter spurious emission requirement.

Proposal 5a: for Wide area NCR-MT transmitter intermodulation requirement, propose to reuse the Rel-16 IAB-MT intermodulation requirements.

Proposal 5b: for Local area NCR-MT transmitter intermodulation requirement, propose to reuse the Rel-16 IAB-MT intermodulation requirements.

Proposal 6: for Local area NCR-MT, to follow the legacy UE noise figure.

Proposal 7: for 15kHz FRC of FR1 IAB-MT, propose to use the following FRC.

Table A1-1: FRC parameters for FR1 reference sensitivity level for NCR-MT.

| Reference channel | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-25 | G-FR1-A1-26 | G-FR1-A1-27 | G-FR1-A1-28 |
| --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 60 | 30 | 60 | 15 | 15 |
| Allocated resource blocks | 11 | 11 | 51 | 24 | 25 | 106 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |  |  |

Proposal 8: for Local area NCR-MT,  propose to use the UE ACS requirement as 33dBc for FR1 NCR-MT and 23,22dBc for FR2 NCR-MT update the IAB-MT or BS requirement accordingly.

Proposal 9: for Local area NCR-MT IBB requirement, propose to follow the legacy UE requirement.

Proposal 10: for OOBB requirement for Local area NCR-MT, propose to follow the legacy UE requirement.

Proposal 11a: for receiver spurious emission requirement for Wide area NCR-MT, propose to reuse the IAB-MT requirement for it.

Proposal 11b: for receiver spurious emission requirement for Local area NCR-MT, propose to reuse the legacy UE requirement.

Proposal 12: for receiver intermodulation requirement for Local areas NCR-MT, propose to legacy UE requirement.

References

- RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips, Approved

- TS 38.106, NR Repeater Radio Transmission and Reception; (Release 17)

- R4-2207290, WF on repeater RF requirements, Huawei, approved.

- R4-2205030,Repeater radiated power requirements, Ericsson, Noted

- TS 38.115-2,NR;Repeater conformance testing, Part 2: Radiated conformance testing.

- TS 38.115-1,NR;Repeater conformance testing, Part 1: Conducted conformance testing.

- R4-2217263,WF on RRM requirements for NCR, ZTE, approved.

- R4-2217471,WF on NCR RF requirements, ZTE, approved.

- TR 38.867,Study on NR Network-controlled Repeaters

- R4-2217512 ,Post_2nd_round_summary_[104bis-e][314]NR_netcon_repeater, ZTE

- R4-2217506,LS to RAN1 on NCR-MT transmission and Beam correspondence, CMCC, approved.

- R4-2215488, discussion on NCR RF requirements, CMCC, Noted

- R4-2220140 Summary for [105][314] NR_netcon_repeater.

- R4-2303759, Topic summary for [106][314] NR_netcon_repeater, Noted.

- R4-2305931 WF on system parameters and RF requirements for NCR_final,ZTE, approved.

- R4-2309893,WF for system parameter and NCR RF requirements, Approved