3GPP TSG-WG4 Meeting #108 	R4-2313186

# Toulouse, France, Aug 21 - Aug 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR intra-frequency measurements for less than 5 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To introduce missing RRM requirements for NR intra-frequency measurements for 3MHz channel bandwidth |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce missing RRM requirements for NR intra-frequency measurements 3MHz channel bandwidth. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE supporting 3MHz channel bandwidth may not fulfil all the performance requirements and its RRM performance cannot be guanranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

## 9.2D	NR intra-frequency measurements for 3MHz channel bandwidth

### 9.2D.1	Introduction

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the reference SSB of the serving cell and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The reference SSB is the SSB defined in BWP-specific servingCellMO under BWP-DownlinkDedicated of active DL BWP. If the field is absent, the reference SSB is the SSB defined in servingCellMO under ServingCellConfig [2].

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell, even if no explicit neighbour list with physical layer cell identities is provided.

The UE can perform intra-frequency SSB based measurements without measurement gaps if

-	the SSB is completely contained in the active BWP of the UE, or

-	the active downlink BWP is initial BWP [3].

For intra-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.2D.5.3.

SSB based measurements are configured along with one or two measurement timing configuration(s) (SMTC(s)) which provides periodicity, duration and offset information on a window of up to 5ms where the measurements are to be performed. For intra-frequency connected mode measurements, up to two measurement window periodicities may be configured. A single measurement window offset and measurement duration are configured per intra-frequency measurement object.

When measurement gaps are needed, the UE is not expected to detect SSB which start earlier than the gap starting time + switching time, nor detect SSB which end later than the gap end – switching time. Switching time is 0.5ms for frequency range FR1.

### 9.2D.2	Requirements applicability

The requirements in clause 9.2D apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.2 for FR1, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 for FR1 respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 for FR1, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

### 9.2D.3	Number of cells and number of SSB

#### 9.2D.3.1	Requirements for FR1

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	14 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources.

### 9.2D.4	Measurement Reporting Requirements

#### 9.2D.4.1	Periodic Reporting

For Lessthan_5MHz: Reported RSRP, RSRQ, and RS-SINR measurements contained in periodic measurement reports shall meet the requirements in clauses TBD (RSRP for FR1), TBD (RSRQ for FR1), TBD (RS-SINR for FR1) .

#### 9.2D.4.2	Event-triggered Periodic Reporting

For 3MHz: Reported RSRP, RSRQ, and RS-SINR measurements contained in event-triggered periodic measurement reports shall meet the requirements in clauses TBD (RSRP for FR1), TBD (RSRQ for FR1), TBD (RS-SINR for FR1)

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.2D.4.3.

#### 9.2D.4.3	Event Triggered Reporting

For 3MHz: Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in clauses TBD (RSRP for FR1), TBD (RSRQ for FR1), TBD (RS-SINR for FR1)).

The UE shall not send any event triggered measurement reports as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index_3MHz or T identify intra without index_3MHz defined in clause 9.2D.5.1 or clause 9.2D.6.2. When L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index_3MHz or Tidentify_intra_with_index_3MHz as defined in clause 9.2D.5.1 or clause 9.2D.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index_3MHz or Tidentify intra with index_3MHz defined in clause 9.2D.5.1 or clause 9.2D.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra_3MHz provided the timing to that cell has not changed more than ± 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected.

### 9.2D.5	Intra-frequency measurements without measurement gaps for 3MHz channel bandwidth

#### 9.2D.5.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index_3MHz if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_3MHz. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_3MHz.

Tidentify_intra_without_index_3MHz = (TPSS/SSS_sync_intra_3MHz + T SSB_measurement_period_intra_3MHz) ms

Tidentify_intra_with_index_3MHz = (TPSS/SSS_sync_intra_3MHz + T SSB_measurement_period_intra_3MHz + TSSB_time_index_intra_3MHz) ms

Where:

TPSS/SSS_sync_intra_3MHz: it is the time period used in PSS/SSS detection given in tables 9.2D.5.1-1,.

TSSB_time_index_intra_3MHz: it is the time period used to acquire the index of the SSB being measured given in tables 9.2D.5.1-2

T SSB_measurement_period_intra_3MHz: equal to a measurement period of SSB based measurement given in table 9.2D.5.2-1.

CSSFintra_3MHz: it is a carrier specific scaling factor and is determined

according to CSSFoutside_gap_3MHz,i in clause TBD for measurement conducted outside measurement gaps, i.e. when intra-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps, or according to CSSFwithin_gap_3MHz,i in clause TBD for measurement conducted within measurement gaps, i.e. when intra-frequency SMTC is fully overlapping with measurement gaps.

if the high layer in TS 38.331 [2] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

For 3MHz:

Mmeas_period_w/o_gaps_3MHz : For a UE supporting power class 1, Mmeas_period_w/o_gaps_3MHz =40. For a UE supporting power class 3, Mmeas_period_w/o_gaps_3MHz =24.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. For calculation of Kp, if the high layer signalling (TS 38.331 [2]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_3MHz or Tidentify_intra_with_index_3MHz

Table 9.2D.5.1-1: Time period for PSS/SSS detection, (Frequency range FR1) for 3MHz channel bandwidth UE

| DRX cycle | TPSS/SSS_sync_intra_3MHz |
| --- | --- |
| No DRX | max(600ms, ceil(5 x Kp) x SMTC period )Note 1 x CSSFintra_3MHz |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_3MHz |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra_3MHz |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 9.2D.5.1-2: Time period for time index detection (FR1) for 3MHz channel bandwidth UE

| DRX cycle | TSSB_time_index_intra_3MHz |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra_3MHz |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_3MHz |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra_3MHz |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

#### 9.2D.5.2	Measurement period

The measurement period for intra-frequency measurements without gaps is as shown in table 9.2D.5.2-1.

If the higher layer signaling in TS38.331 [2] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for TSSB_measurement_period_intra_3MHz.

Table 9.2D.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) for 3MHz channel bandwidth UE

| DRX cycle | T SSB_measurement_period_intra_3MHz |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra_3MHz |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_3MHz |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra_3MHz |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

#### 9.2D.5.3	Scheduling availability of UE during intra-frequency measurements

UE shall be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged [2], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [3] are included.

### 9.2D.6	Intra-frequency measurements with measurement gaps

#### 9.2D.6.1	Intra-frequency cell identification

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index_3MHz if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index_3MHz. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index_3MHz.

Tidentify_intra_without_index_3MHz = TPSS/SSS_sync_intra_3MHz + T SSB_measurement_period_intra_3MHz ms

Tidentify_intra_with_index_3MHz = TPSS/SSS_sync_intra_3MHz + T SSB_measurement_period_intra_3MHz + TSSB_time_index_intra_3MHz ms

Where:

TPSS/SSS_sync_intra_3MHz: it is the time period used in PSS/SSS detection given in table 9.2D.6.1-1 or 9.2D.

TSSB_time_index_intra_3MHz: it is the time period used to acquire the index of the SSB being measured given in table 9.2D.6.1-2.

T SSB_measurement_period_intra_3MHz: equal to a measurement period of SSB based measurement given in table 9.2D.6.2-1.

CSSFintra_3MHz: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap_3MHz,i in clause TBDfor measurement conducted within measurement gaps.

For 3MHz:

Mmeas_period_ with_gaps_3MHz: For a UE supporting power class 1, Mmeas_period_ with_gaps_3MHz =40. For a UE supporting power class 3, Mmeas_period_ with_gaps_3MHz =24.

If the higher layer signaling in TS 38.331 [2] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, requirements are not specified for Tidentify_intra_without_index_3MHz or Tidentify_intra_with_index_3MHz.

Table 9.2D.6.1-1: Time period for PSS/SSS detection (FR1) for 3MHz UE

| DRX cycle | TPSS/SSS_sync_intra_3MHz |
| --- | --- |
| No DRX | max(600ms, 5 x max(MGRP, SMTC period)) x CSSFintra_3MHz |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_3MHz |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra_3MHz |

Table 9.2D.6.1-2: Time period for time index detection (Frequency range FR1) for 3MHz UE

| DRX cycle | TSSB_time_index_intra_3MHz |
| --- | --- |
| No DRX | max(120ms, 3 x max(MGRP, SMTC period)) x CSSFintra_3MHz |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 3) x max(MGRP, SMTC period,DRX cycle) x CSSFintra_3MHz) |
| DRX cycle>320ms | 3 x max(MGRP, DRX cycle) x CSSFintra_3MHz |

#### 9.2D.6.2	Intra-frequency Measurement Period

The measurement period for FR1 intra-frequency measurements with gaps is as shown in table 9.2D.6.2-1.

Table 9.2D.6.2-1: Measurement period for intra-frequency measurements with gaps (FR1) for3MHz UE

| DRX cycle | T SSB_measurement_period_intra_3MHz |
| --- | --- |
| No DRX | max(200ms, 5 x max(MGRP, SMTC period)) x CSSFintra_3MHz |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra_3MHz |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra_3MHz |

<End of change 1>