3GPP TSG-RAN WG4 Meeting #108 	R4-2313188

Toulouse, France, August 21-25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on Less than 5MHz RRM requirements

Agenda item:	8.14.4

Document for:	Discussion

Introduction

This contribution focuses on the remaining issues on FR1 < 5MHz providing analysis of the issues in the light of the latest RAN1 agreements and provide PDCCH simulations with simulation assumptions. Furthermore, the impact on CCE to REG mapping is analysed for 12 RB and 15 RB scenarios.

RAN4 #107 PBCH simulations are presented in R4-2309659 and associated discussion paper from #107 in R4-2309496

RAN4 #108 PDCCH simulations are presented in our companion simulation contribution [4].

Discussion

PDCCH transmission parameter changes

Issue 1-6: PDCCH transmission parameters changes: Way forward: Reduce aggregation level (CCE) or to increase the number of control OFDM symbols from 2 to 3 - Option 1: Agree (please include further details) - Option 2: Other (please describe detailed proposal)

In the last RAN4 meeting PDCCH transmission parameters were discussed. RAN1 has made the following agreements regarding the CORESET#0.

RAN1 Agreement in RAN1#113 - For 3MHz channel bandwidth in all bands (max channel utilization 15 PRBs as already agreed in RAN1/RAN4): - PBCH transmission bandwidth is 12 PRBs - For CORESET#0 transmission bandwidth, both 12 PRBs and 15 PRBs are supported - In Case of 12 PRBs, the legacy interleaved (R=2) CORESET CCE-to-REG mapping is used with 𝑁RB CORESET = 12, i.e., 12PRBs are indicated without puncturing. - In Case of 15 PRBs, the 𝑁RB CORESET = 24 CORESET#0 is punctured - Both interleaved (legacy interleaver size of R=2) and non-interleaved mapping are supported, - Some entries in the table are related with interleaved mapping and some are non-interleaved mapping. - A single table of up to 16 entries to accommodate both cases - Maximum number of CORESET#0 symbols is 3. Minimum number of CORESET#0 symbols is 2. - SSB and CORESET#0 multiplexing pattern 1 is used - REG bundle size = 6

To summarise, RAN1 has made the following agreements for CORESET#0 on 3 MHz channel BW:

  - For CORESET#0 transmission bandwidth on 3 MHz CBW, both 12 PRBs and 15 PRBs are supported

  - In case of CORESET#0 of 12PRBs, 𝑁RB CORESET = 12 is indicated and no puncturing is applied

  - In case of CORESET#0 of 15 PRBs, the 𝑁RB CORESET = 24 CORESET#0 is punctured. Both interleaved (legacy interleaver size of R=2) and non-interleaved mapping are supported

  - For CORESET#0, the supported numbers of OFDM symbols are 2 and 3

  - REG bundle size = 6

Based on the latest agreements from RAN1, 38.133 SSB-based RLM tables for PDCCH transmission parameters need to be updated.

For SSB based radio link monitoring, Table 8.1.2.1-1 (out-of-sync) and Table 8.1.2.1-2 (in-sync) are updated for 12 PRB case and 15PRB case respectively.

In the last RAN4 meeting, it was agreed that for SSB-based out-of-sync and in-sync evaluation, RAN4 will keep the existing SSB based RLM evaluation periods TEvaluate_out_SSB for CBW less then 5MHz. Therefore, for SSB based radio link monitoring the number of samples for TEvaluate_out_SSB  is 10, and for  TEvaluate_in_SSB is 5.

The RAN1 agreements on the CORESET#0 sizes can impact the PDCCH transmission parameters used in the in-sync and out-of-sync evaluations. It is worth to check how the assumptions of 4 CCE and 8 CCE PDCCHs fit into the smaller CORESET#0 sizes, although there are some items on CORESET#0 design remaining open in RAN1, e.g., whether the PDCCH puncturing is done at level of REGs or REG bundles.

The CCE-to-REG bundle mapping is shown in Figure 1 for 15-RB CORESET#0. The Figure 1 considers both, non-interleaved mapping (Figure 1a) and interleaved mapping (Figure 1b) for 2 and 3 OFDM symbol configurations. From the figure, it can be noted for 15-RB CORESET#0 that:

- In case of 2 symbol configuration, CORESET#0 contains 5 REG bundles, each carrying one CCE.

  - AL8 PDCCH is punctured and transmitted on 5 CCEs

  - AL4 PDCCH can be transmitted without any puncturing with non-interleaved mapping, but is punctured down to 3 CCEs with interleaved mapping

- In case of 3 symbol configuration, CORESET#0 can contain up to 7 and half REG bundles: at least half of the REGs of the 8th REG bundle is punctured (note: RAN1 has not yet agreed whether PDCCH puncturing is done at level of REGs or REG bundles).

  - AL8 PDCCH is transmitted on 7 and half CCEs (assuming REG level puncturing) with non-interleaved mapping but on 5 and half CCEs with interleaved mapping

  - AL4 PDCCH can be transmitted without any puncturing with non-interleaved mapping but is punctured down to 3 and half CCEs with interleaved mapping.

It can be noted that for 15-RB CORESET#0, the in-sync evaluation PDCCH parameter of 4 CCEs can be kept with non-interleaved mapping. However, for out-of-sync evaluation, the PDCCH parameter pair of 8 CCEs and 2 symbols needs consideration. Keeping current parameters leads to PDCCH puncturing and transmission on 5 CCEs, with performance degradation. Alternatively, with change of 2 symbols to 3 symbols the PDCCH puncturing, and performance loss can be minimized.

a)						b)

Figure 1: 15 RB CCE to REG mapping with interleaving and non-interleaving options

Figure 2 shows the interleaved CCE-to-REG bundle mapping for the 12-RB CORESET#0 case for both 2 and 3 OFDM symbol configurations. It can be noted for 12-RB CORESET#0 that

- CORESET#0 contains 4 or 6 REG bundles for 2 symbol and 3 symbol configurations, correspondingly. Hence AL8 PDCCH cannot be used.

- On other hand AL4 PDCCH can be transmitted both 2 symbol and 3 symbol configurations.

AL8 cannot be used for 12RB CCE-to-REG mapping.

Hence, it can be noted that for 12-RB CORESET#0, the in-sync evaluation PDCCH parameter of 4 CCEs can be kept while PDCCH parameter of 8 CCEs needs to be replaced with 4 CCEs for out-of-sync evaluation, resulting in performance degradation.

Figure 2. 12 RB CCE to REG mapping with interleaving

As the PDCCH parameter of 8 CCEs needs to be replaced with 4 CCEs for out-of-sync evaluation. The following table presents simulations for PDCCH with various channel models. For 12 RB, the cases which are marked as N/A have not been simulated. However, the 12 RB values of non-interleaved can be used in that case as the performance in 4 CCE case is the same as only the order in which CCEs are mapped on the REGs changes.

| Channel | BW | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Interleaved |  |  |  |
|  |  | BLERin 2% | Delta | BLERout 10% | Delta |
|  |  | [dB] | [dB] | [dB] | [dB] |
| AWGN, | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.6 | -6.3 | -2.5 |
| 3km/h | 24 PRBs Interl. 8 CCE | -8.2 |  | -8.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.3 | -6.3 | -2.3 |
|  | 24 PRBs Interl. 8 CCE | -7.9 |  | -8.6 |  |
| TDL-A30ns, | 12 PRBs Non-Interl. 4 CCE | -2.4 | -1.5 | -4.6 | -2.2 |
| 3 km/h | 24 PRBs Interl. 8 CCE | -3.9 |  | -6.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -3.2 | -0.8 | -4.7 | -2.1 |
|  | 24 PRBs Interl. 8 CCE | -4 |  | -6.8 |  |
| TDL-B100ns, | 12 PRBs Non-Interl. 4 CCE | -3.1 | -2.2 | -4.5 | -2.3 |
| 3 km/h | 24 PRBs Interl. 8 CCE | -5.3 |  | -6.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -2.7 | -2.5 | -4.2 | -2.4 |
|  | 24 PRBs Interl. 8 CCE | -5.2 |  | -6.6 |  |
| TDL-C300ns, | 12 PRBs Non-Interl. 4 CCE | -3.7 | -2.6 | -4.9 | -2.4 |
| 3 km/h | 24 PRBs Interl. 8 CCE | -6.3 |  | -7.3 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -3.5 | -2.9 | -4.6 | -2.7 |
|  | 24 PRBs Interl. 8 CCE | -6.4 |  | -7.3 |  |
| AWGN, | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.6 | -6.3 | -2.5 |
| 500 km/h | 24 PRBs Interl. 8 CCE | -8.2 |  | -8.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.3 | -6.3 | -2.3 |
|  | 24 PRBs Interl. 8 CCE | -7.9 |  | -8.6 |  |
| TDL-A30ns, | 12 PRBs Non-Interl. 4 CCE | -1.7 | -2.7 | -3.8 | -2.6 |
| 500 km/h | 24 PRBs Interl. 8 CCE | -4.4 |  | -6.4 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -1.7 | -2.8 | -3.8 | -2.5 |
|  | 24 PRBs Interl. 8 CCE | -4.5 |  | -6.3 |  |
| TDL-B100ns, | 12 PRBs Non-Interl. 4 CCE | -2.4 | -2.7 | -4.2 | -2.6 |
| 500 km/h | 24 PRBs Interl. 8 CCE | -5.1 |  | -6.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -2.4 | -2.8 | -4.1 | -2.6 |
|  | 24 PRBs Interl. 8 CCE | -5.2 |  | -6.7 |  |
| TDL-C300ns, | 12 PRBs Non-Interl. 4 CCE | -2 | -3.8 | -4.3 | -2.9 |
| 500 km/h | 24 PRBs Interl. 8 CCE | -5.8 |  | -7.2 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -2.2 | -3.4 | -4.4 | -2.6 |
|  | 24 PRBs Interl. 8 CCE | -5.6 |  | -7 |  |

Table 1: Selected PDCCH simulations for 12 PRB

| Channel | PRBs | BW | SNR [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 CCEs |  |  |  | 8 CCEs |  |  |  |
|  |  |  | Interleaved |  |  |  | Interleaved |  |  |  |
|  |  |  | BLERin 2% | Delta | BLERout 10% | Delta | BLERin 2% | Delta | BLERout 10% | Delta |
|  |  |  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| AWGN,3km/h | 15 PRBs | 2 OFDM Symbols | -1.6 | -4 | -3 | -3.3 | -5 | -3.2 | -6.3 | -2.5 |
|  | 24 PRBs |  | -5.6 |  | -6.3 |  | -8.2 |  | -8.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -4.5 | -1.1 | -5.3 | -1 | -6 | -1.9 | -6.9 | -1.7 |
|  | 24 PRBs |  | -5.6 |  | -6.3 |  | -7.9 |  | -8.6 |  |
| TDL-B100ns, 500 km/h | 15 PRBs | 2 OFDM Symbols | -0.7 | -1.7 | -2.7 | -1.5 | -3 | -2.1 | -4.8 | -2 |
|  | 24 PRBs |  | -2.4 |  | -4.2 |  | -5.1 |  | -6.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -1.8 | -0.6 | -3.5 | -0.6 | -1.7 | -3.5 | -5.2 | -1.5 |
|  | 24 PRBs |  | -2.4 |  | -4.1 |  | -5.2 |  | -6.7 |  |

Table 2 Selected PDCCH simulations for 15 PRB

For 15 RB case RLM and BFD, PDCCH BW is reduced to 15 RBs, non-interleaved allocation is used. Additionally, 3 OFDM symbols are used for out-of-sync.

For such changes, SNR increase is less than 1 dB for 8 CCE.

For 12 RB case, PDCCH BW is reduced to 12 RBs and 8 CCE is replaced with 4 CCEs.

Observation 4: For such changes, SNR increase is 3 dB for 8 CCE.

Issue 1-7: Side conditions

RAN4 discussed the Es/Iot side condition for NR target cell detection.

Issue 1-17: Use side condition Es/Iot≥-4 dB for NR target cell detection Way forward: Use side condition Es/Iot≥-4 dB for NR target cell detection - Option 1: Use side condition Es/Iot≥-4 dB for target NR cell detection for RRC connection re-establishment and RRC connection release with re-direction - Option 2: Other

We think that the starting point should be to maintain the same -6 dB side condition as legacy.

Keep the existing side condition Es/Iot≥-6 dB for NR target cell detection

Sub-topic 1-3 Link Recovery Procedures

During the online session is was agreed that the agreements made for RLM can be applied also for link recovery procedures.

BFD requirement table uses the same parameters as RLM

SSB index reading simulations

The following table provides a summary of the simulations performed to derive the number of samples for SSB-index reading, and whether soft-combining needs to be assumed to meet the performance criteria for specific SNR. We present results with 99% performance criteria with single SSB detection, maximum correlation detection from multiple SSBs and non-coherent combining over 2 or 3 SSBs.

20PRB case is used as a baseline performance, and we expect to reach similar performance with less PRBs.

To be able to reach similar performance level with 12PRBs as baseline 20PRBs, combining of either 2or 3 SSBs is needed. Additionally, we note that a single SSB detection with 12-RB PBCH is sufficient at SNR of -8 dB.

For DMRS detection, single SSB detection is sufficient for 12-RB PBCH down to –8 dB SNR. Soft combining can be left up to UE implementation

Feasible operating point / delay is selected based on non-coherent or max correlation results.  Any implementation with similar performance is allowed.

MIB-reading reading simulations

MIB reading is impacted by PBCH detection. The simulations show the impact of MIB reading with AWGN 3km/h and 500km/h. We can observe that the soft combining can help to mitigate the issues caused by the reduced number of PRBs.

The number of SS-blocks required for 99% to successfully decode the PBCH is used as a performance metric.  PBCH detection error vs SNR for different number of SS-blocks. 1% detection error at -6 dB SNR after 10 or 5 SS-blocks.

RAN4 to discuss about MIB reading simulation results for 3km/h and 500km/h, and the impact of soft combining.

Soft combining is assumed for MIB reading.

Nokia has shown through our simulations that with the use of soft combining performance gains of approximately 6 dB can be achieved

Whilst we note that the method of achieving the performance gains achieved within these simulations is down to UE implementation, we note that soft combining through belief propagation or list decoding is core to the efficient decoding of 5G error correction schemes, such as an iterative LDPC decoder for PDSCH decoding, or indeed the processing of a number of decoding approaches for Polar Code (such as successive cancellation list decoding, or any other method described in [1]), as used within PBCH, UCI and PUCCH. Therefore it is unlikely that a UE capable of decoding LDPC or polar codes would be necessarily hardware resource constrained for soft combination/summing of MIB LLRs prior to polar decoding, or there would be constraints with the experience of these soft combination algorithms.

Furthermore, we believe that the performance should be set with a margin of 2dB from the performance observed with 7 combinations, and how the UE achieves this, is up to manufacturer implementation.

We propose RAN4 to introduce MIB requirements based on a performance achieved in the Nokia simulations.

Feasible MIB-reading delay is selected based on soft combining results. Any implementation with similar performance is allowed

Conclusion

In this contribution Nokia have conducted analysis on the remaining issues on FR1 < in the light of the latest RAN1 agreements. Furthermore, the impact on CCE to REG mapping has been analysed for 12 RB and 15 RB scenarios.

Specifically, the following Observations and Proposals have been made:

Observation 1: To summarise, RAN1 has made the following agreements for CORESET#0 on 3 MHz channel BW:

	For CORESET#0 transmission bandwidth on 3 MHz CBW, both 12 PRBs and 15 PRBs are supported

	In case of CORESET#0 of 12PRBs, 𝑁RB CORESET = 12 is indicated and no puncturing is applied

	In case of CORESET#0 of 15 PRBs, the 𝑁RB CORESET = 24 CORESET#0 is punctured. Both interleaved (legacy interleaver size of R=2) and non-interleaved mapping are supported

	For CORESET#0, the supported numbers of OFDM symbols are 2 and 3

	REG bundle size = 6

Proposal 1: For SSB based radio link monitoring, Table 8.1.2.1-1 (out-of-sync) and Table 8.1.2.1-2 (in-sync) are updated for 12 PRB case and 15PRB case respectively.

Observation 2: AL8 cannot be used for 12RB CCE-to-REG mapping.

Proposal 2: For 15 RB case RLM and BFD, PDCCH BW is reduced to 15 RBs, non-interleaved allocation is used. Additionally, 3 OFDM symbols are used for out-of-sync.

Observation 3: For such changes, SNR increase is less than 1 dB for 8 CCE.

Proposal 3: For 12 RB case, PDCCH BW is reduced to 12 RBs and 8 CCE is replaced with 4 CCEs.

Proposal 4: Keep the existing side condition Es/Iot≥-6 dB for NR target cell detection

Proposal 5: BFD requirement table uses the same parameters as RLM

Observation 4: For DMRS detection, single SSB detection is sufficient for 12-RB PBCH down to –8 dB SNR. Soft combining can be left up to UE implementation

Proposal 6: Feasible operating point / delay is selected based on non-coherent or max correlation results.  Any implementation with similar performance is allowed.

Proposal 7: RAN4 to discuss about MIB reading simulation results for 3km/h and 500km/h, and the impact of soft combining.

Proposal 8: Soft combining is assumed for MIB reading.

Observation 5: Nokia has shown through our simulations that with the use of soft combining performance gains of approximately 6 dB can be achieved

Proposal 9: We propose RAN4 to introduce MIB requirements based on a performance achieved in the Nokia simulations.

Proposal 10: Feasible MIB-reading delay is selected based on soft combining results. Any implementation with similar performance is allowed

Appendix B: Text proposal for RLM

3GPP TS 38.133 V18.2.0 (2023-06) ## 8.1x Radio Link Monitoring for FR1 < 5MHz < text omitted> Table 8.1x.2.1-1: PDCCH transmission parameters for out-of-sync evaluation for 3MHz channel bandwidth Attribute; Value for BLER Configuration #0. DCI format; 1-0. Number of control OFDM symbols; 2 3. Aggregation level (CCE); 4 for 12 PRB 8 for 15 PRB. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 4dB 4dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 4dB 4dB. Bandwidth (PRBs); 12, 15. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Note: SCS = 60KHz is not applicable for FR1.. Table 8.1x.2.1-2: PDCCH transmission parameters for in-sync evaluation 3MHz channel bandwidth Attribute; Value for BLER Configuration #0. DCI payload size; 1-0. Number of control OFDM symbols; 2. Aggregation level (CCE); 8; for RedCap UE with 1Rx branch. 4; for RedCap UE with 2Rx branches.. Ratio of hypothetical PDCCH RE energy to average SSS RE energy; 0dB. Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy; 0dB. Bandwidth (PRBs); 12; 15;. Sub-carrier spacing (kHz); SCS of the active DL BWP. DMRS precoder granularity; REG bundle size. REG bundle size; 6. CP length; Normal. Mapping from REG to CCE; Distributed. Note: SCS = 60KHz is not applicable for FR1..

Table 3: 12 PRB and 15 PRB Requirements for SSB based radio link monitoring

- References

- Egilmez, Zeynep B. Kaykac, et al. "The development, operation and performance of the 5G polar codes." IEEE Communications Surveys & Tutorials 22.1 (2019): 96-122

- R4-2310040, Summary of RRM simulation results for less than 5MHz BW, Moderator (Nokia, Nokia Shanghai Bell)

- 3GPP RAN4 Meeting #107, Incheon, Korea, May 22 – 26, 2023

- R4-2313189, Supporting Simulations on Less than 5MHz RRM requirements, Nokia, Nokia Shanghai Bell, 3GPP RAN4 #108

- R4-2310039, WF on RRM requirements for NR support for dedicated spectrum less than 5MHz for FR1, Moderator (Nokia, Nokia Shanghai Bell)

- R4-2309496 Discussion on Less than 5MHz RRM requirements, 3GPP TSG-RAN WG4 Meeting #107, Nokia, Nokia Shanghai Bell

- R4-2309659 Simulations for NR_FR1_lessthan_5MHz_BW, 3GPP TSG-RAN WG4 Meeting #107, Nokia, Nokia Shanghai Bell