3GPP TSG-RAN WG4 Meeting #108 	R4-2313189

Toulouse, France, August 21-25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM simulation results for less than 5MHz BW

Agenda item:	8.14.4

Document for:	Discussion

Introduction

This contribution presents our supporting simulations for our companion discussion TDoc [1] for the remaining issues on FR1 < 5MHz.

Discussion

Background

This paper presents the simulation results to support our corresponding discussion paper for RAN4#108 regarding FR1<5 MHz RRM requirements definition.

Simulation results for PBCH

Way Forward and Introduction to PBCH Simulations

In this section, we present our simulation results for PBCH, specifically the impact of Soft Combining, as mentioned in the WF.

Issue 1-16: Soft Combining Way forward: Soft Combining - Companies are encouraged to collect simulation results with and without soft combining to assess the performance impacts and make the decision then.

PBCH decoding, TDL C 300ns

Table 1 99% PBCH decoding, SSB samples or SINR needed TDL C 300ns

|  |  | SINR of one shotneeded for 12 PRBs |  | Number of attempts needed for 12 PRBs, SINR-6dB |  | Number of attempts needed for 12 PRBs, SINR-4dB |  | SINR of one shotneeded for 15 PRBs |  | Number of attempts needed for 15 PRBs, SINR-6dB |  | Number of attempts needed for 15 PRBs, SINR-4dB |  | SINR of one shotneeded for 20 PRBs |  | Number of attempts needed for 20 PRBs, SINR-6dB |  | Number of attempts needed for 20 PRBs, SINR-4dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm |  | >2dB |  | 5 |  | 3 |  | 0dB |  | 3 |  | 2 |  | - |  | - |  | - |
| Huawei |  | 11dB |  | - |  | - |  | - |  | - |  | - |  | 3dB |  | - |  | - |
| Ericsson |  | - |  | - |  | - |  | -1dB |  | 5 |  | 4 |  | - |  | - |  | - |
| MTK |  | >0dB |  | NA, >4 |  | 4 |  | - |  | - |  | - |  | -3dB |  | 3(Soft combing) |  | 2 |
| Nokia | - |  | 2(Soft combing) |  | - |  | - |  | 2(Soft combing) |  | - |  | - |  | 1 |  | 1 |  |
| Apple |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |  | - |

PBCH decoding, HST

Table 2 99% PBCH decoding, SSB samples or SINR needed HST

|  | SINR of one shotneeded for 12 PRBs | Number of attempts needed for 12 PRBs, SINR-6dB | Number of attempts needed for 12 PRBs, SINR-4dB | SINR of one shotneeded for 15 PRBs | Number of attempts needed for 15 PRBs, SINR-6dB | Number of attempts needed for 15 PRBs, SINR-4dB | SINR of one shotneeded for 20 PRBs | Number of attempts needed for 20 PRBs, SINR-6dB | Number of attempts needed for 20 PRBs, SINR-4dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm | -2dB | 3 | 2 | -6dB | 1 | 1 | - | - | - |
| Huawei | - | - | - | - | - | - | - | - | - |
| Ericsson | -4.5dB | 2 | 1 | -7dB | 1 | 1 | - | - | - |
| MTK | -3dB | 2(Soft combing) | 2 | - | - |  | - | - |  |
| Nokia | -2.5dB | 6 (3 Soft combing) | 2 | -6dB | 1 | 1 | -8.5dB | 1 | 1 |
| Apple | - | - | - | - | - | - | - | - | - |

PBCH decoding, AWGN

Table 3 99% PBCH decoding, SSB samples or SINR needed AWGN

|  | SINR of one shotneeded for 12 PRBs | Number of attempts needed for 12 PRBs, SINR-6dB | Number of attempts needed for 12 PRBs, SINR-4dB | SINR of one shotneeded for 15 PRBs | Number of attempts needed for 15 PRBs, SINR-6dB | Number of attempts needed for 15 PRBs, SINR-4dB | SINR of one shotneeded for 20 PRBs | Number of attempts needed for 20 PRBs, SINR-6dB | Number of attempts needed for 20 PRBs, SINR-4dB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm | -4dB | 2 | 1 | -6dB | 1 | 1 | - | - | - |
| Huawei | - | - | - | - | - | - | - | - | - |
| Ericsson | -5dB | 2 |  | - | - |  | - | - | - |
| MTK | - | - | - | - | - | - | - | - | - |
| Nokia | -4dB | 4 (2 Soft combing) | 1 | -7dB | 1 | 1 | -9dB | 1 | 1 |
| Apple | - | - | - | - | - | - | - | - | - |

Simulation results for PDCCH

Introduction to PDCCH Simulations

In the following sub-sections, we introduce the simulation results in BLER for various channel types for the performance of PDCCH from our simulation campaign.

Simulation Assumptions

The following table presents the simulation assumptions used in our simulation campaign.

| Carrier frequency | 900 MHz |
| --- | --- |
| Antenna setup | 2x2 |
| Channel model | TDL-C 300ns, TDL-A 30ns, TDL-B 100ns, AWGN |
| UE speed | 3km/h, 500km/h |
| Subcarrier spacing | 15 kHz |
| CORESET allocation | 2 and 3 symbols 24 RB CORESET limited to 12 and15 RBs |
| Receiver assumptions on Tx bandwidth | Receiver detects using same BW as in TX side |
| Payload | 40 bits + 24 bit CRC |

TDL-C 300ns

TDL-A 30ns

TDL-B 100ns

AWGN

PDCCH simulation table with 12 PRB and 24 PRB comparison

| Channel | BW | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Interleaved |  |  |  |
|  |  | BLERin 2% | Delta | BLERout 10% | Delta |
|  |  | [dB] | [dB] | [dB] | [dB] |
| AWGN, 3km/h | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.6 | -6.3 | -2.5 |
|  | 24 PRBs Interl. 8 CCE | -8.2 |  | -8.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.3 | -6.3 | -2.3 |
|  | 24 PRBs Interl. 8 CCE | -7.9 |  | -8.6 |  |
| TDL-A30ns, 3 km/h | 12 PRBs Non-Interl. 4 CCE | -2.4 | -1.5 | -4.6 | -2.2 |
|  | 24 PRBs Interl. 8 CCE | -3.9 |  | -6.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -3.2 | -0.8 | -4.7 | -2.1 |
|  | 24 PRBs Interl. 8 CCE | -4 |  | -6.8 |  |
| TDL-B100ns, 3 km/h | 12 PRBs Non-Interl. 4 CCE | -3.1 | -2.2 | -4.5 | -2.3 |
|  | 24 PRBs Interl. 8 CCE | -5.3 |  | -6.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -2.7 | -2.5 | -4.2 | -2.4 |
|  | 24 PRBs Interl. 8 CCE | -5.2 |  | -6.6 |  |
| TDL-C300ns, 3 km/h | 12 PRBs Non-Interl. 4 CCE | -3.7 | -2.6 | -4.9 | -2.4 |
|  | 24 PRBs Interl. 8 CCE | -6.3 |  | -7.3 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -3.5 | -2.9 | -4.6 | -2.7 |
|  | 24 PRBs Interl. 8 CCE | -6.4 |  | -7.3 |  |
| AWGN, 500 km/h | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.6 | -6.3 | -2.5 |
|  | 24 PRBs Interl. 8 CCE | -8.2 |  | -8.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -5.6 | -2.3 | -6.3 | -2.3 |
|  | 24 PRBs Interl. 8 CCE | -7.9 |  | -8.6 |  |
| TDL-A30ns, 500 km/h | 12 PRBs Non-Interl. 4 CCE | -1.7 | -2.7 | -3.8 | -2.6 |
|  | 24 PRBs Interl. 8 CCE | -4.4 |  | -6.4 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -1.7 | -2.8 | -3.8 | -2.5 |
|  | 24 PRBs Interl. 8 CCE | -4.5 |  | -6.3 |  |
| TDL-B100ns, 500 km/h | 12 PRBs Non-Interl. 4 CCE | -2.4 | -2.7 | -4.2 | -2.6 |
|  | 24 PRBs Interl. 8 CCE | -5.1 |  | -6.8 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -2.4 | -2.8 | -4.1 | -2.6 |
|  | 24 PRBs Interl. 8 CCE | -5.2 |  | -6.7 |  |
| TDL-C300ns, 500 km/h | 12 PRBs Non-Interl. 4 CCE | -2 | -3.8 | -4.3 | -2.9 |
|  | 24 PRBs Interl. 8 CCE | -5.8 |  | -7.2 |  |
|  | 12 PRBs Non-Interl. 4 CCE | -2.2 | -3.4 | -4.4 | -2.6 |
|  | 24 PRBs Interl. 8 CCE | -5.6 |  | -7 |  |

Table 1: PDCCH simulation results for 12 RB

PDCCH simulation table with 15 PRB and 24 PRB comparison

| Channel | PRBs | BW | SNR [dB] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 CCEs |  |  |  | 8 CCEs |  |  |  |
|  |  |  | Interleaved |  |  |  | Interleaved |  |  |  |
|  |  |  | BLERin 2% | Delta | BLERout 10% | Delta | BLERin 2% | Delta | BLERout 10% | Delta |
|  |  |  | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] | [dB] |
| AWGN, | 15 PRBs | 2 OFDM Symbols | -1.6 | -4 | -3 | -3.3 | -5 | -3.2 | -6.3 | -2.5 |
| 3km/h | 24 PRBs |  | -5.6 |  | -6.3 |  | -8.2 |  | -8.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -4.5 | -1.1 | -5.3 | -1 | -6 | -1.9 | -6.9 | -1.7 |
|  | 24 PRBs |  | -5.6 |  | -6.3 |  | -7.9 |  | -8.6 |  |
| TDL-A30ns, | 15 PRBs | 2 OFDM Symbols | -0.5 | -1.2 | -3 | -1.2 | -2.3 | -1.6 | -5 | -1.8 |
| 3 km/h | 24 PRBs |  | -1.7 |  | -4.2 |  | -3.9 |  | -6.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -1.6 | -0.3 | -3.8 | -0.6 | -3.3 | -0.7 | -5.5 | -1.3 |
|  | 24 PRBs |  | -1.9 |  | -4.4 |  | -4 |  | -6.8 |  |
| TDL-B100ns, | 15 PRBs | 2 OFDM Symbols | -0.9 | -1.5 | -2.5 | -1.3 | -3.7 | -1.6 | -5 | -1.8 |
| 3 km/h | 24 PRBs |  | -2.4 |  | -3.8 |  | -5.3 |  | -6.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -1.5 | -0.7 | -3 | -0.7 | -4 | -1.2 | -5.3 | -1.3 |
|  | 24 PRBs |  | -2.2 |  | -3.7 |  | -5.2 |  | -6.6 |  |
| TDL-C300ns, | 15 PRBs | 2 OFDM Symbols | -1.7 | -1.7 | -3.2 | -1.5 | -4.7 | -1.6 | -5.7 | -1.6 |
| 3 km/h | 24 PRBs |  | -3.4 |  | -4.7 |  | -6.3 |  | -7.3 |  |
|  | 15 PRBs | 3 OFDM Symbols | -2.7 | -0.8 | -4 | -0.7 | -5 | -1.4 | -5.8 | -1.5 |
|  | 24 PRBs |  | -3.5 |  | -4.7 |  | -6.4 |  | -7.3 |  |
| AWGN, | 15 PRBs | 2 OFDM Symbols | -1.6 | -4 | -3 | -3.3 | -5 | -3.2 | -6.3 | -2.5 |
| 500 km/h | 24 PRBs |  | -5.6 |  | -6.3 |  | -8.2 |  | -8.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -4.5 | -1.1 | -5.3 | -1 | -6 | -1.9 | -6.9 | -1.7 |
|  | 24 PRBs |  | -5.6 |  | -6.3 |  | -7.9 |  | -8.6 |  |
| TDL-A30ns, | 15 PRBs | 2 OFDM Symbols | -0.2 | -1.7 | -2.5 | -1.5 | -2.2 | -2.2 | -4.4 | -2 |
| 500 km/h | 24 PRBs |  | -1.9 |  | -4 |  | -4.4 |  | -6.4 |  |
|  | 15 PRBs | 3 OFDM Symbols | -1.3 | -0.7 | -3.3 | -0.7 | -2.9 | -1.6 | -4.8 | -1.5 |
|  | 24 PRBs |  | -2 |  | -4 |  | -4.5 |  | -6.3 |  |
| TDL-B100ns, | 15 PRBs | 2 OFDM Symbols | -0.7 | -1.7 | -2.7 | -1.5 | -3 | -2.1 | -4.8 | -2 |
| 500 km/h | 24 PRBs |  | -2.4 |  | -4.2 |  | -5.1 |  | -6.8 |  |
|  | 15 PRBs | 3 OFDM Symbols | -1.8 | -0.6 | -3.5 | -0.6 | -1.7 | -3.5 | -5.2 | -1.5 |
|  | 24 PRBs |  | -2.4 |  | -4.1 |  | -5.2 |  | -6.7 |  |
| TDL-C300ns, | 15 PRBs | 2 OFDM Symbols | -0.8 | -1.8 | -2.8 | -1.6 | -3 | -2.8 | -5 | -2.2 |
| 500 km/h | 24 PRBs |  | -2.6 |  | -4.4 |  | -5.8 |  | -7.2 |  |
|  | 15 PRBs | 3 OFDM Symbols | -2 | -0.7 | -3.7 | -0.7 | -3.8 | -1.8 | -5.5 | -1.5 |
|  | 24 PRBs |  | -2.7 |  | -4.4 |  | -5.6 |  | -7 |  |

Table 1: PDCCH simulation results for 15 RBs

- Conclusion

Within this contribution we present simulation results supporting the definition of RRM requirements for FR1 < 5MHz.

- References

- R4-2313188, Discussion on Less than 5MHz RRM, Nokia, Nokia Shanghai Bell, 3GPP RAN4 #108