# 3GPP TSG-RAN WG4 Meeting # 108			R4-2313190

# Toulouse, France, August 21 – August 25, 2023

Source:	Sony

Title:	Views on UL 256-QAM for FR2-1

Agenda item:	8.6.2

Document for:	Approval

- Background

Supporting uplink 256QAM in FR2-1 was further discussed in RAN4 #107, and a WF has been approved [1]. The main remaining issue is the MPR values and PTRS for the EVM test. In this contribution, we provide an analysis on the MPR value and also the PTRS configuration.

- Consideration of MPR Performance

To enable the UL 256 QAM in FR2-1, the corresponding MPR values need to be added. The output power on the power amplifier can be backed off to obtain better linearity and reduce the EVM. On the other hand, to obtain a meaningful performance in the network, the MPR values need to be limited so that the UE can reach reasonable EIRP levels in a real network scenario.

Proposal 1: The MPR of UL 256 QAM needs to be confined so that the UE can reach reasonable EIRP levels and dynamic range in a real network scenario.

Taking power class 1 as an example, the MPR value for 64QAM is already as high as 9 dB in FR2-1 in the worst-case scenario. Therefore, the actual peak EIRP of UEs that pass the 3GPP test for 64 QAM can, in practice, be as low as:

40 dBm [PC1 minimum peak EIRP] – 9 dB [MPR] –5 [Pcmax tolerance] – 3 dB [TT] = 23 dBm [EIRP for 64 QAM].

For 256 QAM, the worst-case MPR could be expected to be larger than 9 dB or even 10 dB at the outer RB location. In this case, the EIRP for 256 QAM is:

40 dBm [PC1 minimum peak EIRP] – X dB [MPR] –7 [Pcmax tolerance] – 3 dB [TT] = Y dBm [EIRP for 256 QAM].

Based on the discussion in the last RAN4 meeting, the lowest proposed EIRP of UL 256QAM in PC1 is 18 dBm, which limits the MPR for UL 256 QAM not to exceeding 12 dB. In other words, if the specified MPR value for 256QAM is larger than 12 dB, this may result in no dynamic range of device operation in the field.

Observation 1: with the assumption of 18 dBm minimum EIRP, it is possible that no dynamic range is available if the corresponding MPR is more than 12 dB.

As mentioned earlier, the largest MPR for 64QAM in FR2-1 is 9 dB. Therefore, it is proposed that the MPR value for 256QAM should be in the range of 0-3 dB higher than 64QAM.

Proposal 2: It is proposed that the MPR for UL 256 QAM shall not exceed 3 dB more than 64QAM.

There have been a few concerns about adopting the MPR confinement method as this is not the conventional method to define the MPR in previous meetings. However, as we can see, supporting 256 QAM in UL requires extraordinary linearity of the transmitter and faces an extremely tight link budget compared to other modulation orders; it is no longer feasible to only consider the UE implementation without including the impact analysis on the network performance. Therefore, it is necessary to cap the MPR value with a reasonable value to guarantee the network performance.

Observation 2 it is no longer feasible to only consider the UE implementation without including the impact analysis on the network performance in FR2 due to the extremely tight link budget when it comes to MPR requirements.

Observation 3: It is necessary to cap the MPR value with a reasonable value to guarantee the network performance.

- Simulation of MPR

To investigate the feasibility of confining the MPR for UL 256QAM, the MPR for CP-OFDM for 100 MHz and 400 MHz BW are simulated for PC1 with 120 kHz SCS. The power amplifier is modeled based on a couple different III-V semiconductor PA models and the RB allocation is referred from contributions in last RAN4 meeting [2]. Other simulation assumptions are listed below and the simulated MPR is shown in Table.1

- The phase noise model proposed by QC has been adopted in the simulation [1], where the EVM budget for Phase noise + IQ imbalance is -30.8 dB.

- L-PTRS = 1 K-PTRS =2 are adopted as PTRS configuration for CP-OFDM.

Table 1. The MPR simulation results for CP-OFDM with 120 kHz SCS for PC1

| Waveform | BWchanne | Outer RB allocations | Inner RB allocations |  |
| --- | --- | --- | --- | --- |
|  |  |  | Region 1 | Region 2 |
| CP-OFDM | 100 MHz | 10.4 | 11.4 | 11.2 |
|  | 400 MHz | 11 | 10.2 | 10.8 |

Based on the simulated results, it can be observed that for 400 MHz, the MPR values are well below the “64QAM MPR + 3dB” range. Though not all cases (inner region of 100 MHz) can be well-fitted within the proposed range, the values are not far from the proposed limits for narrower BW. In addition, it is worth mentioning that no advanced linearity technologies, e.g., DPD or APD, have been included in such a simulation. Therefore, it can be expected that the MPR confinement, as mentioned above, is feasible to be used to define the MPR values for 256 QAM.

Observation 4: It is feasible for implementations to meet the proposed confinement range.

- Consideration of PTRS

The PTRS configuration for EVM test is still open. Based on our simulation, a denser PTRS configuration will not jeopardize the CP-OFDM but may have a negative effect on DFTs-OFDM. This may be caused that when the phase noise level is higher, the PTRS may contribute to the noise level and affect the channel estimation and equalization process.

Observation 5: a denser PTRS configuration does not seem to reduce the EVM performance of CP-OFDM but may affect the DFTs-OFDM.

From the testing aspect, considering RAN4 sets minimum requirements for UE RF, which shall also provide a benchmark performance of UE in the field, a UE-declared PTRS configuration in the test may lead to incomparable results between different UEs.  Therefore, it is recommended to adopt a fixed PTRS configuration in the conformance test rather than a dynamic configuration of PTRS. The feasible configuration of PTRS can be further discussed, though.

Observation 6: A UE-declared PTRS configuration in the test may lead to incomparable results between different UEs.

Proposal 3: It is recommended to adopt a fixed PTRS configuration rather than a dynamic configuration of PTRS in the conformance test, while the feasible configuration of PTRS can be further discussed.

- Conclusion

In this contribution, we have discussed the evaluation of MPR for UL 256QAM in FR2-1. The following observations and proposals have been made:

Observation 1: with the assumption of 18 dBm minimum EIRP, it is possible that no dynamic range is available if the corresponding MPR is more than 12 dB.

Observation 2 it is no longer feasible to only consider the UE implementation without including the impact analysis on the network performance in FR2 due to the extremely tight link budget when it comes to MPR requirements.

Observation 3: It is necessary to cap the MPR value with a reasonable value to guarantee the network performance.

Observation 4: It is feasible for implementations to meet the proposed confinement range.

Observation 5: a denser PTRS configuration does not seem to reduce the EVM performance of CP-OFDM but may affect the DFTs-OFDM.

Observation 6: A UE-declared PTRS configuration in the test may lead to incomparable results between different UEs.

Proposal 1: The MPR of UL 256 QAM needs to be confined so that the UE can reach reasonable EIRP levels and dynamic range in a real network scenario.

Proposal 2: It is proposed that the MPR for UL 256 QAM shall not exceed 3 dB more than 64QAM.

Proposal 3: It is recommended to adopt a fixed PTRS configuration rather than a dynamic configuration of PTRS in the conformance test, while the feasible configuration of PTRS can be further discussed.

- References

R4-2310260	WF on FR2-1 UL 256 QAM 	Xiaomi

R4-2304634 	MPR evaluation results for FR2-1 UL256QAM PC1 and PC2	LG Electronics