3GPP TSG-RAN WG4 Meeting #108	R4-2313197

# Toulouse, France, 21–25 August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3555 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_NTN_solutions-Core] CR on R17 NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-08-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For R-17 NTN, FR2 is not allowed to apply NTN. Furthermore, the NR-DC scenario is not allowed to apply NTN. There are some unnecessary description regarding FR2 and NR-DC about NTN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the unnecessary description regarding FR2 and NR-DC about NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Wrong description related to NTN exists. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2C.2.3 6.2C.3.1 6.2C.3.2 8.1C.7 9.1C.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st changes >>

6.2C	RRC Connection Mobility Control for Satellite Access

6.2C.2	Random access for satellite access

6.2C.2.3	Requirements for 2-step RA type

The UE shall select the type of random access at initiation of the random access procedure based on network configuration, as specified in clause 5.1.1 in TS 38.321 [7].

The UE shall have capability to calculate MsgA PRACH transmission power according to the PRACH power formula defined in clause 7.4 of TS 38.213 [3] and the MsgA PUSCH power formula of clause 7.1.1 of TS 38.213 [3] and apply this power level at the first MsgA or additional MsgA repetitions. The absolute power applied to the first preamble shall have an accuracy as specified in Table 6.3.4.2-1 of TS 38.101-1 [18] for frequency range 1. The relative power applied to additional preambles shall have an accuracy as specified in Table 6.3.4.3-1 of TS 38.101-1 [18] for frequency range 1 .

The UE shall switch to 4-step RA type procedure if the MsgA transmission counter has exceeded msgA-TransMax, if configured, as specified in clause 5.1.4a of TS 38.321 [7]. The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell as specified in clause 5.1.4a in TS 38.321 [7].

The requirements in this clause apply for UE in SA operation mode .

## << End of 1st change >>

<< Start of 2nd changes >>

6.2C	RRC Connection Mobility Control for Satellite Access

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access

6.2C.3.1	Introduction

This clause contains requirements on the UE regarding RRC connection release with redirection procedure. RRC connection release with redirection is initiated by the RRCRelease message with redirection to NR from NR specified in TS 38.331 [2]. The RRC connection release with redirection procedure is specified in clause 5.3.8 of TS 38.331 [2].

6.2C.3.2	Requirements

6.2C.3.2.1	RRC connection release with redirection to NR

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detetable when for each relevant SSB, the side conditions should be met that,

-	the conditions of SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding NR Band are fulfilled.

-	TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

-	Tidentify-NR: It is the time to identify the target NR cell and depends on the FR of the target NR cell. It is defined in Table 6.2C.3.2.1-1. Note that Tidentify-NR = TPSS/SSS-sync + Tmeas, in which TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

-	TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released. TRACH: It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

-	Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise,

-	there is no requirement if the SSB transmission periodicity is larger than 20ms.

The requirements in this clause apply provided that the ephemeris information provided by the serving cell for the target cell is valid during time delay (Tconnection_release_redirect_NR).

Table 6.2C.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| FR of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | K_satellite * MAX (680 ms, 11 x Trs) |
| Note 1: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. Note 2: K_satellite is defined in clause 9.3C.4. |  |

## << End of 2nd change >>

<< Start of 3rd changes >>

8.1C	Radio Link Monitoring for Satellite Access

8.1C.7	Scheduling availability of UE during radio link monitoring

When the reference signal to be measured for RLM has different subcarrier spacing than PDSCH/PDCCH , there are restrictions on the scheduling availability as described in the following clauses.

## << End of 3rd change >>

<< Start of 4th changes >>

9.1C	General measurement requirement for SAN

Editor’s note: Applicability of frequency range, CA, DA, duplex mode, inter-RAT measurement, etc is subject to updates/changes based on the scope of the corresponding WID.

Editor’s note: Terminology will be further clarified and selected between, e.g. NTN and satellite access, based on further agreements.

9.1C.1	Introduction

This clause contains general requirements on the UE regarding measurement reporting in RRC_CONNECTED state. The requirements are split in intra-frequency, inter-frequency and L1-RSRP measurements requirements. These measurements may be used by the NG-RAN. The measurement quantities are defined in TS38.215 [4], the measurement model is defined in TS38.300 [10], TS37.340 [17] and measurement accuracies are specified in clause 10. Control of measurement reporting is specified in TS 38.331 [2].

In the requirements of clause 9, the exceptions for side conditions apply as follows:

-	for the UE configured with only PCell, which is served by SAN, the applicable exceptions for side conditions are specified in Annex B, clause [B.3.x.x] for UE supporting satellite access operation.

## << End of 4th change >>