3GPP TSG-RAN WG4 Meeting #108	R4-2313207

Toulouse, France, 21-25 August, 2023

Source:	Ericsson

Title:	UE Type behavior

Agenda item:	8.11.2

Document for:	Discussion

# 1	Introduction

In RAN4#107 a WF was agreed [1]:

… < Issue 2-1-1: Whether to introduce an optional new IE for Type 2 NR-CA > Agreement: - Introduce an optional new IE for NR CA, [intraBandNonColocatedCA-r18] for UE supporting type 2 UE capability on NR CA operation. … < Issue 2-2-2: How UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2 > - Proposals - Option 1-1: - (Common for all options) Default UE configuration is Type 1 UE if new UE capability is not signalled or supported. - Default UE configuration shall be Type 1 UE if capable to support new UE capability for NR-CA. - Configure UE between Type 1 and Type 2 with the existing RRC Reconf. - BS can switch between Type 1 and Type 2 using by setting ServingCellConfig.maxMIMO-Layers= 4, and 2, respectively during existing RRC Reconf. - If there are critical and technical issues on reusing existing RRC Reconf., RAN4 will discuss them in the future release or the maintenance part (i.e. Rel-18 for NR-CA and Rel-16 for EN-DC). - Option 1-2: - Option 2-1: - Option 2-2: - (Common for all options) Default UE configuration is Type 1 UE if new UE capability is not signalled or supported. - Default UE configuration shall be Type 2 UE if capable to support new UE capability for NR-CA. - Configure UE between Type 1 and Type 2 with a new BS signalling. …

In this contribution we investigate and state our positions regarding options 1-1 and 1-2.

# 2	Discussion

In our view we can base a UE with [intraBandNonColocatedCA-r18] for UE supporting type 2 UE capability on NR CA operation, along the lines of original capability interBandMRDC-WithOverlapDL-Bands-r16 for Dual Connectivity. The reason for this is simplicity and to give priority to the non-colocation objective of the WID also for CA. This means that a UE stating [intraBandNonColocatedCA-r18] or interBandMRDC-WithOverlapDL-Bands-r16 is limited to rank 2 MIMO.

Proposal 1: A UE stating [intraBandNonColocatedCA-r18] is defined in the same way as existing interBandMRDC-WithOverlapDL-Bands-r16 capability. If a UE states it is [intraBandNonColocatedCA-r18] capable then that UE is capable of CA with large power imbalance and rank = 2 MIMO on its component carriers.

When it comes to the Issue 2-2-2: How UE supporting Type 2 NR-CA/EN-DC behaves between Type 1 and Type 2, then we prefer to define the same behavior for Type switching for both the new [intraBandNonColocatedCA-r18] capability and the existing interBandMRDC-WithOverlapDL-Bands-r16 capable UE. It is possible to define this entirely on existing procedures.

The procedure works like this:

- The UE signals that it is type 2 capable.

- If the UE is configured with a secondary cell then the UE will support large power imbalance at UE RX and RAN = 2 MIMO per component carrier.

- If the UE is reconfigured to drop the secondary cell then the remaining primary carrier can support MIMO RANK > 2.

Proposal 2: Define UE supporting Type 2 NR-CA/EN-DC Type 1 and Type 2 switch based on existing procedures:

- The UE signals that it is type 2 capable.

- If the UE is configured with a secondary cell then the UE will support large power imbalance at UE RX and RANK = 2 MIMO per component carrier.

- If the UE is reconfigured to drop the secondary cell then the remaining primary carrier can support MIMO RANK = 4.

# 3 	Summary

Proposal 1: A UE stating [intraBandNonColocatedCA-r18] is defined in the same way as existing interBandMRDC-WithOverlapDL-Bands-r16 capability. If a UE states it is [intraBandNonColocatedCA-r18] capable then that UE is capable of CA with large power imbalance and rank = 2 MIMO on its component carriers.

Proposal 2: Define UE supporting Type 2 NR-CA/EN-DC Type 1 and Type 2 switch based on existing procedures:

- The UE signals that it is type 2 capable.

- If the UE is configured with a secondary cell then the UE will support large power imbalance at UE RX and RAN = 2 MIMO per component carrier.

- If the UE is reconfigured to drop the secondary cell then the remaining primary carrier can support MIMO RANK = 4.

# References

[1] 		R4-2310300, WF on UE RF requirements for intra-band non-collocated EN-DC/NR-CA deployment, KDDI.