3GPP TSG-RAN WG4 Meeting # 108 	R4-2313210

Toulouse, France, August 21 – August 25, 2023

Source: 	vivo

Title: 	[NR_newRAT-Core] Discussion on FR2 ACS/IBB testing

Agenda Item:	4.1

Document for:	Approval

- Introduction

In R15, it is agreed that [1]:

For testing on each polarization, the levels of the wanted signal and the interferer are reduced by 3dB.

This agreement is not correct anymore since the calculation method for EIS was changed, and in this contribution, we further analyze this problem.

- Discussion

The agreement described above is based on the EIS formula below:

EIS = [1/EISq +1/EISf]-1

Based on this formula, a virtual polarization gain is incorporated. Since the ACS/IBB is verified on each polarization sequentially, to keep the same ratio between signal power and interferer power, as shown in the following:

Figure 1 Description in Annex K1.8 Blocking measurement procedure

It makes sense that to reduce 3 dB for both signal and interferer during the test. For example, if EISq = -80 dBm and EISf= -80 dBm, based on the formula above, the EIS = -83 dBm which is 3 dB lower than each polarization.

However, in the current spec, the EIS formula is changed to:

EIS = 2* [1/EISq +1/EISf]-1

This new formula is named “average EIS” and the polarization gain is removed, if EISq = -80 dBm and EISf= -80 dBm, and based on the new formula, the EIS = -80 dBm which is equal to each polarization, so under this situation, the 3 dB reduction for signal and interferer is not needed anymore.

Observation: Based on the new formula for “average EIS”, the 3 dB reduction for signal and interferer in ACS/IBB testing is not needed.

Proposal: Send LS to RAN5 to remove the 3dB reduction in ACS/IBB testing.

A draft LS is provided in [2].

- Conclusion

In this contribution, we analyze a legacy issue in ACS/IBB testing.

Observation: Based on the new formula for “average EIS”, the 3 dB reduction for signal and interferer in ACS/IBB testing is not needed.

Proposal: Send LS to RAN5 to remove the 3dB reduction in ACS/IBB testing.

- References

- R4-1804846 Discussion on testing of ACS & IBB in FR2, Rohde & Schwarz, RAN4#86bis

- R4-230xxxx draft LS on ACS/IBB testing, vivo, RAN4#108