3GPP TSG-RAN WG4 Meeting #107		   R4-2313214

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.19.2.2.1

Source:	Qualcomm Inc

Title:	TP to TR 38.858: Feasibility of FR1 wide area BS aspects

Document for:	Approval

# Introduction

In this document we provide our text proposals for BS feasibility aspects based on the skeleton in the WF agreed in RAN4#106-bis-e (R4-2306005).

# Text proposal

In RAN4+106 RAN4 has agreed on the following work split:

| No. | Section for TR 38.858 | Responsible company |
| --- | --- | --- |
| 1 | 10.1 Background for analysis | Ericsson |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects | Samsung |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects | Nokia |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects | CATT |
| 5 | 10.5 Feasibility of FR2 BS aspects | Huawei |
| 6 | 10.6. FR1 Feasibility of UE aspects | MediaTek |
| 7 | 10.7 FR2 Feasibility of UE aspects | Qualcomm |
| 8 | 10.8 Summary | CMCC |
| 9 | 11.1 Impact on BS RF requirements | ZTE |
| 10 | 11.2 Impact on UE RF requirements | Qualcomm |
| 11 | 12 Adjacent channel co-existence evaluation results | Samsung |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum | CableLabs |
| 13 | Annex <D>: Adjacent channel co-existence evaluation | Ericsson |

Begin changed section ******************************************************

## 10.2 Feasibility of FR1 Wide Area BS aspects

### 10.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 10.2.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

The self-interference analysis of an SBFD-capable gNB within RAN4 was carried out based on a framework that considers the different techniques for self-interference cancellation as well as their challenges. More details about those techniques are provided in this section. In summary, Table 1 shows that it is feasible to meet the 1 dB sensitivity degradation for FR1 SBFD-capable BS considering the different self-interference mitigation stages (i.e., spatial isolation, frequency isolation, beam nulling, and digital cancellation).

Table 1 Self-interference cancellation framework for FR1 WA BS

| BS class |  |  |  | Wide Area BS |
| --- | --- | --- | --- | --- |
| BS TX Power dBPTX = ① dBm |  |  |  | 49 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 80 dBc |
|  |  | Spatial isolation techniques used |  | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation |
|  | Beam nulling dBαSI-beam = ④ dBc |  |  | 15 dBc |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 dBm (=①-②-③-④) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 15 dBc |
|  |  | Frequency isolation techniques |  | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Not a significant contributor on the gNB Rx capability. |
|  |  |  | Rx IM3 contribution (dBm) |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model. |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dB |
| Overall RSIC capability dBRSIC |  |  |  | 155 dB (②+③+④+⑦) |
| Noise floor ⑩dBm |  |  |  | -96 dBm/20 MHz @ 5dB noise figure |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 151 dBc |
| SBFD configuration |  |  |  | DUD |
| Guardband assumption (if exist) |  |  |  | 5 PRBs |
| bandwidth over which suppression is achieved |  |  |  | 100MHz |

### 10.2.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

### 10.2.1.2.1	[Company Name]

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.2.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

### 10.2.1.2.1	Qualcomm

To enable proper reception of the uplink signal at the gNB receiver with simultaneously transmission DL signal, gNB should mitigate the direct self-interference ‘leakage’ and any significant clutter reflections. The self-interference could be mitigated by different techniques such as spatial isolation, analog subband filter, analog interference cancellation, beamforming and digital interference cancellation. In the following, we discuss in detail the knobs for gNB transceiver that enable the mitigation of both component of self-interference, namely direct leakage and clutter reflections.

Antenna techniques and spatial isolation

For SBFD deployments, gNB antenna configurations should be based on two panels configuration with split of the antenna elements for simultaneous downlink transmission and uplink reception as shown in Figure 1. on the other hand, for legacy TDD deployments, gNB antenna configuration is based on single panel for downlink transmission or uplink reception. With the split panel architecture, the gNB can enable larger spatial isolation is an essential component to mitigate self-interference. In addition, the physical separation between the two panels could be used to add electro-magnetic spatial duplexer that enhances the spatial isolation between the panels.

Figure 1 gNB antenna/panels configuration in TDD and SBFD modes

RF measurements for the Tx-Rx spatial has been conducted and results are shown in Figure 2. Each curve represents the spatial isolation measured between all transmit chains of one array to one receiver chain of the other array. This includes the near field transmit and receive antenna gains. The results show more than 80 dB of isolation is achieved at the band of interest.

Figure 2 RF measurements of Tx-Rx spatial isolation between for FR1

Frequency isolation

DL and UL transmissions can be separated in the frequency domain via multiplexing of the DL and UL using non-overlapping DL and UL sub-bands. As a result, large frequency isolation for the UL signal reception is attained as shown Figure 3. For RAN4 further considerations, the frequency isolation represents the ratio of the power of non-linear leakage into the UL subband to the power of the DL signal at the DL subband, which can be approximated by the ACLR requirements specified by RAN4. RAN1 has requested RAN4 to provide value range for the frequency isolation capability of the gNB as well as the accompanying assumptions to those values.

Figure 3 Frequency isolation

A guardband may be needed at the gNB to protect UL reception within the UL subband and reduce the impact of self-interference. In some scenarios, depending on the gNB implementation, a very small guardband or even no guardband may be needed at all. However, from UE perspective, given that there is no UE selectivity, a guardband may be needed to protect the DL reception from the inter-UE CLI. To further analyse this, 80 MHz system bandwidth, the 60 MHz DL subband is allocated with 161 RBs (starting from first RBs at band edge) and the 20 MHz UL subband is allocated with 51 RBs. A guard band of 5RBs in between UL and DL subband. The Tx waveform is pushed to the PA to derive max Tx power of 47 dBm. The subband frequency isolation is defined at the ratio between the power leakage within the 20 MHz UL subband as compared to the transmit signal power within the 60 MHz DL subband as shown in Figure 4.

Figure 4 PSD of DL waveform and frequency isolation

Beam isolation and beamforming/nulling

In FR1, the DL precoder and UL combiner weights could be optimized to provide some beamform nulling for the clutter and/or self-interference. The massive MIMO antenna has large number of degrees of freedom in both digital and analog (i.e., hybrid beamforming) that provide the ability to create some spatial nulls. Beamforming nulling is an efficient technique for clutter mitigation.

Digital self-interference mitigation

The nonlinearities introduced within the gNB front’s end due to non-ideal components of the Tx chain will lead to residual non-linear self-interference that cannot be fully captured in the RF or analog domain due to the associated high complexity, high sensitivity of the canceler and the system’s stability. In this regard, leveraging adaptive filtering and non-linear modeling of the residual self-interference to accurately model and cancel the residual self-interference is performed to provide additional mitigation in the digital domain and enable higher MCS. With the knowledge of the DL samples and the non-linear model, an adaptive filter can be used to synthesize the non-linear leakage and cancel it out from the Rx signal as shown in Figure 5. This technique can be used for cancellation of both self-interference and clutter echo by having multiple taps cancellation.

Figure 5 Digital self-interference cancellation

### 10.2.1.2.2	[Company Name]

### 10.2.1.2.3	[Company Name]

### 10.2.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

Based on the provided self-interference analysis in Section 10.2.2.1, it can be concluded that with spatial isolation, frequency separation between the subband, and digital IC cancellation, it is feasible to meet the 1 dB receiver desensitization target for FR1 wide area base stations operating with maximum Tx power equals 49 dBm.

End changed section ****************************************************************