3GPP TSG-RAN WG4 Meeting #107		   R4-2313215

Incheon, KR, May 22 – May 26, 2023

Agenda item:			8.19.2.2.2

Source:	Qualcomm Inc

Title:	TP to TR 38.858: Feasibility of FR2 wide area BS aspects

Document for:	Approval

# Introduction

In this document we provide our text proposals for BS feasibility aspects based on the skeleton in the WF agreed in RAN4#106-bis-e (R4-2306005).

# Text proposal

In RAN4+106 RAN4 has agreed on the following work split:

| No. | Section for TR 38.858 | Responsible company |
| --- | --- | --- |
| 1 | 10.1 Background for analysis | Ericsson |
| 2 | 10.2 Feasibility of FR1 Wide Area BS aspects | Samsung |
| 3 | 10.3 Feasibility of FR1 Medium Range BS aspects | Nokia |
| 4 | 10.4 Feasibility of FR1 Local Area BS aspects | CATT |
| 5 | 10.5 Feasibility of FR2 BS aspects | Huawei |
| 6 | 10.6. FR1 Feasibility of UE aspects | MediaTek |
| 7 | 10.7 FR2 Feasibility of UE aspects | Qualcomm |
| 8 | 10.8 Summary | CMCC |
| 9 | 11.1 Impact on BS RF requirements | ZTE |
| 10 | 11.2 Impact on UE RF requirements | Qualcomm |
| 11 | 12 Adjacent channel co-existence evaluation results | Samsung |
| 12 | 13 Regulatory aspects for deploying the duplex enhancements in TDD unpaired spectrum | CableLabs |
| 13 | Annex <D>: Adjacent channel co-existence evaluation | Ericsson |

Begin changed section ******************************************************

## 10.4 Feasibility of FR2 Wide Area BS aspects

### 10.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

### 10.4.1.1	Summary table for self-interference analysis

Editor's note: This section captures the summary table which is based on self-interference analysis framework.

Similar to FR1 self-interference analysis, FR2 SBFD-capable gNB feasibility is analyzed here considering the different self-interference cancellation techniques such as spatial isolation, frequency isolation, beamforming and digital interference cancellation. The resulting framework is provided in Table 1 where it can be concluded that it is feasible to meet the 1 dB receiver desensitization target for FR2 wide area base stations.

Table 1 Self-interference cancellation framework for FR2 WA BS

| BS class |  |  |  | Wide Area BS |
| --- | --- | --- | --- | --- |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 28 dBc |
|  |  | Frequency isolation techniques used |  | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 85-95 dBc |
|  |  | Spatial isolation techniques used |  | Two separate panels with added electro-magnetic spatial duplexer for additional cancellation |
|  | Beam nulling dBαSI-beam = ④ dBc |  |  | 5-10 dBc |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -88 dBm (=①-②-③-④) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 15 dBc |
|  |  | Frequency isolation techniques |  | Filtering (does not protect most of the receiver. Right in front of the ADC, by the time blocker is there, damage already has been done). |
|  |  | RX IMD | Rx IIP3 capability (dBm) | Similar conclusion as FR1 (i.e., IIP3 and IM3 are not dominant). |
|  |  |  | Rx IM3 contribution (dBm) |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Noise figure can be modeled as a function of total input power (signal + jammer) with a piecewise linear model. |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10 dB |
| Overall RSIC capability dBRSIC |  |  |  | 128 dB (②+③+④+⑦) |
| Noise floor ⑩dBm |  |  |  | -88 dBm/40 MHz @ 10dB noise figure |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 124 dBc |
| SBFD configuration |  |  |  | DUD |
| Guardband assumption (if exist) |  |  |  | 5 PRBs |
| bandwidth over which suppression is achieved |  |  |  | 200MHz |

### 10.4.1.2	Feasibility study on self-interference

Editor's note: This section captures the feasibility study on self-interference based on individual companies’ analysis.

### 10.4.1.2.1	[Company Name]

Editor's note: Individual company may provide the analysis assumption/configuration used for the corresponding analysis summarized in 10.2.1.1. Additionally, the views on the preference/views on component technology and corresponding trade-off can be provided and analysed.

### 10.4.1.2.1	[Qualcomm]

In the following, we discuss in detail the knobs for FR2 gNB transceiver that enable the mitigation of both component of self-interference, namely direct leakage and clutter reflections.

Antenna techniques and spatial isolation

Similar to FR1, FR2 gNB Radio unit architecture with two physically separated panels for simultaneous transmission and reception enable large spatial isolation. To highlight this more, RF measurements for the Tx-Rx spatial has been conducted at 28 GHz frequency with two separate panels. The Tx and Rx measurement setup of the full duplex antenna array is shown in Figure 1. This measurement setup is on top of the building roof with antenna pointing to the sky, in which case could be without clutter impact or with negligible clutter impact. In this setup, the Tx and Rx beam sweeping is synchronized which is the worst-case scenario - without including clutter. The measurement results, presented in Figure 2, show at least 80-90 dB spatial isolation can be achieved between the two Tx and Rx panels. If the antenna array center-to-center distance is 65 cm, the spatial isolation could be achieved at -86.9 dB or better.

Worse case isolation -86.9dB.

Figure 1 Measurement setup for Tx-Rx spatial isolation of the full duplex antenna array at 28 GHz

Figure 2 RF measurements of Tx-Rx spatial isolation between two subarrays for FR2

Beam isolation and beamforming/nulling

In FR2, spatially isolated and narrow Tx and Rx beam could be selected to provide extra ‘beam’ isolation, which is a combined factor with the antenna isolation. For direct leaked self-interference, it is less related to the beam direction although there is still some dependency. However, for clutter, the signal transmitted from the Tx panel goes through the wireless medium, scattered by the reflectors and then gets back to the Rx panel, which generally has longer delay compared with direct leaked self-interference. The clutter is direction specific, in which case proper selection of Tx and Rx beam pair can alleviate such clutter impact. For FR2, clutter measurements have been conducted for a typical conference room.  The measurement setup and results are shown in Figure 3.  In the measurement setup, the Tx and Rx beam sweeping is synchronized, which is the worst-case scenario.

Figure 3 Clutter measurements for FR2 indoor deployments

Digital self-interference mitigation

In FR2, non-linear interference cancellation (NLIC) measurements have been conducted at 60 GHz for a typical conference room setup. For FR2, the measurement results are shown in for both low MCS and high MCS showing that NLIC could provide ~10 dB improvement on SNR performance.

### 10.4.1.2.2	[Company Name]

### 10.4.1.2.3	[Company Name]

### 10.4.1.3	Conclusion

Editor's note: This section captures the conclusion for feasibility study on self-interference based on RAN4 agreement.

Based on the provided self-interference analysis in Section 10.4.2.1, it can be concluded that with spatial isolation, frequency separation between the subband, and digital IC cancellation, it is feasible to meet the 1 dB receiver desensitization target for FR2 wide area base stations operating with maximum Tx power equals 49 dBm.

End changed section ****************************************************************