3GPP TSG-RAN WG4 Meeting #108			    R4-2313216

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.19.2.2

Source: 	Qualcomm Incorporated

Title: 	SBFD coexistence simulation results

Document for:	Approval

# Introduction

In RAN4#107, additional fine tuning of the adjacent coexistence scenarios, simulation parameters, and discussions timeline SBFD adjacent channel coexistence studies were carried out and a WF was approved for future meetings [1]. In this paper, we present updated simulation results, following the agreements in [1] and further discuss the impact of SBFD deployments on gNB RF requirements considering self-interference, inter-subband CLI, and other coexistence aspects.

# Discussion

As agreed in RAN4#106-bis-e [2], further scoping of the deployment scenarios and transmission configurations for the victim and the aggressor networks have been agreed for SBFD coexistence work as shown below, where it was agreed to focus DU configuration to progress the work as shown in Table 1.

Table 1 Agreed SBFD coexistence scenarios in RAN4#106 [1].

| Victim | Aggressor | Figures: Aggressor(left) and Victim(right) | Aggressor baseline | Priority |
| --- | --- | --- | --- | --- |
| NR TDD DL | SBFD (DU) | Case 1 | NR TDD DL | High |
| NR TDD UL | SBFD (DU) | Case 2 | NR TDD UL | Low |
| SBFD (DU) | NR TDD DL | Case 3 | No system in adjacent channel | High |
| SBFD(DU) | NR TDD UL | Case 4 |  | Low |

The agreed scenarios for SBFD co-existence study along with their priority based on the agreements in [1] are shown Table 2.

Table 2 Scenarios for SBFD co-ex study.

| FR | Scenario No. | Deployment Scenario1 (Aggressor -> Victim) | Priority |
| --- | --- | --- | --- |
| FR1 (4GHz) | 1 | Urban Macro -> Urban Macro | High |
|  | 2 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 3 | Indoor -> Indoor | Low |
|  | 4 | UMa-to-UMi | Note 5 |
|  | 5 | UMi-to-UMi | Note 6 |
| FR2 (30GHz) | 6 | Urban Macro -> Urban Macro | High |
|  | 7 | Urban Hotspot -> Urban Hotspot | Note 4 |
|  | 8 | Urban Micro -> Urban Micro | Low |
|  | 9 | Indoor -> Indoor | Low |
| Note 1: The Urban Macro is agreed as baseline scenario for SBFD co-ex study with high priority in RAN4#104-e, while it does not preclude other scenarios. Note 2: The Urban Hotspot uses the same assumption as Urban Macro, except that Urban Macro uses random dropping method for UE while Urban Hotspot uses cluster-based dropping method for UE. Both random dropping and cluster-based dropping for calibration. Note 3: Consider Urban Macro scenario first for calibration purpose. Note 4: Companies are encouraged to provide simulation results for Urban Hotspot scenario as 2nd priority. [Editor’s Note: Agreement 2.2.1 of R4-2302888] Note 5: Companies also encouraged to simulate Uma-to-UMi co-existence scenario as 2nd priority. [Editor’s Note: Agreement 2.2.3 of R4-2302888] Note 6: Use UMi simulation assumption R4-2305922 as starting point. Consider Tx power refer to 3GPP UMi output power. Detailed simulation assumptions will be discussed after RAN4#107 meeting. |  |  |  |

In this contribution we present our results for the scenarios highlighted in Table 1 and Table 2 and derive our observations and conclusions on the SBFD deployment coexistence impact and challenges.

# Coexistence simulation results

Throughout this section our coexistence simulation results for the different scenarios and configurations are presented. We follow the simulation methodology and parameters agreed in [3], where the RF parameters (i.e., ACLR and ACS) are determined based on the degradation caused by the adjacent channel interference (ACI).

## Simulation assumptions

For the results presented in this section, the simulations assumptions were based on R4-2305923 and for the parameters with multiple options the following table was considered. Unless not mentioned in the table below, the parameters are based on R4-2305923. Our results unless otherwise stated are based on 100% grid shift between the victim and aggressor network. A visualization of the network is shown in Figure 1 where the blue and red lines represent the victim and aggressor network, respectively.

Table 3 Simulation parameters and assumptions considered.

| Simulation | Simulation parameter | Assumption |
| --- | --- | --- |
| Network Layout | Grid shift % | 100% |
|  | UE deployment | Random deployment with Baseline: 20% indoor and 80% outdoor and 100% outdoor for FR2 |
| Path-loss model | Macro-to-Macro: | UMa (h_UE = 25 m) see TR 38.803 |
|  | For LoS probability for Macro-to-Macro case | Reuse the same model as in TR 38.828 with h_UT equals to 25m; |
|  | UE-to-UE | UMi (h_BS=1.5 m ~ 22.5 m)+ penetration loss see TR 38.803. UMi model is not applicable when 2D distance is less than 10m, instead FSPL is applicable. |
| BS and UE configuration | BS Tx power | For legacy TDD: 49dBm/100MHz for dual polarizations. For SBFD antenna configuration 2: 49dBm/100MHz for dual polarizations. |
|  | UE Tx power | 23 dBm FR1 and 13.4 FR2 |
|  | SBFD/TDD PSD configuration | For SBFD power allocation consider constant PSD for transmitted power, which is the same as for legacy TDD |
|  | BS antenna configurations | both SBFD and NR TDD gNB re-use TR 38.828 model without considering sub-arrays. For SBFD antenna configuration 2: (Mg,Ng,M,N,P)= (1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
|  | SBFD configuration | DU configuration with {DU}: {80MHz, 20MHz} FR1 DU configuration with {DU}: {160MHz, 40MHz} FR2 |
|  | UE/BS ACLR model | Agreed ACLR model as in R4-2305923. |
|  | Relative ACIR | Derived from legacy ACLR and ACS of legacy BS and UE |

Figure 1 Visualization of 100% grid shift consideration

## FR1 results

### Urban Macro -> Urban Macro

#### NR TDD as a victim

##### Downlink as a victim

Figure 2 and Table 4 present the SINR and throughput with ACI for FR1 UMa deployment when the victim is operating in a TDD DL slot. As observed, the performance impact of SBFD operation in the adjacent channel is nearly negligible when compared to the legacy NR TDD DL. It is apparent that for Macro-Macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from co-channel and aggressor gNBs. In addition, no performance degradation relating to ACLR/ACS due to inter-UE CLI is observed in SBFD adjacent channel operation.

Figure 2 FR1 UMa downlink statistics for NR TDD DL as a victim

Table 4 FR1 UMa downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | -0.06 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | -0.69 | - | - |  |
|  |  | 50% SINR degradation (dB) | -0.084 | - | - |  |
|  |  | Mean Throughput degradation (%) | -0.18 | - | - |  |

Observation 1: For FR1 urban macro and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

##### Uplink as a victim

Figure 3 and Table 5 present the SINR and throughput with ACI for FR1 UMa deployment when the victim is operating in a TDD UL slot. There is an observed performance impact of SBFD operation in the adjacent channel due to the inter-gNB CLI compared to the legacy NR TDD DL. This degradation is also existent with unsynchronized deployments where the inter-gNB CLI is larger than the legacy inter-UE interference.

Figure 3 FR1 UMa uplink statistics for NR TDD UL as a victim

Table 5 FR1 UMa uplink ACIR results for NR TDD UL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 3.39 | 2.44 | 0.84 | Provided in Table 3 |
|  |  | Throughput degradation (%) | 48.78 | 37.25 | 14.35 |  |
|  |  | 50% SINR degradation (dB) | 1.23 | 0.93 | 0.38 |  |
|  |  | Mean Throughput degradation (%) | 9.34 | 7.14 | 3.09 |  |

Observation 2: For FR1 urban macro and TDD UL as a victim, SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network. However, this degradation is even higher with the case of fully unsynchronized deployments.

#### SBFD as a victim

Figure 4 and Figure 5 present the SINR and throughput for the case where SBFD is a victim for downlink and uplink, respectively, while Table 6 shows the SINR and throughput degradation values. The ACIR values have been benchmarked against the legacy TDD synchronized and TDD fully non-synchronized cases (termed CLI). For SBFD DL as a victim, no SINR degradation is observed compared to legacy TDD DL network, while SINR and throughput degradation is observed for the case SBFD UL slot, however, this degradation is lower when compared to the fully non-synchronized case.

Figure 4 FR1 UMa downlink for NR SBFD DL as a victim

Figure 5 FR1 UMa uplink for NR SBFD UL as a victim

Table 6 FR1 UMa ACIR results for NR SBFD as a victim.

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |  |
| Qualcomm Inc. | SBFD DL | 5% | SINR degradation (dB) | 0.04/0.35 | - | - | TDD sync DL / fully async (UL->DL) | Provided in Table 3 |
|  |  |  | Throughput degradation (%) | 0.47/3.98 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.018/0.41 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.56/1.73 | - | - |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) | 3.57/-0.02 | 2.12/-1.8 | 1.09/-2.87 | TDD sync DL / fully async (DL->UL) |  |
|  |  |  | Throughput degradation (%) | 47.63/-7.3 | 29.77/-54.58 | 15.64/-85.66 |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.96/-0.41 | 0.64/-0.79 | 0.24/-1.85 |  |  |
|  |  |  | Mean Throughput degradation (%) | 7.35/-3.18 | 4.87/-6.66 | 1.84/-10.1 |  |  |

Observation 3: For FR1 urban macro and SBFD as a victim, no SINR or throughput degradation is observed for the DL slot while some degradation is observed for the uplink slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

Proposal 1: It should be captured in the TR that even with degradation in the urban macro SBFD UL as a victim case,  latency reduction gains and UL coverage enhancements are expected with SBFD deployments.

### Urban Hotspot -> Urban Hotspot

This case as has been agreed in [1] considers the typical UMa scenario, that is discussed in Section 3.2.1 with clustered UE deployment. A visualization of the random and clustered UE deployment is shown in Figure 6. A single cluster with a given radius is randomly dropped within every cell with a minimum distance between cluster centre and gNB as well as minimum distance between two neighbouring clusters. Number of UEs are dropped inside and outside of each cluster with ratio of inside to outside UEs equal to the indoor UE ratio (i.e., 20%). All UEs that are deployed outside each cluster are outdoor UEs. A visualization comparing the random UE deployment to the clustered one is shown in Figure 6, where we consider 20% indoor UE ratio with cluster radius of 25m.

Figure 6 UMa deployment with (left) random UE deployment and (right) clustered UE deployment

#### NR TDD as a victim

##### Downlink as a victim

Figure 7 and Table 7 present the SINR and throughput with ACI for FR1 urban-hotspot deployment when the victim is operating in a TDD DL slot. Like the urban macro, the performance impact of SBFD operation in the adjacent channel is nearly negligible when compared to the legacy NR TDD DL. This also implies that the inter-UE CLI from clustering in the adjacent operator is not impactful compared to random UE deployment.

Figure 7 FR1 UMa-Hotspot downlink statistics for NR TDD DL as a victim

Table 7 FR1 UMa-Hotspot downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.08 | - | - | Provided in Table 3 (Clustered UE deployment is adopted with same Outdoor/indoor ratios) |
|  |  | Throughput degradation (%) | 0.92 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.04 | - | - |  |
|  |  | Mean Throughput degradation (%) | 0.15 | - | - |  |

Observation 4: For FR1 urban hotspot and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 5: Based on the single UE scheduling considered in RAN4, inter-UE CLI from clustering in the adjacent operator is not impactful compared to random UE deployment.

##### Uplink as a victim

Figure 8 and Table 8Table 5 present the SINR and throughput with ACI for FR1 urban hotspot deployment when the victim is operating in a TDD UL slot. There is an observed performance impact of SBFD operation in the adjacent channel due to the inter-gNB CLI compared to the legacy NR TDD DL.

Figure 8 FR1 UMa-Hotspot uplink for NR TDD UL as a victim

Table 8 FR1 UMa-Hotspot uplink ACIR results for NR TDD UL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 3.50 | 2.63 | 1.07 | Provided in Table 3 (Clustered UE deployment is adopted with same Outdoor/indoor ratios). |
|  |  | Throughput degradation (%) | 49.67 | 40.03 | 18.38 |  |
|  |  | 50% SINR degradation (dB) | 1.25 | 0.93 | 0.41 |  |
|  |  | Mean Throughput degradation (%) | 9.63 | 7.27 | 3.21 |  |

Observation 6: For FR1 urban hotspot and TDD UL as a victim, SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

#### SBFD as a victim

The same observations for urban macro are valid for the urban hotspot case as can be observed in Figure 9 and Figure 10. In addition, the impact of the clustering on the inter-UE CLI is depicted in Figure 11 where we compare the inter-UE CLI coming from random and clustering deployments as well as the legacy co-channel interference from co-channel interfering gNBs. As observed the performance in the downlink is dominated by the co-channel legacy interference from gNBs.

To investigate the impact of UE clustering on the network statistics, specifically, As observed, the inter-UE CLI for the random and clustered UE cases is quite close due to the fact that the same % of UEs are indoor ones in both scenarios and the gNB is serving a single UE every slot. On the other hand, the case with 0% indoor UEs provide lower CL due to the less experienced losses (e.g., LOS, indoor-to-outdoor losses, penetration losses, etc.). Based on this, it is arguable that UE clustering will have minimal impact on the inter-UE CLI.

Figure 9 FR1 UMa-Hotspot downlink for NR SBFD DL as a victim

Figure 10 FR1 UMa-Hotspot uplink for NR SBFD UL as a victim

Figure 11 Interference power for different UE deployments

Table 9 FR1 UMa uplink ACIR results for NR SBFD as a victim.

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |  |
| Qualcomm Inc. | SBFD DL | 5% | SINR degradation (dB) | 0.05/0.36 | - | - | TDD sync DL / fully async (UL->DL) | Provided in Table 3 (Clustered UE deployment is adopted with same Outdoor/indoor ratios). |
|  |  |  | Throughput degradation (%) | 1.01/4.48 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.05/0.39 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.62/1.76 | - | - |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) | 3.70/-0.25 | 2.80/-1.09 | 1.10/-2.84 | TDD sync DL / fully async (DL->UL) |  |
|  |  |  | Throughput degradation (%) | 48.80/-12.70 | 38.8/-34.66 | 16.55/-83.65 |  |  |
|  |  |  | 50% SINR degradation (dB) | 1.05/-0.37 | 0.74/-0.69 | 0.30/-1.12 |  |  |
|  |  |  | Mean Throughput degradation (%) | 7.99/-3.16 | 5.54/-5.92 | 2.32/-9.52 |  |  |

Observation 7: For FR1 urban hotspot and SBFD as a victim, no SINR or throughput degradation is observed for the DL slot while some degradation is observed for the uplink slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

Proposal 2: It should be captured in the TR that even with degradation in the urban hotspot SBFD UL as a victim case,  latency reduction gains and UL coverage enhancements are expected with SBFD deployments.

### Indoor -> Indoor

#### NR TDD as a victim

##### Downlink as a victim

Figure 12 and Table 10 present the SINR and throughput with ACI for FR1 indoor deployment when the victim is operating in a TDD DL slot. The performance impact of SBFD operation in the adjacent channel is nearly negligible when compared to the legacy NR TDD DL.

Figure 12 FR1 InH downlink statistics for NR TDD DL as a victim

Table 10 FR1 InH downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | -0.01 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.014 | - | - |  |
|  |  | Mean Throughput degradation (%) | -0.046 | - | - |  |

Observation 8: For FR1 indoor and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

##### Uplink as a victim

Figure 13 and Table 11 present the SINR and throughput with ACI for FR1 indoor deployment when the victim is operating in a TDD UL slot. The performance impact of SBFD operation in the adjacent channel is below the 5% degradation mark when compared to the legacy NR TDD DL.

Figure 13 FR1 InH uplink statistics for NR TDD DL as a victim

Table 11 FR1 InH uplink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.19 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | 2.46 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.40175 | - | - |  |
|  |  | Mean Throughput degradation (%) | 1.9588 | - | - |  |

Observation 9: For FR1 indoor and TDD UL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

#### SBFD as a victim

Figure 14 FR1 UMa InH for NR SBFD DL as a victim

Figure 15 FR1 UMa InH for NR SBFD UL as a victim

Table 12 FR1 InH uplink ACIR results for NR SBFD as a victim.

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |  |
| Qualcomm Inc. | SBFD DL | 5% | SINR degradation (dB) | 0.005/0.0237 | - | - | TDD sync DL / fully async (UL->DL) | Provided in Table 3 |
|  |  |  | Throughput degradation (%) | 0/0.011 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0/0 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.09/0.117 | - | - |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) | 0/0 | - | - | TDD sync DL / fully async (DL->UL) |  |
|  |  |  | Throughput degradation (%) | 0/0.01 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.004/0.02 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.09/0.11 | - | - |  |  |

Observation 10: For FR1 indoor and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

### UMa -> Umi

A visualization of the network layout of this scenario is shown in Figure 16 where the red network represents the UMa aggressor while the blue network is the UMi victim network. The rationale behind this scenario is to investigate the impact of SBFD deployment in the adjacent UMa network on the legacy NR TDD network that is an UMi.

Figure 16 UMa->UMi coexistence scenario

#### NR TDD as a victim

##### Downlink as a victim

Figure 17 FR1 UMa-UMi downlink statistics for NR TDD DL as a victim

Table 13 FR1 UMa-UMi downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.03 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | -0.5 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.011 | - | - |  |
|  |  | Mean Throughput degradation (%) | -0.0627 | - | - |  |

Observation 11: For FR1 UMa->UMi and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

##### Uplink as a victim

Figure 18 FR1 UMa-UMi uplink statistics for NR TDD UL as a victim

Table 14 FR1 UMa-UMi uplink ACIR results for NR TDD UL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 1.5 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | NaN | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.9 | - | - |  |
|  |  | Mean Throughput degradation (%) | 6.6 | - | - |  |

Observation 12: For FR1 UMa->UMi and TDD UL as a victim,  some SINR and throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network. However, such degradation is less compared to fully non-synchronized deployments.

### UMi -> UMi

#### NR TDD as a victim

##### Downlink as a victim

Figure 19 FR1 UMi downlink statistics for NR TDD DL as a victim

Table 15 FR1 UMi downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.04 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | -0.66 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.044 | - | - |  |
|  |  | Mean Throughput degradation (%) | -0.21 | - | - |  |

Observation 13: For FR1 UMi and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

##### Uplink as a victim

Figure 20 FR1 UMi uplink statistics for NR TDD UL as a victim

Table 16 FR1 UMi uplink ACIR results for NR TDD UL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.42 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | 0.42 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0 | - | - |  |
|  |  | Mean Throughput degradation (%) | 3.16 | - | - |  |

Observation 14: For FR1 UMi and TDD UL as a victim, marginal SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

#### SBFD as a victim

Figure 21 FR1 UMi for NR SBFD DL as a victim

Figure 22 FR1 UMi for NR SBFD UL as a victim

Table 17 FR1 UMi ACIR results for NR SBFD as a victim.

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |  |
| Qualcomm Inc. | SBFD DL | 5% | SINR degradation (dB) | 0.07/0.30 | - | - | TDD sync DL / fully async (UL->DL) | Provided in Table 3 |
|  |  |  | Throughput degradation (%) | 1.57/5.61 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.005/0.245 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.74/2.17 | - | - |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) | 0.07/0.087 | - | - | TDD sync DL / fully async (DL->UL) |  |
|  |  |  | Throughput degradation (%) | 0/0 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.035/-0.07 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.99/0.1875 | - | - |  |  |

Observation 15: For FR1 UMi and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

## FR2 results

### Urban Macro -> Urban Macro

#### NR TDD as a victim

##### Downlink as a victim

Figure 23 and Table 18 present the SINR and throughput with ACI for FR2 UMa deployment when the victim is operating in a TDD DL slot. As observed, the performance impact of SBFD operation in the adjacent channel is nearly negligible when compared to the legacy NR TDD DL. It is apparent that for Macro-Macro scenarios (impact on UE), the adjacent interference is dominated by legacy DL interference from co-channel and aggressor gNBs.

Figure 23 FR2 UMa downlink statistics for NR TDD DL as a victim

Table 18 FR2 UMa downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | -0.24 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | -2.03 | - | - |  |
|  |  | 50% SINR degradation (dB) | -0.090 | - | - |  |
|  |  | Mean Throughput degradation (%) | -0.256 | - | - |  |

Observation 16: For FR2 urban macro and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

##### Uplink as a victim

Figure 24 FR1 UMa uplink statistics for NR TDD UL as a victim

Table 19 FR1 UMa uplink ACIR results for NR TDD UL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + 2 (dB) | Relative ACIR + 4 (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.29 | 0.23 | 0.21 | Provided in Table 3 |
|  |  | Throughput degradation (%) | 5.10 | 4.02 | 3.69 |  |
|  |  | 50% SINR degradation (dB) | 0.18 | 0.09 | 0.05 |  |
|  |  | Mean Throughput degradation (%) | 1.02 | 0.62 | 0.45 |  |

Observation 17: For FR2 urban macro and TDD UL as a victim, marginal SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

#### SBFD as a victim

Figure 25 FR2 UMa downlink for NR SBFD DL as a victim

Figure 26 FR2 UMa uplink for NR SBFD UL as a victim

Table 20 FR2 UMa uplink ACIR results for NR SBFD as a victim.

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |  |
| Qualcomm Inc. | SBFD DL | 5% | SINR degradation (dB) | 0.1/0.5 | - | - | TDD sync DL / fully async (UL->DL) | Provided in Table 3 |
|  |  |  | Throughput degradation (%) | 0.8/3.8 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.1/0.4 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.7/1.5 | - | - |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) | 0.36/0.03 |  |  | TDD sync DL / fully async (DL->UL) |  |
|  |  |  | Throughput degradation (%) | 4.84/-1.17 |  |  |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.07/-0.26 |  |  |  |  |
|  |  |  | Mean Throughput degradation (%) | 0.6/-0.8 |  |  |  |  |

Observation 18: For FR2 UMa and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR.

### Indoor -> Indoor

#### NR TDD as a victim

##### Downlink as a victim

Figure 27 FR2 InH downlink statistics for NR TDD DL as a victim

Table 21 FR2 InH downlink ACIR results for NR TDD DL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0.02 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | 0.19 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0.25 | - | - |  |
|  |  | Mean Throughput degradation (%) | 0.29 | - | - |  |

Observation 19: For FR2 InH and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

##### Uplink as a victim

Figure 28 FR InH uplink statistics for NR TDD UL as a victim

Table 22 FR2 InH uplink ACIR results for NR TDD UL as a victim.

| Company | Observation point |  | Performance degradation |  |  | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Relative ACIR (dB) (Note 1) | Relative ACIR + step (dB) | Relative ACIR + step (dB) |  |
| Qualcomm Inc. | 5% | SINR degradation (dB) | 0 | - | - | Provided in Table 3 |
|  |  | Throughput degradation (%) | 0.01 | - | - |  |
|  |  | 50% SINR degradation (dB) | 0 | - | - |  |
|  |  | Mean Throughput degradation (%) | 0.06 | - | - |  |

Observation 20: For FR2 InH and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

#### SBFD as a victim

Figure 29 FR2 InH downlink for NR SBFD DL as a victim

Table 23 FR2 InH ACIR results for NR SBFD as a victim.

| Company | Victim | Observation point |  | Performance degradation (SINR in dB, Throughput in %) |  |  | Performance degradation reference | Choice of optional simulation parameters |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Relative ACIR (dB) | Relative ACIR + 2 (dB) | Relative ACIR + 8 (dB) |  |  |
| Qualcomm Inc. | SBFD DL | 5% | SINR degradation (dB) | 0.06/0.87 | - | - | TDD sync DL / fully async (UL->DL) | Provided in Table 3 |
|  |  |  | Throughput degradation (%) | 0/-0.1 | - | - |  |  |
|  |  |  | 50% SINR degradation (dB) | 0.06/0.87 | - | - |  |  |
|  |  |  | Mean Throughput degradation (%) | 2.52/4.43 | - | - |  |  |
|  | SBFD UL | 5% | SINR degradation (dB) | 0.005/0.01 |  |  | TDD sync DL / fully async (DL->UL) |  |
|  |  |  | Throughput degradation (%) | 0.06/0.1 |  |  |  |  |
|  |  |  | 50% SINR degradation (dB) | 0/0 |  |  |  |  |
|  |  |  | Mean Throughput degradation (%) | 0/0.07 |  |  |  |  |

Observation 21: For FR2 InH and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

# Conclusion

Throughout this contribution, we provided our views on remaining simulation scenarios and aspects for the SBFD adjacent coexistence work within RAN4 as well as simulation results investigating the impact of SBFD deployment. In summary we have made the following observations:

Proposal 1: It should be captured in the TR that even with degradation in the SBFD UL as a victim case,  latency reduction gains and UL coverage enhancements are expected with SBFD deployments.

Proposal 2: It should be captured in the TR that even with degradation in the urban hotspot SBFD UL as a victim case,  latency reduction gains and UL coverage enhancements are expected with SBFD deployments.

Observation 1: For FR1 urban macro and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 2: For FR1 urban macro and TDD UL as a victim, SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network. However, this degradation is even higher with the case of fully unsynchronized deployments.

Observation 3: For FR1 urban macro and SBFD as a victim, no SINR or throughput degradation is observed for the DL slot while some degradation is observed for the uplink slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

Observation 4: For FR1 urban hotspot and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 5: Based on the single UE scheduling considered in RAN4, inter-UE CLI from clustering in the adjacent operator is not impactful compared to random UE deployment.

Observation 6: For FR1 urban hotspot and TDD UL as a victim, SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 7: For FR1 urban hotspot and SBFD as a victim, no SINR or throughput degradation is observed for the DL slot while some degradation is observed for the uplink slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

Observation 8: For FR1 indoor and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 9: For FR1 indoor and TDD UL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 10: For FR1 indoor and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

Observation 11: For FR1 UMa->UMi and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 12: For FR1 UMa->UMi and TDD UL as a victim,  some SINR and throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network. However, such degradation is less compared to fully non-synchronized deployments.

Observation 13: For FR1 UMi and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 14: For FR1 UMi and TDD UL as a victim, marginal SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 15: For FR1 UMi and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

Observation 16: For FR2 urban macro and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 17: For FR2 urban macro and TDD UL as a victim, marginal SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 18: For FR2 UMa and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR.

Observation 19: For FR2 InH and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 20: For FR2 InH and TDD DL as a victim, no SINR or throughput degradation is observed when adjacent network is SBFD-capable compared to when the adjacent network is a TDD DL network.

Observation 21: For FR2 InH and SBFD as a victim, no SINR or throughput degradation is observed for either the DL slot or the UL slot when the victim network is SBFD-capable with NR TDD DL in the adjacent channel.

# References

- R4-2309793, “WF for SBFD co-existence study”, CMCC, RAN4#107.

- R4-2305921, “WF for SBFD adjacent channel co-existence study”, CMCC, RAN4#106-bis-e.

- R4-2305923, “Summary of co-existence simulation parameters”, CMCC, CableLabs, Samsung, RAN4#106-bis-e.

- R4-2305922, “WF for SBFD adjacent channel co-existence study for UMa-to-UMi scenario”, CMCC, CableLabs, RAN4#106-bis-e.