3GPP TSG-RAN WG4 Meeting #107			    R4-2313218

Toulouse, France, August 21 – August 25, 2023

Agenda item:	8.33.2

Source: 	Qualcomm Incorporated

Title: 	Preliminary mobile IAB and NR coexistence study

Document for:	Approval

# Introduction

In RAN#94e, the work item on MIAB (Integrated Access and Backhaul) for NR was approved in RP-222671. RAN4 is expected to study impacts on RF and RRM requirements as follows:

- Conduct co-existence study to assess the impact of moving cells. Based on the study outcome, specify RF and RRM requirements and mechanisms for the mIAB-node to enable co-existence, if needed. - Specify RRM requirements for the mIAB-node to enable IAB-node mobility, if needed.

In RAN4#107, discussion and fine tuning of the coexistence framework of Rel-18 mIAB was carried out and a WF was agreed in [1]. In this paper we provide our preliminary results on the mobile IAB and NR coexistence analysis.

# Discussion

## Deployment scenarios

In Rel-16 IAB, the coexistence study conducted in RAN4 [3] was based on two network layouts, namely homogenous and heterogeneous. Error! Reference source not found. shows a pictorial representation of the homogeneous layout where the blue layout represents the legacy NR network, and the red layout represents the IAB network. Both layouts are characterized by a hexagonal deployment with antenna heights of 25m. Each IAB node is tri-sectorial and is connected to the donor node through a specific route (topology).

Figure 1 Pictorial representation of the homogeneous layout (layout 2).

The other network layout in Rel-16 IAB study is shown in Figure 2 where micro IAB child nodes are randomly dropped inside a circle at 40m distance in each cell. The green asterisks represent the IAB nodes while the black and red lines represent the NR BSs and IAB donor nodes, respectively.

Figure 2 Pictorial representation of the heterogeneous layout (layout 1)

For Rel-18 mIAB, as the IAB-node is assumed to be mounted in vehicles and serving UEs either inside (on-board) or outside the vehicle, mobility is introduced in the IAB network and the location of the IAB nodes cannot be fixed anymore. In Error! Reference source not found. a pictorial representation of layout 1 for mIAB is presented where the IAB nodes (1 IAB node per macro BS) are randomly dropped within each cell. Layout 1 presents a more realistic scenario for mIAB deployment where the IAB donors have a fixed location (e.g., light posts or top of small buildings) while the IAB nodes can exist randomly at a given snapshot in the network, whereas layout 2 presents a fixed location deployment which does not fit with Rel-18 mIAB use cases. As a result, we propose to focus the work in RAN4 for mIAB on layout 1 as a starting point and consider layout 2 with lower priority.

Observation 1: Heterogeneous layout (layout 1) with randomly deployed IAB nodes within each cell is the realistic deployment scenario to be investigated by RAN4 in the Rel-18 mIAB coexistence study.

Figure 3 Pictorial representation of the heterogeneous layout (layout 1) for mIAB.

To further showcase the differences between layout 1 for Rel-16 IAB and Rel-18 mIAB, we show in Figure 4 the distribution of the 3D-distance between the IAB node and IAB donor for an FR2 UMa deployment with ISD = 200m. As expected, the distance distribution for the case where the IAB node is dropped randomly within each cell spans larger values compared to the static Rel-16 IAB deployment scenarios.

Figure 4 3D-distance distribution for Rel-16 and Rel-18 IAB nodes to their serving IAB donors

## Co-existence parameters

The list of scenarios to be considered for the coexistence study has been agreed in [2]. The objective of the coexistence study within RAN4 is to observe what would be the impact of mobile IAB deployments on the legacy network (i.e., cases where NR is a victim and IAB is an aggressor) as well as the impact of legacy NR deployment on IAB operation (i.e., cases where IAB is a victim and NR is an aggressor). The list of considered simulation parameters is given in Table 1.

Table 1 Simulation parameters for mIAB coexistence

| Parameter | Value |
| --- | --- |
| Duplex mode | TDD |
| Frequency range | FR1: 4.9GHz – FR2: 30GHz |
| Inter-site disctance | FR1: 500m – FR2: 200m |
| Beamforming | Enabled for FR1/ FR2 |
| Simulation bandwidth | 100MHz for FR1 – 200MHz for FR2 |
| Number of Ues in the network | 1 active UE/sector |
| gNB Tx power | FR1: 46 dBm – FR2: 33 dBm |
| Mobile IAB node Tx power | FR1 & FR2: 33dBm/ 38dBm as baseline value |
| Mobile IAB node antenna configurations | FR1: (Mg,Ng,M,N,P)=(1,1,8,8,2), dH,dV)=(0.5,0.0.5)λ, element gain: 5 dBi FR2: (Mg,Ng,M,N,P)=(1,1,8,16,2), dH,dV)=(0.5,0.0.5)λ, element gain: 3 dBi |
| gNB antenna height | 25m for macro cells |
| Mobile IAB node antenna height | 4m |
| Macro BS to mobile IAB minimum distance | 5m |
| UL SNR target | 22dB |
| gNB receiver noise figure | FR1: 5dB – FR2: 10dB |
| Mobile IAB node receiver noise figure | FR1: 5dB – FR2: 10dB |

# Simulation results

In this section we provide preliminary analysis on the adjacent channel specification for the IAB MT by considering the NR network as the victim network in case of UL operation and the IAB network as the victim network in case of DL operation.

### NR as a victim and IAB as aggressor

The list of scenarios to be considered as well as the impacted RF power when NR network is a victim and IAB is an aggressor is presented in Table 2. Note that cases #1 and #4 can be skipped as they consider legacy scenarios.

Table 2 Coexistence scenarios for NR as a victim and IAB as aggressor

| Case # | Baseline | Aggressor | Victim | Impacted RF parameter |
| --- | --- | --- | --- | --- |
| 1 | NR DL active: UE receiving. No IAB operation | IAB: access link inactive, backhaul link DL active: IAB donor transmitting | NR DL: UE receiving | IAB donor ACLR is the same with normal BS, this case could be skipped. |
| 2 | NR DL active: UE receiving. No IAB operation | IAB: access link active, backhaul link inactive : IAB-DU transmitting | NR DL: UE receiving | IAB-DU Tx power IAB-DU ACLR |
| 3 | NR UL active: BS receiving. No IAB operation | IAB: access link inactive, backhaul link UL active: IAB-MT transmitting | NR UL: BS receiving | IAB-MT Tx power IAB-MT ACLR |
| 4 | NR UL active: BS receiving. No IAB operation | IAB: access link active, backhaul link inactive: IAB donor receiving | NR UL: BS receiving | This is legacy scenario, could be skipped. |

#### Case 2: NR DL (UE receiving) & IAB-DU transmitting

In Figure 5 the SINR and throughput distribution for the case with and without adjacent channel interference from IAB network is shown. Considering that the IAB-DU has the same ACLR as the gNB ACLR (i.e., 45 dB), degradation due to IAB operation in the adjacent channel is not excessive with mean throughput loss = 1.8% and 5% TP loss = 7%. With an enhanced ACIR=35 dB (compared to 32.7 legacy ACIR), the 5% TP loss becomes 5%. Nevertheless, we propose to use legacy gNB ACLR requirement of 45 dB for the IAB-DU ACLR. This can be motivated by the fact that in the agreed RAN4 coexistence, no assumptions are considered on additional losses due to bus shielding or lower IAB-DU Tx powers.

Figure 5 SINR and Throughput statistics for FR1 when NR DL is victim and IAB-DU is aggressor

Similar to FR1, Figure 5 presents the SINR and throughput distribution for FR2 considering that the IAB-DU has an ACLR of 28dB (i.e., ACIR~22dB @ UE ACS=23dB). As observed, the TP loss due to IAB operation in the adjacent channel is around 8% for the cell edge user. As a solution for this, an ACIR of 25dB will lead to TP loss for cell edge user=5%. However, as FR1, additional losses can be incorporated into the IAB link, so the interference from the IAB network towards the legacy NE Ues in reality will be lower.

Figure 6 SINR and Throughput statistics for FR2 when NR DL is victim and IAB-DU is aggressor

The experienced mean and cell edge TP losses are summarized in Table 3, where it can be observed that the legacy UE ACLR requirement for IAB-MT can be used (with some acceptable degradation above the 5% threshold).

Table 3 NR DL throughput loss due to IAB operation

| NR Network UL throughput loss (%) |  |  |
| --- | --- | --- |
| Metric | FR1 | FR2 |
| 5%-TP loss | 7% | 8% |
| Mean TP loss | 1.7% | 1.4% |

Observation 2: Simulation results show that in the heterogeneous scenario for FR1 the impact of IAB-DU transmission in the adjacent channel to legacy NR network performance can be tolerated when considering 45dB ACLR for the IAB-DU.

Proposal 1: RAN4 to agree on 45dB and 28dB ACLR for IAB-DU for FR1 and FR2, respectively.

#### Case 3: NR UL (gNB receiving) & IAB-MT transmitting

In this section we present simulation results showing the impact of IAB MT UL interference to NR UL performance. In , we show the SINR and throughput distribution for FR1 and FR2, respectively, where the IAB-MT ACLR is set to 30dB and 17dB, respectively. It can be observed that the IAB-MT transmission in the adjacent channel has negligible impact on the NR UL for both FR1 and FR2.

Figure 7 SINR and Throughput statistics for FR1 when NR UL is victim and IAB-MT is aggressor

Figure 8 SINR and Throughput statistics for FR2 when NR UL is victim and IAB-MT is aggressor

Observation 3: Simulation results show that in the heterogeneous scenario for FR1 and FR2 the impact of IAB-MT transmission in the adjacent channel to legacy NR network performance is negligible when considering legacy UE ACLR requirement for IAB-MT.

The experienced mean and cell edge TP losses are summarized in Table 4, where it can be observed that the legacy UE ACLR requirement for IAB-MT are sufficient to meet the 5% TP loss degradation requirement.

Table 4 NR UL throughput loss due to IAB operation

| NR Network UL throughput loss (%) |  |  |
| --- | --- | --- |
| Metric | FR1 | FR2 |
| 5%-TP loss | 0.4% | 0.35% |
| Mean TP loss | 0.145% | 0.124% |

Proposal 2: RAN4 to agree on 30dB and 17dB ACLR for IAB-MT for FR1 and FR2, respectively.

### IAB as a victim and NR as aggressor

The list of scenarios to be considered as well as the impacted RF power when IAB network is a victim and NR network is an aggressor is presented in Table 5.

Table 5 Coexistence scenarios for IAB as a victim and NR as aggressor

| Case # | Baseline | Aggressor | Victim | Impacted RF parameter |
| --- | --- | --- | --- | --- |
| 5 | IAB: access link inactive, backhaul link DL active: IAB-MT receiving No NR operation | NR DL active: BS transmitting | IAB backhaul link: IAB-MT receiving | IAB-MT ACS |
| 6 | IAB: access link inactive, backhaul link UL active: IAB-Donor receiving No NR operation | NR UL active: UE transmitting | IAB backhaul link: IAB-Donor receiving | IAB donor ACS is the same with normal BS, this case could be skipped. |
| 7 | IAB: access link UL active, backhaul link inactive: IAB-DU receiving No NR operation | NR UL active: UE transmitting | IAB access link : IAB-DU receiving | IAB-DU ACS |
| 8 | IAB: access link DL active, backhaul link DL inactive: IAB-DU transmitting No NR operation | NR DL active: BS transmitting | IAB access link: UE receiving | UE and BS are legacy, this scenario can be skipped. |

#### Case 5: IAB-MT receiving & NR gNB transmitting

In this section we present simulation results showing the impact of  NR DL interference to IAB-MT reception performance. In Figure 9 and Figure 10, we show the SINR and throughput distribution for FR1 and FR2, respectively, where the IAB-MT ACS is set to 33dB and 23dB, respectively. It can be observed that the NR gNB transmission in the adjacent channel has negligible impact on the IAB-MT reception for both FR1 and FR2.

Figure 9 SINR and Throughput statistics for FR1 when IAB-MT is victim and NR gNB is aggressor

Figure 10 SINR and Throughput statistics for FR2 when IAB-MT is victim and NR gNB is aggressor

Observation 4: Simulation results show that in the heterogeneous scenario for FR1 and FR2 the impact of NR gNB transmission in the adjacent channel to IAB-MT performance is negligible when considering legacy UE ACS requirement for IAB-MT.

The experienced mean and cell edge TP losses are summarized in Table 6, where it can be observed that the legacy UE ACLR requirement for IAB-MT are sufficient to meet the 5% TP loss degradation requirement.

Table 6 IAB DL throughput loss due to NR operation

| IAB-DL throughput loss (%) |  |  |
| --- | --- | --- |
| Metric | FR1 | FR2 |
| 5%-TP loss | 0.16% | 0.04% |
| Mean TP loss | 0.8% | 0.5% |

Proposal 3: RAN4 to agree on 33dB and 23dB ACS for IAB-MT for FR1 and FR2, respectively.

#### Case 6: IAB-DU receiving & NR UE transmitting

In this section we present simulation results showing the impact of  NR UL interference to IAB-DU reception performance. In Figure 12 and Figure 13, we show the SINR and throughput distribution for FR1 and FR2, respectively, where the IAB-DU ACS is set to 46dB and 24dB, respectively. It can be observed that for FR1, the NR UE transmissions in the adjacent channel has an impact on the cell edge IAB-MT nodes (~14% TP loss). Although this is above the typical 5% threshold considered in RAN4, the pessimistic assumptions of the FR1 IAB parameters (e.g., antenna configuration and TRP) are a contributing factor to this degradation. This presents the corner case scenario where a cell edge NR UE might be physically very close to the transmitting IAB-MT (e.g., inside the bus). As a solution, higher ACIR values can be considered as shown in Figure 11, where we show the TP loss for legacy ACIR (i.e., 30 dB) as well as legacy ACIR + 5dB and +10dB. However, for FR2, that cell edge TP loss is well below the 5% threshold mark.

Figure 11 FR1 Throughput loss for different ACIR values when IAB-DU is receiving and NR UL is the aggressor

Observation 5: Simulation results show that in the heterogeneous scenario for FR1 the impact of NR UE transmission in the adjacent channel to IAB-MT performance is not negligible with the current simulation assumptions. More discussions on this case is required within RAN4.

Figure 12 SINR and Throughput statistics for FR1 when IAB-DU is victim and NR gNB is aggressor

Figure 13 SINR and Throughput statistics for FR2 when IAB-DU is victim and NR gNB is aggressor

Observation 6: Simulation results show that in the heterogeneous scenario for FR2 the impact of NR UE transmission in the adjacent channel to IAB-MT performance is negligible when considering legacy UE ACS requirement for IAB-MT.

The experienced mean and cell edge TP losses are summarized in Table 6, where it can be observed that the legacy UE ACLR requirement for IAB-MT are sufficient to meet the 5% TP loss degradation requirement.

Table 7 IAB UL throughput loss due to NR operation

| IAB-UL throughput loss (%) |  |  |
| --- | --- | --- |
| Metric | FR1 | FR2 |
| 5%-TP loss | 14% | 3.7% |
| Mean TP loss | 1.4% | 0.7% |

Proposal 4: RAN4 to agree on 46dB and 24dB ACS for IAB-DU for FR1 and FR2, respectively.

# Conclusion

In this paper, we have shared our initial views on mIAB RAN4 co-existence aspects. Our observations and proposals can be summarized as follows:

Observation 1: Heterogeneous layout (layout 1) with randomly deployed IAB nodes within each cell is the realistic deployment scenario to be investigated by RAN4 in the Rel-18 mIAB coexistence study.

Observation 2: Simulation results show that in the heterogeneous scenario for FR1 the impact of IAB-DU transmission in the adjacent channel to legacy NR network performance can be tolerated when considering 45dB ACLR for the IAB-DU.

Proposal 1: RAN4 to agree on 45dB and 28dB ACLR for IAB-DU for FR1 and FR2, respectively.

Observation 3: Simulation results show that in the heterogeneous scenario for FR1 and FR2 the impact of IAB-MT transmission in the adjacent channel to legacy NR network performance is negligible when considering legacy UE ACLR requirement for IAB-MT.

Proposal 2: RAN4 to agree on 30dB and 17dB ACLR for IAB-MT for FR1 and FR2, respectively.

Observation 4: Simulation results show that in the heterogeneous scenario for FR1 and FR2 the impact of NR gNB transmission in the adjacent channel to IAB-MT performance is negligible when considering legacy UE ACS requirement for IAB-MT.

Proposal 3: RAN4 to agree on 33dB and 23dB ACS for IAB-MT for FR1 and FR2, respectively.

Observation 5: Simulation results show that in the heterogeneous scenario for FR1 the impact of NR UE transmission in the adjacent channel to IAB-MT performance is not negligible with the current simulation assumptions. More discussions on this case is required within RAN4.

Observation 6: Simulation results show that in the heterogeneous scenario for FR2 the impact of NR UE transmission in the adjacent channel to IAB-MT performance is negligible when considering legacy UE ACS requirement for IAB-MT.

Proposal 4: RAN4 to agree on 46dB and 24dB ACS for IAB-DU for FR1 and FR2, respectively.

# References

- R4-2309863, “WF on mIAB RF requirements and coexistence”, Qualcomm, RAN4#107.

- R4-2305935, “WF on mIAB RF requirements and coexistence”, Qualcomm, RAN4#106-bis-e.

- , “NR; Background for integrated access and backhaul radio transmission and reception”, 3GPP.