3GPP TSG-RAN4 Meeting #108	R4-2313238

# Toulouse, France, 21-25 August, 2023

Source:				Ericsson

Title:					NTN enhancements – System parameters

Agenda item:		8.26.1.1

Document for:		Approval

# Introduction

This document discusses the system parameters and bands definition in the scope of the NTN enhancements WI ([1]), based on the agreed Way Forward [2] from last RAN4#107 meeting. The only remaining issue is related to the sync raster entries.

# Discussion

## Synchronization raster

In last RAN4#107 meeting, the way forward [2] has captured the following 2 options to be further studied:

We proposed option 1 and further explain in the following the rationale for it, based on the excel file ([3]) which was used to determine NR sync raster for all NR bands. We will only explain in details for the lowest bound of GSCN and for a SSB using 120 kHz SCS,  a similar rationale could be used for the other boudaries.

- We first calculate the minimum frequency where SSB reference frequency (fmin) could stand:

With fmin = 17.3 GHz + 1.9 MHz (minimum guard band) + 120 kHz * (20*12+1) / 2.

- This will give a minimum N value of:

With Nmin = Ceiling ( (17.326360000 GHz – 3 GHz) / 1.44 MHz) ; 1)

- And from Nmin, we could termine the GSCN mimimum boundary for band n512:

With GSCNmin = Ceiling ( (9942 + 7499) ; 12)

with 12 being the GSCN step size.

Based on the above, 17448 should then be the lowest GSCN value for band n512.

And, following the same rationale to determine GSCN ranges for bands n512, n511 and n510, we make the following proposal:

Proposal1: Specify n512, n511 and n510 synchronization raster entries according to following table:

| NTN operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |

# Conclusion

In this contribution, we discussed the system parameters to be used for the NTN satellite bands agreed in last RAN4 meeting. We made the following proposals:

Proposal1: Specify n512, n511 and n510 synchronization raster entries according to following table:

| NTN operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2309766, WF for system parameters on Ka band, Thales

- TR 38.817-01, General aspects for User Equipment (UE) Radio Frequency (RF) for NR, v15.5.0