3GPP TSG-RAN4 Meeting #108	R4-2313242

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | 0007 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.863: NTN Ka-band – Regulatory aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR proposes updates of TR 38.863 mentioning the regulatory information of NTN Ka-band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The regulatory information related to NTN Ka-band have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Regulatory information of NTN Ka-band won’t be captured in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

# 2	Reference

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-213691: "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)".

[3]	ITU-R Radio Regulations, 2020 Edition

[4]	ECC Decision 06(09): "Designation of the bands 1980-2010 MHz and 2170-2200 MHz for use by systems in the Mobile-Satellite Service including those supplemented by a Complementary Ground Component (CGC)", Approved 01 December 2006, Amended 05 September 2007,

[5]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[6]	3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks (NTN)".

[7]	3GPP RP-152284: "Revised Work Item: Narrowband IoT ".

[8]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios".

[9]	3GPP TR 45.820: "Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)".

[10]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[11]	Report ITU-R M.2292: "Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses".

[12]	3GPP RP-200559: "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz) ".

[13]	ITU-R Annex 4.4 to Document 5D/716-E.

[14]	3GPP TR 38.921: "Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz".

[15]	3GPP TR 36.802: "Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception".

[16]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[17]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[18]	3GPP R4-2111460: "On the Rx Parameters and Rx Testing Setup for NTN gNB".

[19]	3GPP R4-2108099: "WF on [312] NTN_Solutions_Part1".

[20]	FCC Online Table of Frequency Allocations, 47 C.F.R § 2.106, February 1, 2021.

[21]	FCC Order 03-15, Report and Order and Notice of Proposed Rule Making, January 29, 2003.

[22]	FCC Order 20-48, Order and Authorization, April 19, 2020.

[23]	FCC Code of Federal Regulations.

[24]	SoftBank, Loon LLC, Nokia, Ericsson, “Proposed deployment and system characteristics of HIBS in the working document towards a preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS],” ITU WP-5D contribution, Sep. 28, 2020.

[25]	3GPP TS 38.181: "NR; Satellite Node conformance testing".

[26]	3GPP TS 38.108: "NR; Satellite Node radio transmission and reception".

[27]	3GPP R4-213618: "Moderator’s summary of discussion [94e-41-R17-NRNTN-RAN4Spec] ".

[28]	ERC Recommendation 74-01: "Unwanted emissions in the spurious domain".

[29]	Recommendation ITU-R SM.1541-6: "Unwanted emissions in the out-of-band domain".

[30]	Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain".

[31]	3GPP TR 38.817-02: "General aspects for Base Station (BS) Radio Frequency (RF) for NR".

[32]	3GPP TR 38.817-01: "	General aspects for User Equipment (UE) Radio Frequency (RF) for NR".

[33]	FCC 16-89, REPORT AND ORDER AND FURTHER NOTICE OF PROPOSED RULEMAKING

[34]	FCC 20-159, REPORT AND ORDER

[35]	FCC 20-166, SECOND REPORT AND ORDER IN IB DOCKET NO. 17-95 AND REPORT AND ORDER IN IB DOCKET NO. 18-315 AND FURTHER NOTICE OF PROPOSED RULEMAKING

[36]	ECC Decision(05)01, The use of the band 27.5-29.5 GHz by the Fixed Service and uncoordinated Earth stations of the Fixed-Satellite Service (Earth-to-space)

[37]	ECC Decision(13)01,  The harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz

<End of the change>

<Start of the change>

### 5.2.3	NTN Satellite band: UL 27.5-30.0 GHz / DL 17.3-20.2 GHz

#### 5.2.3.1	ITU-R

The ITU-R Radio Regulations [3] specify the following service allocation for those frequency ranges, shown in Table 5.2.3.1-1:

Table 5.2.3.1-1: Allocation of 17.3-20.2 GHz and 27.5-30.0 GHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) 5.516A 5.516B Radiolocation 5.514 | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 BROADCASTING-SATELLITE Radiolocation 5.514 5.515 | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation 5.514 |
| 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A (Earth-to-space) 5.516 MOBILE | 17.7-17.8 FIXED FIXED-SATELLITE (space-to-Earth) 5.517 5.517A (Earth-to-space) 5.516 BROADCASTING-SATELLITE Mobile 5.515 | 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A (Earth-to-space) 5.516 MOBILE |
|  | 17.8-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A (Earth-to-space) 5.516 MOBILE 5.519 |  |
| 18.1-18.4 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A (Earth-to-space) 5.520 MOBILE 5.519 5.521 |  |  |
| 18.4-18.6 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A MOBILE |  |  |
| 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.517A 5.522B MOBILE except aeronautical mobile Space research (passive) 5.522A 5.522C | 18.6-18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.517A 5.522B MOBILE except aeronautical mobile SPACE RESEARCH (passive) 5.522A | 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.517A 5.522B MOBILE except aeronautical mobile Space research (passive) 5.522A |
| 18.8-19.3 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.517A 5.523A MOBILE |  |  |
| 19.3-19.7 FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) 5.517A 5.523B 5.523C 5.523D 5.523E MOBILE |  |  |
| 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A Mobile-satellite (space-to-Earth) | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A MOBILE-SATELLITE (space-to-Earth) | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A Mobile-satellite (space-to-Earth) |
| 5.524 | 5.524 5.525 5.526 5.527 5.528 5.529 | 5.524 |
| 20.1-20.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A MOBILE-SATELLITE (space-to-Earth) 5.524 5.525 5.526 5.527 5.528 |  |  |
| 27.5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.539 MOBILE 5.538 5.540 |  |  |
| 28.5-29.1 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.523A 5.539 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.1-29.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.516B 5.517A 5.523C 5.523E 5.535A 5.539 5.541A MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) 5.540 5.542 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.525 5.526 5.527 5.529 5.540 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) 5.540 5.542 |
| 29.9-30.0 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 - 5.526 5.527 5.538 5.540 5.542 |  |  |

In addition:

- WRC-15 authorized ESIM operations in 19.7-20.2 GHz (space-to-Earth) and 29.5-30.0 GHz (Earth-to-space) under certain conditions, and for GSO systems only.

- WRC-19 authorized ESIM operations in 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) under certain conditions, and for GSO systems only.

#### 5.2.3.2	FCC

In 2016, FCC issued Report and Order FCC 16-89 [33] allocating the 27.50-28.35 GHz frequency range for terrestrial licensed mobile operations (IMT 5G). This frequency range corresponds to the 3GPP band n261.

In 2020, FCC issued Report and Order FCC 20-159 [34] which streamlines part 25 rules that govern the licensing of satellite space and earth stations in a satellite system through a unified license. The unified licensing framework specified frequency bands which includes:

- Space-to-Earth:

  - For GSO: 18.3-18.8 GHz and 19.7-20.2 GHz

  - For NGSO: 17.8-18.6 GHz, 18.8-19.4 GHz and 19.6-20.2 GHz

- Earth-to-space:

  - For GSO: 28.35-29.1 GHz and 29.25-30.0 GHz.

  - For NGSO: 28.35-29.1 GHz and 29.5-30.0 GHz.

In 2020 as well, FCC issued the Report and Order FCC-20-66 [35] which facilitates the deployment of, and reduce the regulatory burdens on, Earth Stations in Motion (ESIMs). It allows ESIMs to communicate in additional frequency bands with GSO satellites operating in FSS bands and adopts rules for ESIMs to communicate with NGSO satellites in specific FSS bands. Those FSS bands include:

- Space-to-Earth:

  - For GSO: 17.8-19.4 GHz, 19.6-20.2 GHz.

  - For NGSO: 17.8-18.6 GHz, 18.8-19.4 GHz, 19.6-20.2 GHz

- Earth-to-space:

  - For GSO: 28.35-29.1 GHz and 29.25-30.0 GHz.

  - For NGSO: 28.4-29.1 GHz and 29.5-30.0 GHz.

#### 5.2.3.3	CEPT

The ECC Decision (05)01 [36] addresses the use of band 27.5-29.5GHz, regulated for Fixed Service and Fixed Satellite Service:

- The bands 27.5-27.8285 GHz, 28.4445-28.8365 GHz and 29.4525-29.5 GHz are designated for the use of uncoordinated FSS earth stations.

- The band 28.8365-28.9485 GHz is designated for the use of uncoordinated FSS earth stations, considering, until 1 January 2020, the legacy FS systems licensed in this band in some countries before 18 March 2005.

- The bands 27.8285-28.4445 GHz and 28.9485-29.4525 GHz are designated for the use of FS systems.

Also, this ECC Decision(05)01 states that the CEPT administrations shall not authorize the deployment of uncoordinated FSS earth stations in the bands designed for the use of FS systems.

Figure 5.2.3.3-1: ECC Decision(05)01 - 27.5-29.5 band segmentation

The ECC Decision(13)01 [37] specifies the harmonized use, free circulation, and exemption from individual licensing of Earth stations on mobile platforms (ESOMPs) within the ranges 17.3-20.2 GHz and 27.5-30.0 GHz,  in the portion identified for their use within the territory of operation, and under the conditions specified in Annexes 1,2 and 3 of this ECC Decision. ESOMPs are terminals with small directional antennas for the provision of broadband communication services. The terminals may be mounted on aircraft, ships or land vehicles or may be transportable devices used in motion or at temporary halts. This ECC Decision also specifies the conditions for those ESOMP devices to operate.

<End of the change>