3GPP TSG-RAN4 Meeting #108	R4-2313253

# Toulouse, France, 21-25 August 2023

Title:	LS on the handling of additional regulatory requirements for UAV UEs

Response to:

Release:	Rel-18

Work Item:	-

Source:	RAN WG4

To:	RAN WG2

Cc:	-

Contact person:	Dominique Everaere

dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

# 1	Overall description

In the scope of the UAV UEs WI, RAN4 discussed how to handle the additional regulatory out-of-band emissions (OOBE) requirements specified by CEPT in ECC Decision(22)07.

To handle such new requirements specific to a certain Region/country, RAN4 usually introduce new NS and specify required A-MPR values to respect those requirements. While this could be done with NR after the NR band has been specified, this can’t be done with LTE after the band has been released (the legacy LTE UEs will not recognize the new NS and then not attach to the cell anymore).

Considering UAV UE would operate not only on NR bands but also on LTE bands, this usual NS approach could not be considered.

RAN4 investigated several options and came to the conclusion that the most pragmatic option would be to specify a new Information Element. This new IE should contain the list of NS values applicable for UAV UEs only for the considered bands. When attaching to the cell, the UAV shall then support both the list of legacy NS values (from “NR-NS-PmaxList” in SIB4), and the list of “UAV specific” NS values (from the new IE element).

RAN4 kindly asks RAN2 to consider the above RAN4 request and would welcome any feedback from RAN2.

# 2	Actions

To RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to consider above RAN4 request to specify a new Information Element containing the list of “UAV specific” NS values.

# 3	Dates of next TSG RAN WG4 meetings

TSG-RAN4 Meeting #108-bis	October 9-13, 2023	Xiamen, China

TSG-RAN4 Meeting #109	November 13-17, 2023	Chicago, USA