3GPP TSG-RAN4 Meeting #108	R4-2313257

# Toulouse, France, 21-25 August, 2023

Source:			Ericsson

Title:			Introduction of 900 MHz NR Band in the US – System parameters

Agenda item:		7.33.2

Document for:		Discussion

# Introduction

The WI ([1]) to introduce a 900 MHz NR Band in the US has been put on hold, waiting for the progress on the other WI on NR support for dedicated spectrum less than 5MHz for FR1 ([2]).

After latest RAN4/RAN1 agreements on the 3 MHz channel BW and related topics, the work on the new 900MHz NR band could be resumed.

In this contribution, we are proposing the system parameters for this new band.

# Discussion

## General

### Band definition

The corresponding LTE band 106 was specified in last RAN4#107 and should be used as baseline for specifying this NR band.

The US 900MHz NR band should then be added to list of NR operating bands in FR1, table 5.2-1 as below:

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n106 | 896 MHz – 901 MHz | 935 MHz – 940 MHz | FDD |

### Channel bandwidth

Similar to LTE band 106, the following channel BW shall be added to table 5.3.5-1:

|  | NR band / SCS / BS channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band |  | SCS kHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | 15 | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n106 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Channel arrangement

The NR-ARFCN for n106 shall be added in table 5.4.2.3-1:

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n106 | 100 | 179200 – <20> – 180200 | 187000 – <20> – 188000 |

For LTE band, the following note was added:

NOTE 7:	In the present version of the specification, only EARFCN 70686 and 134322 is applicable for 3 MHz channel bandwidth.

If the same restriction applies for NR, the following note should be added to the NR-ARFCN table:

NOTE xx:	In the present version of the specification, only NREF 179800 and 187600 is applicable.

For the sync raster, considering only 3 MHz channel BW will be specified for band n106, only the new sync raster design (agreed in last RAN4#107) should be considered for n106.

Considering our proposal for this new sync raster specification ([3]), we would then propose to add the following information to table 5.4.3.3-1:

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n106 | 15 kHz | Case A | 54679 – <1> – 54693 |

Keeping the same restriction on the channel raster, the range for the sync raster should be restricted accordingly. Following note should then be added to the sync raster table:

NOTE xx:	In the present version of the specification, only GSCN [54688-54690] are applicable.

# Conclusion

In this contribution, we analyzed the system parameters when introducing the new US 900MHz NR band.

Proposal: Consider the above band definition, channel bandwidth and channel arrangement. Further discuss if needed the applicability of LTE band 106 channel restriction (EARFCN) for NR.

# References

- RP-230723, Revised WID: Introduction of 900 MHz NR Band in the US, Anterix

- RP-231425, NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Anterix, T-Mobile USA

- R4-23xxxx, Spectrum less than 5 MHz - System parameters, Ericsson