3GPP RAN WG4 Meeting #108	       R4-2313263

Toulouse, France, Aug 21 - 25, 2023

Agenda item:	8.15.2.3

Source:	Apple

Title:	Enhancements to Rel18 Lab Alignment Template

Release:	Rel-18

Related WI:	NR_FR1_TRP_TRS_enh-Perf

Document for:	Discussion and Decision

# 1	Introduction

The work on UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC) has continued into Rel-18, with the work item having been approved in RAN #97 [3] and revised in [4]. During the RAN4 #107 meeting the list of volunteer labs was finalized to commence the lab alignment campaign. This contribution highlights some aspects to enhance the Release 18 lab alignment process.

# 2	Discussion

## 2.1	Background from recent studies by CTIA impacting BHH testing

During the 2022/early 2023 timeframe, there was a detailed study performed by CTIA across nine volunteer labs to determine whether the labs could effectively identify hand degradation by measuring the delta in TRP performance on a select device. Based on the results, it appeared that most labs could successfully identify problematic phantom hands [7].

Also indicated in the study was that SPEAG has identified that phantom properties degrade over time based on handling and the oils in the user’s hand.  When this happens, the dielectric parameters of the phantom hand degrade and the hand loss/loading decreases which results in an inaccurate test result.  A simple way for labs to detect this degradation was proposed using TRP testing in the BHH configuration.

Good vs. Bad phantom hand (i.e. compliant vs. non-compliant hand) TRP deltas [7]

It was found that there was a significant impact on BHHL/BHHR measurements due to degradation of dielectric properties of some hand phantom used in testing and the general knowledge was that this is pronounced for low bands.

Observation 1: An important outcome of the CTIA round-robin in 2022 was that 3 of 11 hands tested appear to be marginal or non-compliant, indicating the potential need for hand phantom monitoring among labs going forward, summarized in a guidance in early 2023.

Observation 2: TRP testing in BHH configuration was used by the round-robin labs participating in the CTIA effort and this configuration highlighted and detected the issue as per [7]

The outcome of the CTIA study was published in February 2023. Among other detailed recommendations, there were  several Hand Phantom maintenance procedures that were suggested.

Several other corrective actions were suggested as an outcome of the report which are detailed in [7]

## 2.2	Enhancements to 3GPP Rel18 Lab Alignment Template

The upcoming Release 18 lab alignment is the first time that the BHH configuration will be used for the planned lab alignment testing on low and high bands. It should also be noted that the Release 17 lab alignment campaign did not involve low bands.

Observation 3: 3GPP Release 18 lab alignment campaign is the first time BHH will be tested. Also, low band is now part of the scope.

Based on the useful outcomes captured in the CTIA round-robin testing and study in [7] and outlined in Section 2.1 of that report, it would be useful to collect some additional information during the 3GPP Rel-18 lab alignment campaign. This could potentially alert all involved stakeholders to take necessary measures and proactively avoid any errors identified in [7] rather than being reactive after all the time and effort to test the LADs across 8 volunteer labs.

This can be accomplished by requesting the information below from volunteer labs.

- Version# of Hand Phantom Used

- Age (estimate) of the Hand Phantom Used

- Estimated date of last verification of Hand Phantom Used along similar lines as suggested in [7] section on “Phantom Hand compliance” or any equivalent Hand Phantom verification procedure

This simple step would provide RAN4 as well as the volunteer labs necessary confidence when proceeding with the lab alignment process.

Proposal 1: Update the “Lab Information” section of lab alignment data measurement template with additional items pertain to the Hand Phantom used (as depicted in the screenshot below).

# 3	Conclusions

This contribution seeks to propose some enhancements to the Rel-18 lab alignment template.

Observation 1: An important outcome of the CTIA round-robin in 2022 was that 3 of 11 hands tested appear to be marginal or non-compliant, indicating the potential need for hand phantom monitoring among labs going forward, summarized in a guidance in early 2023.

Observation 2: TRP testing in BHH configuration was used by the round-robin labs participating in the CTIA effort and this configuration highlighted and detected the issue as per [7]

Observation 3: 3GPP Release 18 lab alignment campaign is the first time BHH will be tested. Also, low band is now part of the 3GPP Rel-18 scope.

Proposal 1: Update the “Lab Information” section of lab alignment data measurement template with additional items pertain to the Hand Phantom used (as depicted in the screenshot below).

# 4	References

- RP-211158, "Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)," vivo, OPPO, CMCC, 3GPP RAN #92, June 2021

- RP-222152, "Status report for WI: Introduction of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC); rapporteur: vivo," 3GPP RAN #97, September 2022

- RP-222669, “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies,” vivo, 3GPP RAN #97, September 2022

- RP-223112, “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC),” vivo, 3GPP RAN #98, December 2022

- R4-2302920, “Working procedure for Rel-18 TRP TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

- R4-2302917, “WF for Rel-18 TRP/TRS requirements,” vivo, 3GPP RAN4 #106, February 2023

- OTANFP_2022_001_003 Good Bad Hand Phantom Round Robin Results V2 Good Bad Hand Phantom Round Robin Results, CTIA Certification Program Working Group Contribution