3GPP TSG-RAN WG4 Meeting # 108	R4-2313269

Toulouse, France, August 21 – August 25, 2023

Title: 	TP on 38.878: Introduction of parameters for link level evaluation

Source: 	Huawei, HiSilicon

Agenda item:	8.18.1.2

Document for:	Approval

- Background

This contribution provides the simulation parameters part in TR 38.878.

Advanced receiver to cancel inter-user interference for MU-MIMO

Scenario and interference modelling

Receiver structure

Analysis on the required information

Link performance characterization

4.4.1	 Parameters for link level evaluation

General link level simulation parameters are captured in Table 4.4-1:

Table 4.4-1: General parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |
| Channel Bandwidth/SCS |  | MHz/KHz | 10/15 |  |  |
| Duplex mode |  |  | FDD |  |  |
| Allocation for interference UE and target UE | Rank allocation |  | 1 | 1 |  |
|  |  |  | 2 | 2 |  |
|  | Scrambling ID |  | Same scrambling ID for both UEs |  |  |
| MIMO configuration |  |  | Rank 1+1: 2T2R Medium Rank 2+2: 4T4R Low |  |  |
| Port allocation |  |  | Rank 1+1: 1000 Rank 2+2: 1000,1001 |  | Rank 1+1: 1001 Rank 2+2: 1002,1003 |
| Number of CDM groups without data |  |  | 1 for paired UE allocated in same CDM groups and 2 for paired UE allocated in different CDM groups |  |  |
| HARQ process number |  |  | 4 |  |  |
| Maximum number of HARQ transmission |  |  | 4 |  |  |
| Precoding model | Target UE |  | Random precoding with Single panel Type 1 per PRB bundling size per slot | Rank 2+2: Select the precoding matrix to ensure orthogonality with target UE Rank 1+1: Select the precoding matrix randomly ensuring the selected precoding matrix shall not be identical to the precoding matrix of target UE |  |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | Frequency domain allocation |  | Full bandwidth allocation |  | Partial bandwidth allocation |
| PDSCH DMRS configuration | DMRS Type |  | DMRS Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Propagation conditions |  |  | Rank 1+1: TDLC300-100 Rank 2+2: TDLA30-10 |  |  |
| Test metric |  |  | SNR @ %70 of maximum Throughput | N/A |  |

Detailed link level simulation parameters are captured in Table 4.4-2:

Table 4.4-2: Simulation parameters

| Case | Target UE |  | Co-scheduled UE |  |  | Receiver assumption |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rank | MCS | Rank | Modulation order | RB allocation |  |
| 1 | 1 | 4 | 1 | QPSK | Full allocation(52PRBs) | MMSE-IRC, E-MMSE-IRC,R-ML receiver with genie aided knowledge of all the required information of scheduled UE |
| 2 |  | 13 |  | QPSK |  |  |
| 3 | 2 | 13 | 2 | QPSK |  |  |
| 4 |  |  |  | 16QAM |  |  |
| 5 |  |  |  | 64QAM |  |  |
| 6 |  | 17 |  | QPSK |  |  |
| 7 |  |  |  | 16QAM |  |  |
| 8 |  |  |  | 64QAM |  |  |
| 9 | 1 | 13 | 1 | QPSK | Full allocation(52PRBs) | E-MMSE-IRC,R-ML receiver with genie aided knowledge of all the required information of scheduled UE except for DMRS port and frequency domain allocation which are blindly detected by target UE |
| 10 |  |  |  |  | Partial allocation(0~25PRBs) |  |
| 11 | 2 | 13 | 2 | 64QAM | Full allocation(52PRBs) |  |
| 12 |  |  |  |  | Partial allocation(0~25PRBs) |  |
| 13 | 1 | 13 | 1 | QPSK | Full allocation(52PRBs) | E-MMSE-IRC,R-ML receiver with genie aided knowledge of all the required information of scheduled UE except for DMRS port, frequency domain allocation and modulation order which are blindly detected by target UE |
| 14 | 2 | 17 | 2 | 16QAM | Full allocation(52PRBs) |  |

4.4.2	 Link level simulation results

4.4.3	 Summary of link level evaluation

Conclusions