3GPP TSG-RAN4 Meeting #108	R4-2313294

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 3560 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [NR_feMIMO-Core]CR on maintenance of feMIMO RRM requirements in R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FeMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-08-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. According to RAN1 agreements, no RRM requirements are persued for simultaneous reception of PDSCH/PDCCH and SSB for L1-RSRP measurement for the case that SSB and PDCCH /PDSCH overlap on the same RE. The applicability of RRM requirements are needed. 2. Editorial changes is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add clarification that performance degradation is expected for the PDSCH/PDCCH and SSB collision case while different PCIs are assumed. 2. Editorial changes to 8.16.3 and 9.13.4.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct or not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.533 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

### 8.16.3	MAC-CE based uplink TCI state switch delay

The requirements in this clause shall apply for UL TCI state switch using separate UL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in UL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.16.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, UE is not expected to transmit on UL based on the target TCI state before UE completes the DL and UL TCI state switch.

For separate UL TCI state switch or joint TCI state switch for PUCCH or PUSCH, or semi-persistent/aperiodic/periodic SRS, when beamCorrespondenceWithoutUL-BeamSweeping is set to 1, upon receiving PDSCH carrying MAC-CE activation command in slot n on serving cell,

-	If target TCI state is known,

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	If target TCI state is unknown,

-	The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + (TL1-RSRP + Tfirst_target-PL-RS + 4*Ttarget_PL-RS + 2ms) / NR slot length.

-	The UE shall be able to transmit with the old UL TCI state until slot n+ THARQ + 3Nslotsubframe,µ.

Where,

-	THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

In FR2, in case that the target PL-RS associated with or included in the target UL or joint TCI state is SSB, the requirements in this clause shall apply when this target PL-RS is maintained by the UE.

-	PL-RS is maintained provided:

-		the target PL-RS is associated with or included in the UL or joint TCI states in the active TCI list for PUSCH/PUCCH/SRS transmissions

-	There are no more than 4 different RS activated as PL-RS per serving cell among all active UL TCI states (UL or joint TCI state) for PUSCH/PUCCH/SRS transmissions

-	The target pathloss reference signal remains detectable during TCI state switching period

-	SNR of the target pathloss reference signal≥-3dB

-	The associated SSBs with the target pathloss reference signal remain detectable during the TCI state switching period.

-	SNR of the associated SSB ≥-3dB

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after L1-RSRP measurement when target TCI state is unknown.

-	Tfirst_target-PL-RS is time to first pathloss RS transmission after MAC CE command is decoded by the UE for known TCI State.

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB or NZP CSI-RS when PL-RS is associated with serving cell

-	Ttarget_PL-RS is the periodicity of the target pathloss reference signal which would be SSB when PL-RS is associated with PCI different from serving cell

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSRP_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1 or 9.13.4.1,

-	with the assumption of M=1

-	with TReport = 0

-	TL1-RSRP_Measurement_Period_CSI-RS for CSI-RS as specified in clause 9.5.4.2

-	CSI-RS based L1-RSRP measurement only apply for TCI state switch when source RS is associated with serving cell

-	configured with higher layer parameter repetition set to ON

-	with the assumption of M=1 for periodic CSI-RS

-	for aperiodic CSI-RS if number of resources in resource set at least equal to MaxNumberRxBeam

-	with TReport = 0

The requirements specified in this clause are applicable if no more than 4 different RSs are activated as PL-RS per serving cell among all active UL (or joint) TCI states.

<End of Change #1>

<Start of Change #2>

9.5.2	Requirements applicability

The requirements in clause 9.5 apply, provided:

-	The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band.

A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.19.2 and 10.1.20.2 for FR1 and FR2, respectively, for a corresponding band,

-	CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.4.2 for a corresponding band.

A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Requirements are defined for periodic, semi-persistent and aperiodic resources.

In case the SSB resources configured for L1-RSRP measurements outside SMTCs are overlapped with the PDSCH/PDCCH that is associated to a PCI different from serving cell, L1-RSRP measurement performance degradation is expected.

<End of Change #2>

<Start of Change #3>

9.13.2	Requirements Applicability

The requirements in the clause 9.13 are applicable to inter-cell beam management and inter-cell multi-TRP scenarios.

The requirements in clause 9.13 apply, provided the SSB from cell with PCI different from serving cell configured for L1-RSRP if the following conditions are met:

-	the number of cells with PCI different from seving cells Nmax = 1 for FR2 and Nmax = maxNrofAdditionalPCI for FR1. Where, maxNrofAdditionalPCI is defined in TS 38.331 [2].

-	The SSB from the cell with different PCI completely contained in the active BWP or associated with initial downlink BWP of the UE

-	The SSB of the cell with different PCI from serving cell has the same SCS, SFN offset and center frequency as the SSB of the serving cell

-	The timing difference of arrival at UE between the SSBs of serving cell and cell with different PCI is less than CP length of the corresponding SCS

-	The cell with different PCI from serving cell is known

-	The SSB resources configured for L1-RSRP measurements are measurable

An SSB resource configured for L1-RSRP for cell with different PCI from serving cell shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band.

The cell with different PCI from serving cell is considered as known if the following conditions are met in this requirement:

-	The UE has sent a valid L3 measurement report during the last 5 seconds, and

-	The SSB from the cell with different PCI remains detectable according to the cell identification requirements specified in clause 9.2.

Otherwise, the cell is unknown.

In case the SSB resources configured for L1-RSRP measurements outside SMTCs are overlapped with PDSCH/PDCCH that is associated to serving cell, L1-RSRP measurement performance degradation is expected.

<End of Change #3>

<Start of Change #4>

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting

If a cell with PCI different from serving cell is known according 9.13.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_CDP. The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured, and

-	SSBs of CDP outside SMTCs for L1-RSRP measurement are not overlapped with concurrent gaps.

The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 9.13.4.1-1 for FR1, The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 9.13.4.1-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, where

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	N= 8.

For FR1,

-	P=11-TSSB_CDPMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

For FR2,

-	P is PL1_sharing*Psharing factor, CDP, when SSB is not overlapped with measurement gap and SSB is fully overlapped with SMTC period (TSSB_CDP = TSMTCperiod), and TSSB_SC = TSMTCperiod.

-	P is  PL1_sharing*Psharing factor1-TSSBxRP, when SSB is partially overlapped with measurement gap and SSB is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP), and TSSB_SC = TSMTCperiod.

-	P2=11-TSSB_CDPTSMTCperiod, when SSB is not overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod).

-	P2= 11-TSSB_CDPMGRP-TSSB_CDPTSMTCperiod, when SSB is partially overlapped with measurement gap and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB_CDP < 0.5*TSMTCperiod

-	P2= 11-TSSB_CDPmin(TSMTCperiod,MGRP),when SSB is partially overlapped with measurement gap (TSSB_CDP <MGRP) and SSB is partially overlapped with SMTC occasion (TSSB_CDP < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	If SSB resource from serving cell is configured for L1-RSRP measurements, and P1 is valid accoding to 9.5.4.1, and any symbol of the SSBs from serving cell and cell with different PCI are overlapping or adjacent (in time domain)

-	P = P21-P2*TSSB_CDPP1*TSSB_SC ,   if P2*TSSB_CDP < P1*TSSB_SC.

-	P = P2, if P2*TSSB_CDP> P1*TSSB_SC.

-	P = 2*P2, if P1*TSSB_SC = P2*TSSB_CDP.

-	Otherwise, P = P2

<Unchanged part omitted>

<End of Change #4>