3GPP TSG-RAN WG4 Meeting #108 	R4-2313300

# Toulouse, France, August 21 – August 25, 2023

Source: 	vivo

Title: 	Discussion on cell switch delay requirements in R18 LTM

Agenda Item:	8.24.2.3

Document for:	Discussion

Introduction

In RAN4 106-bis-e and RAN4 107, R18 L1L2-triggered mobility was discussed in RRM session and the WFs are agreed in [1], [2].

Based on discussion RAN4 has common understanding on the following baseline delay requirements for cell switch.

Issue 3-3-1: RACH-based Cell switch delay for Pcell/PSCell

< Agreement>:

  - The baseline of RACH-based cell switch delay requirements is Tdelay = Tcmd + Tprocessing / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity /TIU, where Tuncertainity /TIU is the uncertainty in acquiring the first available PRACH occasion in the new cell.

    - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed.

    - FFS: add/remove/modify other component(s).

Note: ‘/’ means ‘or’ here.

Issue 3-3-2: RACH-less Cell switch delay for Pcell/PSCell

< Agreement>:

  - The baseline of RACH-less cell switch delay requirements is Tdelay = Tcmd + Tprocessing,2 / TLTM-processing + Tsearch + T∆ + Tmargin + Tuncertainity/TIU,

    - FFS: the ending point

    - FFS: the exact value of each component. Some components can be 0 in certain cases, if agreed.

    - FFS: add/remove/modify other component(s).

Note: ‘/’ means ‘or’ here.

The above baseline is generally aligned with RAN2’s assumption listed in [3].

Figure 2-1: Components of Mobility Latency (before enhancements) in [3]

Based on all above information, we provide our views on the cell switch delay requirements for R18 L1L2-triggered mobility.

Discussion

<On generals and principles>

In last meeting, the following issue is discussed.

Issue 3-2-2: Procedure of cell switch

<Way Forward> FFS the following options:

  - Option 1 (MTK): Further discuss whether UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 2 (CTC):  Under the condition that target cell is known, UE can perform T/F fine tracking (TΔ) if needed at first and then L1/L2/L3 processing (Tprocessing,2) to reduce the interruption time during cell switch.

  - Option 3 (Apple): If T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2) as baseline.

  - Option 4 (xiaomi): If T/F fine tracking (TΔ) is needed after receiving cell switch command, UE is not required to perform it before L1/L2/L3 processing (Tprocessing,2)

  - Option 5 (vivo): UE cannot perform T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. If needed, some other L1 configuration, L2/L3 reconfiguration and L2 reset can be performed during cell switch, i.e., in some cases it can be performed after the T/F tracking.

  - Option 6 (Ericsson): If UE is configured with pre-synchronisation, UE can perform T/F fine time tracking before the cell switch command and UE do not need extra delay for T/F tracking after receiving cell switch command

As discussed in [3], TRS based tracking is already precluded in RAN1. SSB based rough tracking can be performed before cell switch command, while TRS based fine tracking can only be performed after cell switch. UE can perform TRS based fine tracking after L1/L2/L3

Proposal 1  UE can not perform TRS-based fine T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. From conformance requirement perspective, UE may perform TRS tracking after L1/L2/L3 processing.

<On Detail of cell switch delay requirements for Pcell/PSCell >

Full configuration, reference configuration and delta configuration are being discussed in RAN2. In RAN2 #112 meeting the following agreements is achieved.

- Agreements in RAN2 #112:

Usage of reference configuration:

- 	Candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration (FFS if done at cell switch or before the cell switch)

- 	The complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset RLC or PDCP.

-	To support reconfigurations that requires reset of RLC PDCP, this should be possible (in principle same a full config)

-	FFS if more than RLC PDCP should be kept and how much of “replacing” need to be specified.

-	FFS if the reference configuration can be derived from the current UE configuration at some point of time.

Agreement in RAN2 #112-bis-e:

Whether the Reference configuration is a complete configuration or not is up to the network implementation.

Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration.

The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current).

Confirm that only the replacement procedure (the “full config without L2 reset”) is supported for Execution of LTM cell switch.

The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

Based on reference configuration, the UE internal memory needed for saving RRC configurations can be saved. Forming the full configuration before cell switch does not have significant gain in reducing interruption, but may have quite large cost in UE’s memory if multiple candidate cells are configured.

Proposal 2  For the baseline requirement, RAN4 assume UE forms the complete configuration and applies it during the execution of cell switch command.

However, in last meeting, the following issue is discussed.

Issue 3-3-6-1: Execution time

<Way Forward> FFS the following proposals:

  - Texecution_time for ASN.1 decoding and validity/compliance check of target cell configuration should be added in the cell switch delay requirements.

In our understanding, the checking of ASN.1 should be performed during TRRC. In case the checking fails, UE may enter RLF state right after the decoding RRC reconfiguration is completed. In CHO, execution time is for UE to form the full configuration, rather than decoding and checking. We can agree that T_execution time needs to be added in cell switch command, but the explanation needs to be more accurate. Moreover, clarification is needed regarding the difference between this term and the term Tprocessing,2

Proposal 3  Texecution_time for the forming of complete RRC configuration from reference configuration should be added in the cell switch delay requirements. The relation between this term and the Tprocessing,2 needs more clarification in RAN4.

In last meeting, the following issue is discussed.

Issue 3-3-2-1: T/F fine tracking: TΔ and Tmargin

<Way Forward> FFS the following options:

  - Option 1 (CTC, Nokia, Ericsson, Apple, CATT, MTK,): The baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms. FFS: whether TΔ and Tmargin can be 0 under certain conditions.

    - Option 1a (CATT, MTK): the baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms

      - TΔ and Tmargin can be 0 if UE has obtained SFN of the target cell and have fine tracked the target cell in the latest 160ms.

    - Option 1b (CTC, Ericsson): the baseline is: TΔ=1 Tfirst-RS, Tmargin = 2ms

      - TΔ and Tmargin can be 0 under the condition that pre-Sync for candidate cell(s) has been performed before cell switch command.

  - Option 2 (CMCC,): TΔ = 0 for the case that DL synchronization for candidate cell(s) is performed before cell switch command or target cell is current active SCell.

  - Option 3 (vivo, Huawei, ZTE): TΔ = 0 for the case that SSB based fine synchronization for candidate cell(s) is performed before cell switch command.

Issue 3-3-4-1: TCI state switching time

<Way Forward> FFS the following options:

  - Option 1 (Huawei): If SSB based fine synchronization is performed before cell switch, TCI state switch delay is needed.

  - Option 2 (Nokia, Ericsson): wait for more progress in RAN1.

  - Option 3 (vivo): add extra switching time, while the detailed value equals to beam application time, which will be further specified by RAN1.

Issue 3-3-5-1: Extra time for PL-RS measurement

<Way Forward> FFS the following option:

  - Option 1 (MTK): For PCell/PSCell switch delay, further discuss whether extra time is needed to measure PL-RS.

Firstly, RAN1 has made the following agreements.

Agreement in RAN1 #112

- The agreement on scenario 2 (Beam indication together with cell switch command) at RAN1#111 is further clarified as the following:

  - Beam indication for the target cell(s) is conveyed in the MAC CE used for LTM triggering for scenario 2

Agreement in RAN1 #112-bis-e

For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported:

- Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell,

- Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell

  - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command

- Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability

FFS: signalling details for TCI state activation

FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed

Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

In latest RAN1 agreements, RAN1 has clarified that their previous agreement was for beam indication, which is DCI based TCI switch. Actually, the TCI state activation is assumed to be performed before cell switch. As discussed in our companion paper, DL synchronization can be performed before cell switch command. Although only SSB-based synchronization is considered, in our view there is no need to add TCI state switching time for this case.

Based on above, the TCI state switch delay would be the beam application time. In R17, RAN1 has the basic design for the beam application time. In R18, the MAC CE based beam application time should also be discussed and clarified in RAN1 first. Whether the beam application and cell switch can be performed simultaneously can be further discussed in RAN4.

On the other hand, after cell switch command, UE is expected to transmit based on the target TCI state. However, if PL-RS of target TCI is not maintained, additional time for PL-RS updating is needed. This issue is still being discussed in R17 feMIMO WI. For FR2, current spec still needs revision, as discussed in [4]. Therefore, it is proposed to wait for more conclusion from the R17 feMIMO maintenance.

Proposal 4  At least support TΔ = 0, Tsearch = 0, Tmargin = 0, for the case that DL synchronization for candidate cell(s) is completed before cell switch command.

Proposal 5  Add a new term Tswitch, while the detailed value would equals to beam application time, which will be further specified by RAN1.

Proposal 6  Whether additional time for PL-RS measurement is needed follows the conclusions of R17_feMIMO/ R16_eMIMO maintenance.

In last meeting, the following issue is discussed.

Issue 3-3-8-1: Tinterruption

<Way Forward> FFS the following proposals:

- Proposals

  - Proposal 1 (CATT, CTC, CMCC, ZTE, OPPO): The components of L1/L2 cell switch interruption Tinterruption are the components of L1/L2 inter-cell mobility delay except Tcmd

  - Proposal 2 (Nokia): Interruption caused by LTM handover shall not exceed the interruption time caused by L3 handover

  - Proposal 3 (Huawei): There is almost no interruption during cell switch procedure when target Pcell/SCell is current SCell/PCell.

  - Proposal 4 (CATT, vivo):

    - For RACH-based cell switch, T_interruption at least include the time of Tprocessing,2 and T_IU.

    - For RACH-less cell switch, T_interruption at least include T_processing,2

The LTM interruption mainly comprise the needed RF warm-up and reconfiguration of L1/L2/L3, which should be T_processing,2 in Figure 1. For the case when RACH is needed, RACH collision needs to be considered in the interruption time. Before RACH is sent to the target cell, there is no possibility that network can schedule this UE in the corresponding CG.

On the other hand, if TCI state is indicated in cell switch command, no interruption is needed according to R17 feMIMO conclusions.

Proposal 7  For RACH-based cell switch, T_interruption at least include the time of T_processing,2 and T_IU.

For RACH-less cell switch, T_interruption at least include T_processing,2

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  UE can not perform TRS-based fine T/F tracking before necessary L1 reconfiguration, which get the UE's L1 ready to receive DL of the target cell. From conformance requirement perspective, UE may perform TRS tracking after L1/L2/L3 processing.

Proposal 2  For the baseline requirement, RAN4 assume UE forms the complete configuration and applies it during the execution of cell switch command.

Proposal 3  Texecution_time for the forming of complete RRC configuration from reference configuration should be added in the cell switch delay requirements. The relation between this term and the Tprocessing,2 needs more clarification in RAN4.

Proposal 4  At least support TΔ = 0, Tsearch = 0, Tmargin = 0, for the case that DL synchronization for candidate cell(s) is completed before cell switch command.

Proposal 5  Add a new term Tswitch, while the detailed value would equals to beam application time, which will be further specified by RAN1.

Proposal 6  Whether additional time for PL-RS measurement is needed follows the conclusions of R17_feMIMO/ R16_eMIMO maintenance.

Proposal 7  For RACH-based cell switch, T_interruption at least include the time of T_processing,2 and T_IU.

For RACH-less cell switch, T_interruption at least include T_processing,2

- References

[1]  R4-2306395  WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek.  RAN4 #106-bis-e

[2]  R4-2310160  WF on NR mobility enhancements (part1), MediaTek.  RAN4 #107

[3]  R4-2313298  Discussion on general aspects in R18 LTM, vivo.  RAN4 #108

[4]  R4-2313292  Discussion on maintenance issues in R17 feMIMO RRM requirements, vivo.  RAN4 #108