GPP TSG-RAN WG4 Meeting # 108	R4-2313307

Toulouse, FR, August 21 – August 25, 2023

Agenda Item:	8.4.3.1.1

Source:	Ericsson

Title:	Remaining issues on general aspects for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

In this draft, we go through the remaining issues on general aspects. We examine the applicability rules for PDSCH and CSI tests in FDD duplex mode and provide our views on carrier aggregation (CA) for 8 Rx in FR1.

Issue 1-1: Whether to consider FDD requirements RAN4 starts to discuss FDD part from next meeting and focuses on TDD part for this meeting. Issue 1-2: Applicability rules for PDSCH/PDCCH/PBCH tests Table 5.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list; Exceptions. UE supports only 2RX; PDSCH; All tests in Clause 5.2.2. PDCCH; All tests in Clause 5.3.2. PBCH; All tests in Clause 5.4.2. UE supports only 4RX or both 2RX and 4RX; PDSCH; All tests in Clause 5.2.3 (Note 2). PDCCH; All tests in Clause 5.3.3 (Note 2). PBCH; All tests in Clause 5.4.2 or 5.4.3 (Note 1). UE supports 8Rx, 4Rx and 2Rx; PDSCH; Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause 5.2.4.(Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.2.1-6 (Test 4-1). or; PDCCH; All tests in Clause 5.3.3.(Note 2,3). UE supports only 8Rx and 4Rx; PBCH; All tests in Clause 5.4.3(Note 1). UE supports only 8Rx and 2Rx; PDSCH; Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause 5.2.4.(Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2.(Note 2,4). PBCH; All tests in Clause 5.4.2. UE supports only 8Rx; PDSCH; All tests in Clause 5.2.4.(Note 2). PDCCH; N/A. PBCH; N/A. Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied.. Issue 1-3: Applicability rules for CSI test Updated Table 6.1.1.2-1: Requirements applicability Supported RX antenna ports; Test type; Test list; Exception. UE supports only 2RX; CQI; All tests in Clause 6.2.2. PMI; All tests in Clause 6.3.2. RI; All tests in Clause 6.4.2. UE supports only 4RX or both 2RX and 4RX; CQI; All tests in Clause 6.2.3. PMI; All tests in Clause 6.3.3. RI; All tests in Clause 6.4.3. UE supports 2Rx, 4Rx and 8Rx, or UE supports 4Rx and 8Rx; CQI; Tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4; If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.1.1.1. PMI; All tests in Clause 6.3.3. RI; All tests in Clause 6.4.3. UE supports 2Rx and 8Rx; CQI; Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4; If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.1.1.1. PMI; All tests in Clause 6.3.2. RI; All tests in Clause 6.4.2. UE supports only 8Rx; CQI; All tests in Clause 6.2.4. PMI; N/A. RI; N/A. Note 1: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 2: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied..

# 2	Remaining issues on the general aspects for UE demodulation and CSI reporting requirements

A good progress has been achieved on the general aspects to be considered. However, it remained the applicability rules to be extended to FDD duplex case for 8 Rx UE in FR1.

Proposal 1: Extend the applicability rules to FDD duplex mode.

In the following, we update the applicability rules for PDSCH, PDCCH and PBCH test cases that have been defined for TDD duplex mode in [1] to the FDD duplex mode case.

Proposal 2: Consider the applicability rules for PDSCH, PDCCH and PBCH test cases in TS 38.101-4 in TDD and FDD duplex modes as follow

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exceptions |
| --- | --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |  |
|  | PDCCH | All tests in Clause 5.3.2 |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |  |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |  |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause 5.2.4.(Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4, Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6 (Test 4-1) |
| or | PDCCH | All tests in Clause 5.3.3.(Note 2,3) |  |
| UE supports only 8Rx and 4Rx | PBCH | All tests in Clause 5.4.3(Note 1) |  |
| UE supports only 8Rx and 2Rx | PDSCH | Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause 5.2.4.(Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4 and Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2.(Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.4.(Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

Accordingly, we update the applicability rules for CSI reporting test cases to include FDD duplex mode.

Proposal 3: Consider the applicability rules for CSI reporting test cases in TS 38.101-4 in TDD and FDD duplex modes as follow

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exception |
| --- | --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |  |
|  | PMI | All tests in Clause 6.3.2 |  |
|  | RI | All tests in Clause 6.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |  |
|  | PMI | All tests in Clause 6.3.3 |  |
|  | RI | All tests in Clause 6.4.3 |  |
| UE supports 2Rx, 4Rx and 8Rx, or UE supports 4Rx and 8Rx | CQI | Tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4 | If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.1.1.1 |
|  | PMI | All tests in Clause 6.3.3 |  |
|  | RI | All tests in Clause 6.4.3 |  |
| UE supports 2Rx and 8Rx | CQI | Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4 | If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.1.1.1 |
|  | PMI | All tests in Clause 6.3.2 |  |
|  | RI | All tests in Clause 6.4.2 |  |
| UE supports only 8Rx | CQI | All tests in Clause 6.2.4 |  |
|  | PMI | N/A |  |
|  | RI | N/A |  |
| Note 1: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 2: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

# 3	Carrier Aggregation for 8RX in FR1

RAN#100 has approved the revised WID RP-231490 that RAN4 will specify UE RF requirements for both single carrier and CA/DC for 8Rx, FFS the band combos and configurations to be defined.

Proposal 4: Define PDSCH demodulation requirements for 8RX using carrier aggregation in both TDD and FDD duplex modes using CBW of 40 MHz and 10 MHz, respectively.

Proposal 5: Do not define CA test cases for 8RX CQI reporting in FR1.

# 4	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: Limit the applicability rules for TDD case and postpone FDD related rules to be treated thoroughly once FDD cases are defined.

Proposal 2: Consider the applicability rules for PDSCH, PDCCH and PBCH test cases in TS 38.101-4 in TDD and FDD duplex modes as follow

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exceptions |
| --- | --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |  |
|  | PDCCH | All tests in Clause 5.3.2 |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2) |  |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 (Note 1) |  |
| UE supports 8Rx, 4Rx and 2Rx | PDSCH | Tests in Clause 5.2.3.(Note 2, 3) All tests in Clause 5.2.4.(Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4 and Table 5.2.3.2.1-4, Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6 and Table 5.2.3.2.1-6 (Test 4-1) |
| or | PDCCH | All tests in Clause 5.3.3.(Note 2,3) |  |
| UE supports only 8Rx and 4Rx | PBCH | All tests in Clause 5.4.3(Note 1) |  |
| UE supports only 8Rx and 2Rx | PDSCH | Tests in Clause 5.2.2.(Note 2, 4) All tests in Clause 5.2.4.(Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4 and Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2.(Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx | PDSCH | All tests in Clause 5.2.4.(Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

Proposal 3: Consider the applicability rules for CSI reporting test cases in TS 38.101-4 in TDD and FDD duplex modes as follow

Table 6.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list | Exception |
| --- | --- | --- | --- |
| UE supports only 2RX | CQI | All tests in Clause 6.2.2 |  |
|  | PMI | All tests in Clause 6.3.2 |  |
|  | RI | All tests in Clause 6.4.2 |  |
| UE supports only 4RX or both 2RX and 4RX | CQI | All tests in Clause 6.2.3 |  |
|  | PMI | All tests in Clause 6.3.3 |  |
|  | RI | All tests in Clause 6.4.3 |  |
| UE supports 2Rx, 4Rx and 8Rx, or UE supports 4Rx and 8Rx | CQI | Tests in Clause 6.2.3(Note 1) All tests in Clause 6.2.4 | If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.3.1.1.1 |
|  | PMI | All tests in Clause 6.3.3 |  |
|  | RI | All tests in Clause 6.4.3 |  |
| UE supports 2Rx and 8Rx | CQI | Tests in Clause 6.2.2(Note 2) All tests in Clause 6.2.4 | If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.2.1.1 If UE has passed Tests in Clause 6.2.4, UE can skip tests in Clause 6.2.2.1.1.1 |
|  | PMI | All tests in Clause 6.3.2 |  |
|  | RI | All tests in Clause 6.4.2 |  |
| UE supports only 8Rx | CQI | All tests in Clause 6.2.4 |  |
|  | PMI | N/A |  |
|  | RI | N/A |  |
| Note 1: 8Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx with data source from system simulator, and the other 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 2: 8Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx with data source from system simulator, and the other 6 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. |  |  |  |

Proposal 4: Define PDSCH demodulation requirements for 8RX using carrier aggregation in both TDD and FDD duplex modes using CBW of 40 MHz and 10 MHz, respectively.

Proposal 5: Do not define CA test cases for 8RX CQI reporting in FR1.

# References

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei.