3GPP TSG-RAN WG4 Meeting # 108	R4-2313308

Toulouse, FR, August 21 – August 25, 2023

Agenda Item:	8.4.3.1.2

Source:	Ericsson

Title:	Remaining issues on PDSCH requirements for 8 Rx in FR1

Document for:	Discussion

# 1	Introduction

Last meeting #107, further progress has been achieved for PDSCH in TDD duplex mode [1], summarized as follow. In this meeting, we extend our proposals for FDD mode and consider carrier aggregation for both modes TDD and FDD.

Issue 2-1: Propagation conditions and antenna correlation for Rank 2 test - TDLC300-100 ULA Medium B Issue 2-2: MCS for Rank 2 test - Agree [MCS 20] (Table 2) for this meeting, if any issues are figured out for next meeting, MCS 19 (Table 1) will be selected Issue 2-3: MCS for Rank 4 test - Agree [MCS 26] (Table 1) for this meeting, if any issues are figured out for next meeting, MCS 17 (Table 1) will be selected Issue 2-4: MCS configuration for Rank 8 test - MCS 17

# 2	PDSCH remaining issues

In this meeting, we go through the remaining issues that still need to be addressed for TDD. Furthermore, we are considering FDD with same propagation environment and MCS levels as agreed for TDD case.

## 2.1 TDD duplex mode

In this section, we provide PDSCH demodulation requirements in TDD mode for Rank 2, Rank 4, and Rank 8.

Observation 1: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 8 DL MIMO Layers in FR1 TDD using MCS17 (Table 1) under TDLA30-10 propagation environment, where an SNR of 22.1 dB is required to achieve 70% of the peat throughput when considering the impairments. This requirement remains 2.3 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 1 is provided for PDSCH mapping Type A test cases with Rank 8.

Proposal 1: Consider PDSCH mapping Type A for 8 Rx UE and Rank 8 in FR1 TDD mode as follow

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB)/ | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 22.1 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

For 8RX UE with Rank 4, the agreement has been to check PDSCH performance requirements considering two different MCS level for 64QAM (Table 1) given by MCS17 and MCS26.

Observation 2: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 4 DL MIMO Layers in FR1 TDD using MCS26 (Table 1) under TDLA30-10 propagation environment, where an SNR of 21.3 dB is required to achieve 70% of the peat throughput. This requirement remains 3.1 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE in FR1 TDD mode, Proposal 2 is provided for PDSCH mapping Type A test cases with Rank 4.

Proposal 2: Consider PDSCH mapping Type A for 8 Rx UE and Rank 4 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.43 MCS 17 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 11.6 |
| 2-2 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 21.3 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

For Rank 2, the discussion still holds to check whether 256QAM can be achieved or not. To this end, WF [1] recommended to check the PDSCH demodulation requirements considering 64QAM MCS19 (Table 1) and 256QAM MCS20 (Table 2) where TDLC300-100 channel model with Medium B correlation.

Our view is that 256QAM has been used in Table 5.2.3.2.1-3 [2] for 4 Rx UE with Rank 1. Consequently, we do believe, and based on our simulation results for 8 Rx UE, 256QAM can also be considered for Rank 2 in 2x8 ULA Medium B channels.

Observation 3: It is worth reminding that 256QAM has been used in Table 5.2.3.2.1-3 (TS 38.101-4) for 4 Rx UE with Rank 1. Consequently, and based on our simulation results in R4-2313309 [3], 256QAM can be considered for 8 RX UE with Rank 2 in 2x8 ULA Medium B channels.

Observation 4: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2313309 [3] show that MCS20 (Table 2) can be achieved in TDLC300-100 ULA Medium B channel, where an SNR of 21.3 dB is required to achieve 70% of the peak throughput. This requirement remains 9.2 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 3 is provided as our strong preference (to keep both test cases) for PDSCH mapping Type A test cases with Rank 2.

Proposal 3: Consider PDSCH mapping Type A for 8 Rx UE and Rank 2 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 Note1 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 Note2 | 40 / 30 (106) | FR1.30-1 | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Note 2: MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for Test 3-1. Use Tx EVM 3% for Test 3-2. |  |  |  |  |  |  |  |  |

## 2.2 FDD duplex mode

In this section, we provide PDSCH demodulation requirements in FDD mode for Rank 2, Rank 4, and Rank 8.

Observation 5: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 8 DL MIMO Layers in FR1 FDD using MCS17 (Table 1) under TDLA30-10 propagation environment, where an SNR of 21.7 dB is required to achieve 70% of the peat throughput when considering the impairments. This requirement remains 2.7 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 1 is provided for PDSCH mapping Type A test cases with Rank 8.

Proposal 4: Consider PDSCH mapping Type A for 8 Rx UE and Rank 8 in FR1 FDD mode as follow

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB)/ | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 10 / 15 (52) | FDD | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 21.7 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

For 8RX UE with Rank 4, the agreement has been to check PDSCH performance requirements considering two different MCS level for 64QAM (Table 1) given by MCS17 and MCS26.

Observation 6: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 4 DL MIMO Layers in FR1 FDD using MCS26 (Table 1) under TDLA30-10 propagation environment, where an SNR of 20.7 dB is required to achieve 70% of the peat throughput. This requirement remains 3.7 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE in FR1 FDD mode, Proposal 2 is provided for PDSCH mapping Type A test cases with Rank 4.

Proposal 5: Consider PDSCH mapping Type A for 8 Rx UE and Rank 4 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 10 / 15 (52) | FDD | 64QAM, 0.43 MCS 17 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 11.7 |
| 2-2 | 10 / 15 (52) | FDD | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 20.7 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

For Rank 2, We keep same arguments as provided for TDD mode.

Observation 7: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2313309 [3] show that MCS20 (Table 2) can be achieved in TDLC300-100 ULA Medium B channel, where an SNR of 20.8 dB is required to achieve 70% of the peak throughput. This requirement remains 9.7 dB below the maximum SNR levels for conducted requirements.

For 8 Rx UE, Proposal 6 is provided as our strong preference (to keep both test cases) for PDSCH mapping Type A test cases with Rank 2.

Proposal 6: Consider PDSCH mapping Type A for 8 Rx UE and Rank 2 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 Note1 | 10 / 15 (52) | FDD | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 Note2 | 10 / 15 (52) | FDD | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Note 2: MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for Test 3-1. Use Tx EVM 3% for Test 3-2. |  |  |  |  |  |  |  |  |

# 3	PDSCH demodulation requirements for CA

RAN#100 has approved the revised WID RP-231490 that RAN4 will specify UE RF requirements for both single carrier and CA/DC for 8Rx, FFS the band combos and configurations to be defined. Our view is to consider same channel bandwidths as for TDD and FDD cases in CC. Furthermore, we believe that MCS17 (Table 1) is relevant when considering 8RX using 8 layers.

Proposal 7: Consider PDSCH demodulation requirements for CA with the following test cases.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 Note1 | 40 / 30 (106) | TDD | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 Note1 | 10 / 15 (52) | FDD | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for Test 1-1 and Test 1-2. |  |  |  |  |  |  |  |  |

# 4	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Observation 1: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 8 DL MIMO Layers in FR1 TDD using MCS17 (Table 1) under TDLA30-10 propagation environment, where an SNR of 22.1 dB is required to achieve 70% of the peat throughput when considering the impairments. This requirement remains 2.3 dB below the maximum SNR levels for conducted requirements.

Proposal 1: Consider PDSCH mapping Type A for 8 Rx UE and Rank 8 in FR1 TDD mode as follow

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB)/ | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 22.1 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

Observation 2: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 4 DL MIMO Layers in FR1 TDD using MCS26 (Table 1) under TDLA30-10 propagation environment, where an SNR of 21.3 dB is required to achieve 70% of the peat throughput. This requirement remains 3.1 dB below the maximum SNR levels for conducted requirements.

Proposal 2: Consider PDSCH mapping Type A for 8 Rx UE and Rank 4 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.43 MCS 17 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 11.6 |
| 2-2 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 21.3 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

Observation 3: It is worth reminding that 256QAM has been used in Table 5.2.3.2.1-3 (TS 38.101-4) for 4 Rx UE with Rank 1. Consequently, and based on our simulation results in R4-2313309 [3], 256QAM can be considered for 8 RX UE with Rank 2 in 2x8 ULA Medium B channels.

Observation 4: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2313309 [3] show that MCS20 (Table 2) can be achieved in TDLC300-100 ULA Medium B channel, where an SNR of 21.3 dB is required to achieve 70% of the peak throughput. This requirement remains 9.2 dB below the maximum SNR levels for conducted requirements.

Proposal 3: Consider PDSCH mapping Type A for 8 Rx UE and Rank 2 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | TDD UL-DL pattern | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 Note1 | 40 / 30 (106) | FR1.30-1 | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 Note2 | 40 / 30 (106) | FR1.30-1 | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Note 2: MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for Test 3-1. Use Tx EVM 3% for Test 3-2. |  |  |  |  |  |  |  |  |

Observation 5: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 8 DL MIMO Layers in FR1 FDD using MCS17 (Table 1) under TDLA30-10 propagation environment, where an SNR of 21.7 dB is required to achieve 70% of the peat throughput when considering the impairments. This requirement remains 2.7 dB below the maximum SNR levels for conducted requirements.

Proposal 4: Consider PDSCH mapping Type A for 8 Rx UE and Rank 8 in FR1 FDD mode as follow

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB)/ | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 10 / 15 (52) | FDD | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 21.7 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

Observation 6: Referring to our simulation results in R4-2313309 [3], we note that 8 Rx UE can support 4 DL MIMO Layers in FR1 FDD using MCS26 (Table 1) under TDLA30-10 propagation environment, where an SNR of 20.7 dB is required to achieve 70% of the peat throughput. This requirement remains 3.7 dB below the maximum SNR levels for conducted requirements.

Proposal 5: Consider PDSCH mapping Type A for 8 Rx UE and Rank 4 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 10 / 15 (52) | FDD | 64QAM, 0.43 MCS 17 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 11.7 |
| 2-2 | 10 / 15 (52) | FDD | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 20.7 |
| MCS were defined based on TS 38.214 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for 64QAM |  |  |  |  |  |  |  |  |

Observation 7: Considering 256QAM for 8 Rx UE with Rank 2, our simulation results in R4-2313309 [3] show that MCS20 (Table 2) can be achieved in TDLC300-100 ULA Medium B channel, where an SNR of 20.8 dB is required to achieve 70% of the peak throughput. This requirement remains 9.7 dB below the maximum SNR levels for conducted requirements.

Proposal 6: Consider PDSCH mapping Type A for 8 Rx UE and Rank 2 with the following test cases and parameters

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 Note1 | 10 / 15 (52) | FDD | 64QAM, 0.56 MCS 13 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| 3-2 Note2 | 10 / 15 (52) | FDD | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | TBD |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Note 2: MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for Test 3-1. Use Tx EVM 3% for Test 3-2. |  |  |  |  |  |  |  |  |

Proposal 7: Consider PDSCH demodulation requirements for CA with the following test cases.

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Duplex Mode | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 Note1 | 40 / 30 (106) | TDD | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| 1-2 Note1 | 10 / 15 (52) | FDD | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | TBD |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for Test 1-1 and Test 1-2. |  |  |  |  |  |  |  |  |

# References

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.8.0.

- R4-2313309, “Simulation results for 8 Rx PDSCH requirements in FR1”, Ericsson.

# Appendix	Simulation assumption

## A.1	PDSCH Simulation Parameters

Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH

Table 1: Simulation assumptions for PDSCH performance

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD, FDD |
| TDD pattern |  |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  |  | TDD: 30/40 FDD: 15/10 |
| MIMO layer and Antenna configuration |  |  | Rank 2: 2x8 Rank 4: 4x8 Rank 8: 8x8 |
| Propagation condition and antenna correlation |  |  | Rank 2: TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) Rank 4: TDLA30-10 Low Rank 8: TDLA30-10 Low |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL |  | Rank2/4: Single-symbol Rank 8: Double-symbol |
| TRS configuration |  |  | Symbol#{5, 9} |
| PT-RS |  |  | Not configured |
| NZP-CSI-RS configuration |  |  | Rank 8: row 6 |
| Overhead |  |  | 0 |
| N1 and N2 configurations for 8Tx cases |  |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  |  | Not schedule PDSCH in special slot for 8 layers cases |
| Number of HARQ Processes |  |  | TDD: 8 FDD: 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Maximum HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  |  | 64QAM:6% 256QAM:3% |
| Test metric |  |  | SNR@70% max TP |