3GPP TSG-RAN WG4 Meeting # 108	R4-2313309

Toulouse, FR, August 21 – August 25, 2023

Agenda Item:	8.4.3.1.2

Source:	Ericsson

Title:	Simulation results for 8 Rx PDSCH requirements in FR1

Document for:	Information

# 1	Introduction

In this paper, we present the simulations results for 8Rx PDSCH in FR1. Based on last meeting discussions [1], we carried out simulations for PDSCH considering the agreed parameters. It is worth reminding that FDD simulations have been caried out considering same propagations environments and MCS levels agreed for TDD mode.

# 2	PDSCH Simulation Results

## 2.1 TDD Case: Remaining simulation results

According to the WF [1], we summarize the general parameters in the following tables.

| Assumptions | Value |
| --- | --- |
| Duplex mode | TDD |
| Carrier Frequency [GHz] | 3.5 GHz |
| Subcarrier Spacing [KHz] | 30 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLC (300 ns delay spread) |
| Antenna configuration | 2x8 ULA Medium B, 4x8 ULA Low, and 8x8 ULA Low |
| Doppler spread | 10 Hz and 100 Hz |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (64QAM) and 3% (256QAM) |
| I-Q Imbalance | None |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4 and Rank 8 |
| DMRS Configuration | 2+2 Type 1 DMRS symbols at (2,3,10,11) symbol indexes for Rank 8 1+1 Type 1 DMRS symbols at (2, 10) symbol indexes for Rank 2 and Rank 4 |
| MCS/TBS | From MCS Table 1 (TS38.214): 64QAM. From MCS Table 2 (TS38.214): 256QAM |
| NOH | 0 |
| Number of HARQ Processes | 8 |
| Maximum HARQ transmissions | 4 |

| Parameter |  | Value |
| --- | --- | --- |
| SCS (kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for Rank ≤ 4 2 for Rank > 4 |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions, with different channel models, different MCSs but up to 8 MIMO layer (=< Rank 8). | 1-1, 2-1, 2-2, 3-1, 3-2, |

In the sequel, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). Moreover, all figures have been provided in the Appendix.

Test cases for Rank 8:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for Test case 1-1 |  |  |  |  |  |  |  |  |

Observation 1: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 8 DL MIMO Layers in FR1 TDD using MCS17 under TDLA30-10 channel model. The ideal SNR value is at 19.3 dB while the proposed SNR requirement is at 22.1 dB which remains 2.3 dB below the maximum SNR level.

Test cases for Rank = 4:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 3.5 | 40 / 30 (106) | 64QAM, 0.43 MCS 17 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 8.77 |
| 2-2 | 3.5 | 40 / 30 (106) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 18.5 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for Test case 2-1 and Test case 2-2 |  |  |  |  |  |  |  |  |

Observation 2: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 4 DL MIMO Layers in FR1 TDD using MCS26 under TDLA30-10 channel model. The ideal SNR value is at 18.5 dB while the proposed SNR requirements is at 21.3 dB which remains 3.1 dB below the maximum SNR level.

Test cases for Rank = 2:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 Note 1 | 3.5 | 40 / 30 (106) | 64QAM, 0.50 MCS 19 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.72 |
| 3-2 Note 2 | 3.5 | 40 / 30 (106) | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 18.5 |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Note 2: MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for Test 3-1 Use Tx EVM 3% for Test 3-2. |  |  |  |  |  |  |  |  |

Observation 3: We can note that 8 Rx UE can support 2 DL MIMO Layers in FR1 TDD using MCS20 (256QAM) under TDLC300-100 with ULA Medium B propagation environment. The ideal SNR value is at 18.5 while the proposed SNR requirement is at 21.3 dB which remains 9.2 dB below the maximum SNR level.

## 2.2 FDD Case: Simulation results

For FDD duplex mode, we will use as general parameters:

| Assumptions | Value |
| --- | --- |
| Duplex mode | FDD |
| Carrier Frequency [GHz] | 2.6 GHz |
| Subcarrier Spacing [KHz] | 15 KHz |
| Waveform | CP-OFDM |
| CP Type | Normal CP |
| Channel Model | TDLA (30 ns delay spread) and TDLC (300 ns delay spread) |
| Antenna configuration | 2x8 ULA Medium B, 4x8 ULA Low, and 8x8 ULA Low |
| Doppler spread | 10 Hz and 100 Hz |
| PA Model | None |
| Pre-loaded Tx EVM | 6% (64QAM) and 3% (256QAM) |
| I-Q Imbalance | None |
| Channel Estimation | Realistic channel estimation |
| Transmission Rank | Rank 2, Rank 4 and Rank 8 |
| DMRS Configuration | 2+2 Type 1 DMRS symbols at (2,3,10,11) symbol indexes for Rank 8 1+1 Type 1 DMRS symbols at (2, 10) symbol indexes for Rank 2 and Rank 4 |
| MCS/TBS | From MCS Table 1 (TS38.214): 64QAM. From MCS Table 2 (TS38.214): 256QAM |
| NOH | 0 |
| Number of HARQ Processes | 4 |
| Maximum HARQ transmissions | 4 |

| Parameter |  | Value |
| --- | --- | --- |
| SCS (kHz)/Bandwidth (MHz) |  | 15/10 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for Rank ≤ 4 2 for Rank > 4 |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of peak Throughput | Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions, with different channel models, different MCSs but up to 8 MIMO layer (=< Rank 8). | 1-1, 2-1, 2-2, 3-1, 3-2, |

In the sequel, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). Moreover, all figures have been provided in the Appendix.

Test cases for Rank 8:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 2.6 | 10 / 15 (52) | 64QAM, 0.43 MCS 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18.86 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for Test case 1-1 |  |  |  |  |  |  |  |  |

Observation 4: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 8 DL MIMO Layers in FR1 FDD using MCS17 under TDLA30-10 channel model. The ideal SNR value is at 18.86 dB while the proposed SNR requirement is at 21.7 dB which remains 2.7 dB below the maximum SNR level.

Test cases for Rank = 4:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | 2.6 | 10 / 15 (52) | 64QAM, 0.43 MCS 17 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 8.94 |
| 2-2 | 2.6 | 10 / 15 (52) | 64QAM, 0.85 MCS 26 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 17.86 |
| MCS were defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Use Tx EVM 6% for Test case 2-1 and Test case 2-2 |  |  |  |  |  |  |  |  |

Observation 5: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 4 DL MIMO Layers in FR1 FDD using MCS26 under TDLA30-10 channel model. The ideal SNR value is at 17.86 dB while the proposed SNR requirements is at 20.7 dB which remains 3.7 dB below the maximum SNR level.

Test cases for Rank = 2:

| Test num. | fc (GHz) | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 Note 1 | 2.6 | 10 / 15 (52) | 64QAM, 0.50 MCS 19 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.52 |
| 3-2 Note 2 | 2.6 | 10 / 15 (52) | 256QAM, 0.67 MCS 20 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 17.95 |
| Note 1: MCS was defined based on TS 38.101-4 Table 5.1.3.1-1: MCS index table 1 for PDSCH (64QAM Table) Note 2: MCS were defined based on TS 38.101-4 Table 5.1.3.1-2: MCS index table 2 for PDSCH (256QAM Table) Use Tx EVM 6% for Test 3-1 Use Tx EVM 3% for Test 3-2. |  |  |  |  |  |  |  |  |

Observation 6: We can note that 8 Rx UE can support 2 DL MIMO Layers in FR1 FDD using MCS20 (256QAM) under TDLC300-100 with ULA Medium B propagation environment. The ideal SNR value is at 17.95 while the proposed SNR requirement is at 20.8 dB which remains 9.7 dB below the maximum SNR level.

# 3	Summary

In this paper, we provided the needed simulation results that should help in the definition of the PDSCH requirements. In the following, our observations that can be further discussed.

Observation 1: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 8 DL MIMO Layers in FR1 TDD using MCS17 under TDLA30-10 channel model. The ideal SNR value is at 19.3 dB while the proposed SNR requirement is at 22.1 dB which remains 2.3 dB below the maximum SNR level.

Observation 2: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 4 DL MIMO Layers in FR1 TDD using MCS26 under TDLA30-10 channel model. The ideal SNR value is at 18.5 dB while the proposed SNR requirements is at 21.3 dB which remains 3.1 dB below the maximum SNR level.

Observation 3: We can note that 8 Rx UE can support 2 DL MIMO Layers in FR1 TDD using MCS20 (256QAM) under TDLC300-100 with ULA Medium B propagation environment. The ideal SNR value is at 18.5 while the proposed SNR requirement is at 21.3 dB which remains 9.2 dB below the maximum SNR level.

Observation 4: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 8 DL MIMO Layers in FR1 FDD using MCS17 under TDLA30-10 channel model. The ideal SNR value is at 18.86 dB while the proposed SNR requirement is at 21.7 dB which remains 2.7 dB below the maximum SNR level.

Observation 5: Considering Tx EVM at 6%, we can note that 8 Rx UE can support 4 DL MIMO Layers in FR1 FDD using MCS26 under TDLA30-10 channel model. The ideal SNR value is at 17.86 dB while the proposed SNR requirements is at 20.7 dB which remains 3.7 dB below the maximum SNR level.

Observation 6: We can note that 8 Rx UE can support 2 DL MIMO Layers in FR1 FDD using MCS20 (256QAM) under TDLC300-100 with ULA Medium B propagation environment. The ideal SNR value is at 17.95 while the proposed SNR requirement is at 20.8 dB which remains 9.7 dB below the maximum SNR level.

# References

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei.

# Appendix

## A.1 Table 5.1.3.1-1: MCS index table 1 for PDSCH (TS 38.214)

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

## A.2 Table 5.1.3.1-2: MCS index table 2 for PDSCH (TS 38.214)

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

## A.3 PDSCH Simulations

Fig. 1 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 8 with up to 64QAM, using TDD 40 MHz/30 kHz (106 RBs) and ULA Low under TDLA30-10 channels.

(a) MCS17 in TDLA30-10 ULA Low                   (b) Up to MCS26 (Table 1)

Fig. 2 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 4 with up to 64QAM, using TDD 40 MHz/30 kHz (106 RBs) and ULA Low under TDLA30-10 channels.

Fig. 3 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 2 with MCS19 (Table 1), using TDD 40 MHz/30 kHz (106 RBs) and ULA Medium B, under TDLC300-100 channel conditions.

Fig. 4 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 2 with MCS13 (64QAM Table 2) and MCS20 (256QAM Table 2), respectively, using

TDD 40 MHz/30 kHz (106 RBs) and ULA Medium B, under TDLC300-100 channel conditions.

Fig. 5 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 2 with MCS19 (Table 1) and MCS20 (Table 2), using FDD 10 MHz/15 kHz (52 RBs) and ULA Medium B, under TDLC300-100 channel conditions.

Fig. 6 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 4 with MCS17 (Table 1) and MCS26 (Table 1), using FDD 10 MHz/15 kHz (52 RBs) and ULA Low, under TDLA30-10 channel conditions.

Fig. 7 - PDSCH simulation results depicting the PDSCH normalized throughput as function of SNR, considering Rank 8 with MCS17 (Table 1), using FDD 10 MHz/15 kHz (52 RBs) and ULA Low under TDLA30-10 channels.