3GPP TSG-RAN WG4 Meeting # 108	R4-2313314

Toulouse, FR, August 21 – August 25, 2023

Agenda Item:	8.4.3.1.4

Source:	Ericsson

Title:	Simulation results for CQI reporting for 8Rx in FR1

Document for:	Discussion

# 1	Introduction

In this paper, we present the simulation results for CQI reporting for 8Rx UE in FR1 TDD and FDD. Based on last meeting discussions [1] and the proposed simulation parameters for FDD mode, we carried out simulations for CQI reporting.

# 2	CQI Reporting Simulation Results in TDD mode

## 2.1 Simulation setup

As stated in WF [1], we will consider the following table throughout the simulations

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,0 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | 00000001 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-4 |  |  |  |

It is worth reminding Table A.4-3 in TS 38.101-4 given by

| TBS Scheme |  |  |  | TBS.3-1 | TBS.3-2 | TBS.3-3 | TBS.3-4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 3 | 4 | 3 | 4 |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |  |  |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |  |  |
| 1 | 0.2344 | 0 | QPSK | 4360 | 5896 | 8976 | 11784 |  |  |
| 2 | 0.3770 | 1 |  | 7048 | 9480 | 14344 | 18976 |  |  |
| 3 | 0.8770 | 3 |  | 16392 | 22032 | 33816 | 45096 |  |  |
| 4 | 1.4766 | 5 | 16QAM | 27656 | 36896 | 56368 | 75792 |  |  |
| 5 | 1.9141 | 7 |  | 35856 | 48168 | 73776 | 98376 |  |  |
| 6 | 2.4063 | 9 |  | 45096 | 60456 | 92200 | 122976 |  |  |
| 7 | 2.7305 | 11 | 64QAM | 51216 | 67584 | 104496 | 139376 |  |  |
| 8 | 3.3223 | 13 |  | 62504 | 81976 | 127080 | 167976 |  |  |
| 9 | 3.9023 | 15 |  | 73776 | 98376 | 147576 | 196776 |  |  |
| 10 | 4.5234 | 17 |  | 83976 | 112648 | 172176 | 229576 |  |  |
| 11 | 5.1152 | 19 |  | 96264 | 127080 | 196776 | 262376 |  |  |
| 12 | 5.5547 | 21 | 256QAM | 104496 | 139376 | 213176 | 278776 |  |  |
| 13 | 6.2266 | 23 |  | 116792 | 155776 | 237776 | 319784 |  |  |
| 14 | 6.9141 | 25 |  | 129128 | 172176 | 262376 | 352440 |  |  |
| 15 | 7.4063 | 27 |  | 139376 | 184424 | 278776 | 376896 |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 |  |  |  |  |  |  |  |  |  |

## 2.2 Simulation results

The following Table summarizes the results obtained when using EVM = 3%. However, and as it will be shown below, EVM values does not impact the CQI reporting requirements.

| SNR | Median CQI | Pr(Median CQI – 1 < CQI < Median CQI+1) [%] | BLER of Median CQI-1 [%] | BLER of Median CQI [%] | BLER of Median CQI+1 [%] |
| --- | --- | --- | --- | --- | --- |
| -6 | 2 | 100 | 0 | 63 | 100 |
| -5 | 2 | 100 | 0 | 0.1 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -3 | 3 | 100 | 0 | 100 | 100 |
| -2 | 3 | 100 | 0 | 33.2 | 100 |
| -1 | 3 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 1 | 4 | 100 | 0 | 92.6 | 100 |
| 2 | 4 | 100 | 0 | 8.6 | 100 |
| 3 | 5 | 100 | 0 | 94.4 | 100 |
| 4 | 5 | 100 | 0 | 0.3 | 100 |
| 5 | 6 | 100 | 0.1 | 93 | 100 |
| 6 | 6 | 100 | 0 | 0 | 100 |
| 7 | 7 | 100 | 0 | 77.9 | 100 |
| 8 | 7 | 100 | 0 | 0 | 100 |
| 9 | 8 | 100 | 0 | 19.9 | 100 |
| 10 | 9 | 100 | 0.1 | 100 | 100 |
| 11 | 9 | 100 | 0 | 0.6 | 100 |
| 12 | 10 | 100 | 0 | 100 | 100 |
| 13 | 10 | 100 | 0 | 11.7 | 100 |
| 14 | 10 | 100 | 0 | 100 | 100 |
| 15 | 11 | 100 | 0 | 86.1 | 100 |
| 16 | 12 | 100 | 0.1 | 100 | 100 |
| 17 | 12 | 100 | 0 | 0.9 | 100 |
| 18 | 13 | 100 | 0 | 100 | 100 |
| 19 | 13 | 100 | 0 | 15.3 | 100 |
| 20 | 14 | 100 | 0.7 | 100 | 100 |
| 21 | 14 | 100 | 0.1 | 12.6 | 100 |
| 22 | 15 | 100 | 0 | 100 | NaN |
| 23 | 15 | 100 | 0 | 17.6 | NaN |
| 24 | 15 | 100 | 0 | 0.1 | NaN |
| 25 | 15 | 100 | 0 | 0 | NaN |

Fig.1 - Simulation results depicting the median CQI as function of SNR for 8 Rx UE in FR1 TDD

using 30 kHz/40 MHz under Rank 4 and Tx EVM = 3%.

Observation 1: The results show that the lowest SNR to report efficiently a CQI with Rank 4 in FR1 TDD related to

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# 3	CQI Reporting Simulation Results in FDD mode

## 2.1 Simulation setup

As stated in WF [1], we will consider the following table throughout the simulations

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |  |
| Duplex Mode |  |  |  | FDD |  |  |  |
| SNR |  |  | dB | TBD | TBD | TBD | TBD |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | XP 4×8 with static channel specified in updated Annex B.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |  |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |  |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |  |  |
|  | CDM Type |  |  | FD-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 4,0 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |  |  |
|  | CSI-IM RE pattern |  |  | 0 |  |  |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |  |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 2 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| csi-ReportingBand |  |  |  | 1111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |  |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |  |  |  |
|  |  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |  |  |  |
|  |  | CodebookSubsetRestriction |  | 00000001 |  |  |  |
|  |  | RI Restriction |  | 00001000 |  |  |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.4-3, TBS.3-2 |  |  |  |

It is worth reminding Table A.4-3 in TS 38.101-4 given by

| TBS Scheme |  |  |  | TBS.3-1 | TBS.3-2 | TBS.3-3 | TBS.3-4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 52 | 52 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |  |  |
| Number of PDSCH MIMO layers |  |  |  | 3 | 4 | 3 | 4 |  |  |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |  |  |
| Available RE-s for PDSCH |  |  |  | 6240 | 6240 | 12720 | 12720 |  |  |
| CQI index | Spectral efficiency | MCS index | Modulation | Information Bit Payload per Slot |  |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |  |  |
| 1 | 0.2344 | 0 | QPSK | 4360 | 5896 | 8976 | 11784 |  |  |
| 2 | 0.3770 | 1 |  | 7048 | 9480 | 14344 | 18976 |  |  |
| 3 | 0.8770 | 3 |  | 16392 | 22032 | 33816 | 45096 |  |  |
| 4 | 1.4766 | 5 | 16QAM | 27656 | 36896 | 56368 | 75792 |  |  |
| 5 | 1.9141 | 7 |  | 35856 | 48168 | 73776 | 98376 |  |  |
| 6 | 2.4063 | 9 |  | 45096 | 60456 | 92200 | 122976 |  |  |
| 7 | 2.7305 | 11 | 64QAM | 51216 | 67584 | 104496 | 139376 |  |  |
| 8 | 3.3223 | 13 |  | 62504 | 81976 | 127080 | 167976 |  |  |
| 9 | 3.9023 | 15 |  | 73776 | 98376 | 147576 | 196776 |  |  |
| 10 | 4.5234 | 17 |  | 83976 | 112648 | 172176 | 229576 |  |  |
| 11 | 5.1152 | 19 |  | 96264 | 127080 | 196776 | 262376 |  |  |
| 12 | 5.5547 | 21 | 256QAM | 104496 | 139376 | 213176 | 278776 |  |  |
| 13 | 6.2266 | 23 |  | 116792 | 155776 | 237776 | 319784 |  |  |
| 14 | 6.9141 | 25 |  | 129128 | 172176 | 262376 | 352440 |  |  |
| 15 | 7.4063 | 27 |  | 139376 | 184424 | 278776 | 376896 |  |  |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is not scheduled on slots containing CSI-RS for tracking, CSI-RS for CSI acquisition and CSI-RS for beam refinement or slots which are not full DL Note 3: PDSCH is not scheduled on slots containing PBCH, i.e. slot#0 per 20ms periodicity Note 4: Spectral efficiency is based on MCS Table defined in Table 5.1.3.1-2 of TS 38.214 |  |  |  |  |  |  |  |  |  |

## 2.2 Simulation results

The following Table summarizes the results obtained when using EVM = 3%. However, and as it will be shown below, EVM values does not impact the CQI reporting requirements.

| SNR | Median CQI | Pr(Median CQI – 1 < CQI < Median CQI+1) [%] | BLER of Median CQI-1 [%] | BLER of Median CQI [%] | BLER of Median CQI+1 [%] |
| --- | --- | --- | --- | --- | --- |
| -6 | 2 | 100 | 0 | 94.4 | 100 |
| -5 | 2 | 100 | 0 | 1.8 | 100 |
| -4 | 2 | 100 | 0 | 0 | 100 |
| -3 | 3 | 100 | 0 | 100 | 100 |
| -2 | 3 | 100 | 0 | 34.1 | 100 |
| -1 | 3 | 100 | 0 | 0 | 100 |
| 0 | 3 | 100 | 0 | 0 | 100 |
| 1 | 4 | 100 | 0 | 94.7 | 100 |
| 2 | 4 | 100 | 0 | 15.8 | 100 |
| 3 | 5 | 100 | 0 | 85.5 | 100 |
| 4 | 5 | 100 | 0 | 0.4 | 100 |
| 5 | 6 | 100 | 0 | 89.5 | 100 |
| 6 | 6 | 100 | 0 | 0.4 | 100 |
| 7 | 7 | 100 | 0 | 72.3 | 100 |
| 8 | 7 | 100 | 0 | 100 | 100 |
| 9 | 8 | 100 | 0 | 17.7 | 100 |
| 10 | 9 | 100 | 0 | 100 | 100 |
| 11 | 9 | 100 | 0 | 46.6 | 100 |
| 12 | 10 | 100 | 0.1 | 100 | 100 |
| 13 | 10 | 100 | 0 | 12 | 100 |
| 14 | 10 | 100 | 0 | 100 | 100 |
| 15 | 11 | 100 | 0 | 37.6 | 100 |
| 16 | 12 | 100 | 0.1 | 100 | 100 |
| 17 | 12 | 100 | 0 | 2.1 | 100 |
| 18 | 13 | 100 | 0 | 100 | 100 |
| 19 | 13 | 100 | 0 | 12 | 100 |
| 20 | 14 | 100 | 0.7 | 100 | 100 |
| 21 | 14 | 100 | 0.1 | 24.5 | 100 |
| 22 | 15 | 100 | 0 | 99.5 | NaN |
| 23 | 15 | 100 | 0 | 3.5 | NaN |
| 24 | 15 | 100 | 0 | 0 | NaN |
| 25 | 15 | 100 | 0 | 0 | NaN |

Fig.2 - Simulation results depicting the median CQI as function of SNR for 8 Rx UE in FR1 FDD

using 15 kHz/10 MHz under Rank 4 and Tx EVM = 3%.

Observation 2: The results show that the lowest SNR to report efficiently a CQI with Rank 4 in FR1 FDD related to

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# 4	Summary

In this paper, we provided simulation results that should help steering the discussions towards an agreement on Test points for CQI reporting.

Observation 1: The results show that the lowest SNR to report efficiently a CQI with Rank 4 in FR1 TDD related to

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

Observation 2: The results show that the lowest SNR to report efficiently a CQI with Rank 4 in FR1 FDD related to

- 64QAM is at SNR = 7/8 dB (CQI 7).

- 16QAM is at SNR = 1/2 dB (CQI 4).

# References

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei.