3GPP TSG-RAN WG4 Meeting # 108	R4-2313316

Toulouse, FR, August 21 – August 25, 2023

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	PDSCH demodulation requirements for FR2 multi-Rx chain

Document for:	Discussion

# 1	Introduction

Last meeting #107, several related general aspect parameters have been agreed. In this meeting, we continue discussing the open issue on PDSCH performance requirements. Below, we summarize the PDSCH related open issues as stated in [1].

Issue 2-1-1: Channel model Agreement - At least consider TDLA30-75 for 100 MHz/120 kHz - FFS TDLA30-300 for 100 MHz/120 kHz. Issue 2-1-2: PTRS Port for sDCI schemes - Proposals - Option 1: One PTRS port per TRP for sDCI schemes - Option 2: One PTRS port for sDCI SDM based scheme. - Recommended WF: Encourage comments if any Issue 2-1-3: PTRS Port for mDCI schemes Agreement - One PTRS port per TRP for mDCI schemes Issue 2-1-4: Time offset between TRPs Agreement - Use the time offset between the two TRPs transmission points as {0.25us, -0.0625us}. - Further discuss which time offset to be used with certain configurations. Issue 2-1-5: Frequency offset between TRPs Agreement - Assume 600Hz frequency offset between TRPs Issue 2-1-6: PDSCH rate matching in mTRP transmission - Proposals - Option 1: PT-RS allocation does not overlap with PDSCH allocation per TRP. - Option 2: PT-RS allocation does not overlap with any PDSCH allocation. - Recommended WF: Encourage comments if any Issue 2-1-7: Sending an LS to RAN1 for specification clarification on PTRS rate matching behaviour for mTRP transmission (if ambiguous) - Proposals - Option 1: Yes - Recommended WF: Encourage comments if any Issue 2-1-8: Test cases and simulation parameters for sDCI SDM - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for sDCI based SDM scheme as presented in Tables 1 and 2. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any Issue 2-1-9: Test cases and simulation parameters for mDCI fully overlapping. - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based SDM scheme as presented in Tables 3 and 4. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any Issue 2-1-10: Test cases and simulation parameters for mDCI non-overlapping - Proposals - Option 1: Consider PDSCH test cases and simulation parameter for mDCI based non-overlapping scheme as presented in Tables 5 and 6. Tables can be found in R4-2308945. - Recommended WF: Encourage comments if any

# 2	PDSCH Demodulation Requirements

## 2.1 PTRS

NR operations in FR2-1 band are susceptible to phase noise (PN) effects that should be assessed and addressed. In WF [1], we agreed to use Rel-15 pattern with K = 2 and L = 1. As per WF [1], we agreed to consider one PTRS port per TRP from DCI schemes, while sDCI case remains open for further discussions.

In TS. 38.306, it was shown that the capability parameter ‘supportTwoPortDL-PTRS-r16’ is optional, as

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportTwoPortDL-PTRS-r16 Indicates whether UE supports 2-port DL PT-RS. UE supports this feature should indicate support singleDCI-SDM-scheme-r16 for the band. | Band | No | N/A | N/A |

Consequently, we can consider only one PTRS port for sDCI SDM based scheme. This is captured in Proposal 2 as follow.

Proposal 1: Considering that the capability parameter ‘supportTwoPortDL-PTRS-r16’ is not mandatory (optional), we can consider only one PTRS port for sDCI SDM based scheme.

About the PDSCH rate matching issue, stated as Issue 2-1-6 above, we believe that it is more judicious to consider a PTRS allocation that does not overlap with any PDSCH allocation for mTRP transmissions.

Proposal 2:

Consider PTRS allocation that does not overlap with any PDSCH allocation for sDCI and mDCI cases.

## 2.2 PDSCH demodulation parameters

To save efforts, it is preferrable to limit our requirements to PDSCH Type A for all agreed schemes: sDCI based SDM scheme, mDCI with full overlapping resource allocation scheme and mDCI with non-overlapping resource allocation scheme.

Furthermore, we need to specify the maximum allowable testable DL SNR for this WI. In Observation 1, we provide TR 38.884 Table 7.1-2 that summarizes the maximum achievable SNR [3].

Observation 1: Consider the maximum achievable SNR for the demodulation requirements given by

Table 2.2-1: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

|  | CBW (MHz) | Max SNR (TR38.810) | Max SNR (n262) |
| --- | --- | --- | --- |
| Single band UE | 100 | [19.7 dB] | [15.2] |
|  | 200 | [16.7 dB] | [12.2] |
| Multi-band UE (NOTEs 1,2) | 100 | [17.7 dB] | [14.2] |
|  | 200 | [14.6 dB] | [11.2] |
| NOTE 1: For ∑MBp from TS 38.101-2 Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands |  |  |  |

Hereafter, we summarize the needed parameters to define the PDSCH requirements in the following proposals.

Proposal 3: Consider PDSCH test cases for sDCI based SDM scheme using UE independent 2x2 processing, as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 | 1002 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlappling |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 and test 1-3 0.25 for test 1-2 and test 1-4 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 and test 1-3 0 for test 1-2 and test 1-4 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Minimum performance

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and gantenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 Note 4 | R.PDSCH.5-3.2 TDD | 100/120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| 1-2 Note 4 | R.PDSCH.5-3.2 TDD | 100/120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| 1-3 Note 5 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| 1-4 Note 5 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 5: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |

Proposal 4: Consider PDSCH test cases for mDCI with full overlapping resource allocation scheme using UE joint processing, as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 Note 4 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2, XP Low | 70 | TBD |
| 1-2 Note 4 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP Medium | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

Proposal 5: Consider PDSCH test cases for mDCI with non-overlapping scheme with UE independent 2x2 processing, as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 Note 4 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP High | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

# 3	Summary

In this paper we tried to contribute with the following proposals for further discussions:

Proposal 1: Considering that the capability parameter ‘supportTwoPortDL-PTRS-r16’ is not mandatory (optional), we can consider only one PTRS port for sDCI SDM based scheme.

Proposal 2:

Consider PTRS allocation that does not overlap with any PDSCH allocation for sDCI and mDCI cases.

Observation 1: Consider the maximum achievable SNR for the demodulation requirements given by

Table 2.2-1: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

|  | CBW (MHz) | Max SNR (TR38.810) | Max SNR (n262) |
| --- | --- | --- | --- |
| Single band UE | 100 | [19.7 dB] | [15.2] |
|  | 200 | [16.7 dB] | [12.2] |
| Multi-band UE (NOTEs 1,2) | 100 | [17.7 dB] | [14.2] |
|  | 200 | [14.6 dB] | [11.2] |
| NOTE 1: For ∑MBp from TS 38.101-2 Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands |  |  |  |

Proposal 3: Consider PDSCH test cases for sDCI based SDM scheme using UE independent 2x2 processing, as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 | 1002 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlappling |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 and test 1-3 0.25 for test 1-2 and test 1-4 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 and test 1-3 0 for test 1-2 and test 1-4 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Minimum performance

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition (Note 1) | Correlation matrix and gantenna configuration (Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) (Note 3) |
| 1-1 Note 4 | R.PDSCH.5-3.2 TDD | 100/120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| 1-2 Note 4 | R.PDSCH.5-3.2 TDD | 100/120 | 64QAM, 0.43 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| 1-3 Note 5 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| 1-4 Note 5 | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP Low | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 5: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |

Proposal 4: Consider PDSCH test cases for mDCI with full overlapping resource allocation scheme using UE joint processing, as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 Note 4 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-300 | 2x2, XP Low | 70 | TBD |
| 1-2 Note 4 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100 / 120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP Medium | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

Proposal 5: Consider PDSCH test cases for mDCI with non-overlapping scheme with UE independent 2x2 processing, as follow

Test Parameters

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Minimum performance

| Test num. | Reference channel |  | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition(Note 1) | Correlation matrix and antenna configuration(Note 2) | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB)(Note 3) |
|  | TRxP #1 | TRxP #2 |  |  |  |  |  |  |  |
| 1-1 Note 4 | R.PDSCH.5-2.2 TDD | R.PDSCH.5-2.2 TDD | 100/120 | 16QAM, 0.48 | FR2.120-1 | TDLA30-75 | 2x2, XP High | 70 | TBD |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

# References

- R4-2309824, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm Inc.

- 3GPP TS 38.101-4, “NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”, v.17.9.0.

- 3GPP TR 38.884, “Study on enhanced test methods for Frequency Range 2 (FR2) NR User Equipment (UE)”, v.18.2.0.