3GPP TSG-RAN WG4 Meeting # 108	R4-2313317

Toulouse, FR, August 21 – August 25, 2023

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	Simulation results for PDSCH of FR2 Multi-Rx UE

Document for:	Information

# 1	Introduction

In this paper, we present the initial simulations results for FR2 Multi-Rx PDSCH. Based on last meeting discussions [1], we carried out simulations for PDSCH considering the general agreed parameters and channel model.

# 2	PDSCH Simulation Results

Consider the baseline correlation model assuming the crosstalk between the TRPs and UE Rx antenna as below

R16×16=Eh16×1h16×1H=11+ρ∙diag1  ρ  ρ  1⊗RMIMO

=11+ρ∙RMIMOρRMIMOρRMIMORMIMO,

where h16×1=vec(H4x4) is the re-arranged channel values and

- ⊗ denotes the Kronecker product, diag denotes the diagonal matrix.

- RMIMO is the 4×4 correlation matrix for each link between TRPs to UE panels generated with parameters α (Tx correlation coefficient), β (Rx correlation coefficient), and γ (cross-polarization correlation coefficient) as stated in 38.101-4 B2.3.2

- ρ is the crosstalk power = [-15, -12, -9, -6] dB.

Furthermore, we consider the following parameters for the correlation matrices given by

XP Low:       = 0,  = 0, γ = 0.125

XP Medium:   = 0.3,  = 0.6, γ = 0.125

XP High:      = 0.9,  = 0.9, γ = 0.125

Observation 1: Consider the correlation matrices parameters as follow

XP Low:       = 0,  = 0, γ = 0.125

XP Medium:   = 0.3,  = 0.6, γ = 0.125

XP High:      = 0.9,  = 0.9, γ = 0.125

In the sequel, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). Furthermore, it is worth mentioning that we will refer to the SNR requirement without considering the phase noise (PN) effects as SNR0, while we will refer to the SNR requirement when considering PN effects but no compensation to it as SNR1.

Observation 2: Consider PN effects at 30 GHz and provide SNR requirements

- without PN effects, referred to as SNR0 and

- with PN effects but no compensation, referred to as SNR1.

Observation 3: Consider Tx EVM at 6% since we are considering up to 64QAM modulation.

Furthermore, we need to specify the maximum allowable testable DL SNR for this WI. In Observation 4, we provide TR 38.884 Table 7.1-2 that summarizes the maximum achievable SNR [2].

Observation 4: Consider the maximum achievable SNR for the demodulation requirements given by

Table 2-1: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

|  | CBW (MHz) | Max SNR (TR38.810) | Max SNR (n262) |
| --- | --- | --- | --- |
| Single band UE | 100 | [19.7 dB] | [15.2] |
|  | 200 | [16.7 dB] | [12.2] |
| Multi-band UE (NOTEs 1,2) | 100 | [17.7 dB] | [14.2] |
|  | 200 | [14.6 dB] | [11.2] |
| NOTE 1: For ∑MBp from TS 38.101-2 [2] Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands |  |  |  |

## 2.1 Single-DCI based SDM scheme

According to the WF [1], we summarize the general parameters for sDCI based SDM scheme in the following table.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 |  |
|  |  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | 1000 | 1002 |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full-overlappling |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 for test 1-1, test 1-3 and test 1-5 0.25 for test 1-2, test 1-4 and test 1-6 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1, test 1-3 and test 1-5 0 for test 1-2, test 1-4 and test 1-6 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 2: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of the peak Throughput | Verify the PDSCH mapping Type A performance for FR2 multi-Rx chain UE using sDCI based SDM scheme. Two Rx chain, each using 2 Rx antennas, receiving up to (2+2) MIMO layers (=< Rank 4). | 1-1, 1-2 1-3, 1-4 1-5, 1-6 |

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition (Note 1) | Correlation matrix and antenna config (Note 2) | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fracti- on of max TP (%) | The crosstalk power ρ (dB) | SNR0 with no PN (dB) (Note 3) | SNR1 with PN – No PN comp (dB) (Note 3) |
| 1-1 Note 4 | 100/120 | 64QAM, 0.43 | 1+1 | TDLA30-75 | 2x2, XP Low | 70 | -15 | 12.7 | 12.9 |
|  |  |  |  |  |  |  | -12 | 13.0 | 13.2 |
|  |  |  |  |  |  |  | -9 | 13.5 | 13.7 |
|  |  |  |  |  |  |  | -6 | 14.3 | 14.5 |
| 1-2 Note 4 | 100/120 | 64QAM, 0.43 | 1+1 | TDLA30-75 | 2x2, XP Low | 70 | -15 | 12.5 | 12.7 |
|  |  |  |  |  |  |  | -12 | 12.8 | 13.0 |
|  |  |  |  |  |  |  | -9 | 13.3 | 13.4 |
|  |  |  |  |  |  |  | -6 | 14.2 | 14.3 |
| 1-3 Note 5 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-300 | 2x2, XP Medium | 70 | -15 | 16.6 | 16.7 |
|  |  |  |  |  |  |  | -12 | 16.9 | 17.1 |
|  |  |  |  |  |  |  | -9 | 17.6 | 17.7 |
|  |  |  |  |  |  |  | -6 | 18.7 | 18.9 |
| 1-4 Note 5 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-300 | 2x2, XP Medium | 70 | -15 | 17.5 | 17.8 |
|  |  |  |  |  |  |  | -12 | 17.9 | 18.2 |
|  |  |  |  |  |  |  | -9 | 18.6 | 18.7 |
|  |  |  |  |  |  |  | -6 | 19.6 | 19.8 |
| 1-5 Note 5 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-75 | 2x2, XP Low | 70 | -15 | 16.0 | 16.2 |
|  |  |  |  |  |  |  | -12 | 16.3 | 16.5 |
|  |  |  |  |  |  |  | -9 | 16.9 | 17.1 |
|  |  |  |  |  |  |  | -6 | 18.1 | 18.2 |
| 1-6 Note 5 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-75 | 2x2, XP Low | 70 | -15 | 16.6 | 16.8 |
|  |  |  |  |  |  |  | -12 | 17.0 | 17.2 |
|  |  |  |  |  |  |  | -9 | 17.6 | 17.8 |
|  |  |  |  |  |  |  | -6 | 18.7 | 18.9 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) Note 5: PDSCH transmission is done from both TRxPs (PDSCH Layers 0 and 1 are transmitted from TRxP #1 and PDSCH layers 2 and 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

Observation 5: Considering UE independent processing for sDCI SDM based scheme considering up to 64QAM using either 1+1 layers or 2+2 layers, PN effects did not exceed 0.2 dB.

Observation 6: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), sDCI SDM based scheme can consider MCS17 (64QAM CR = 0.43) for 1+1-layer combination under TDLA30-75 and low cross polarization under RF impairments (PN, TO (-0.0625 or 0.25 us) and FO (600 Hz)) and crosstalk power of up to -6 dB.

Observation 7: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), sDCI SDM based scheme and by applying joint 4x4 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-300 and medium cross polarization under RF impairments (PN, TO (-0.0625 us) and FO (600 Hz)) and crosstalk power ρ = - 12 dB. However, the results under TDLA30-75 and Low cross polarization should be more adequate.

## 2.2 Multi-DCI with full-overlapping resource allocation

According to the WF [1], we summarize the general parameters for mDCI with full-overlapping resource allocation scheme in the following table.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Full overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | Us | -0.0625 for test 1-1 0.25 for test 1-2 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 for test 1-1 0 for test 1-2 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of the peak Throughput | Verify the PDSCH mapping Type A performance for FR2 multi-Rx chain UE using mDCI with full overlapping resource allocation scheme. Two Rx chain, each using 2 Rx antennas, receiving (2+2) MIMO layers. | 1-1, 1-2 1-3, 1-4 |

Independent 2x2 processing

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition (Note 1) | Correlation matrix and antenna config (Note 2) | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fracti- on of max TP (%) | The crosstalk power ρ (dB) | SNR0 with no PN (dB) (Note 3) | SNR1 with PN – No comp (dB) (Note 3) |
| 1-1 Note 4 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-300 | 2x2, XP Low | 70 | -15 | 17.5 | 17.8 |
|  |  |  |  |  |  |  | -12 | 17.5 | 17.7 |
|  |  |  |  |  |  |  | -9 | 19.3 | 19.7 |
| 1-2 Note 4 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-75 | 2x2, XP Medium | 70 | -15 | 16.6 | 17.1 |
|  |  |  |  |  |  |  | -12 | 17.2 | 17.4 |
|  |  |  |  |  |  |  | -9 | 19.1 | 19.4 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

Observation 8: Considering UE independent processing for mDCI with full overlapping resource allocation scheme considering 16QAM using 2+2-layer combination, PN effects did not exceed 0.2 dB.

Observation 9: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), mDCI with full overlapping resource allocation scheme and by applying independent 2x2 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-75 and medium cross polarization under RF impairments (PN, TO (0.25 us) and No FO (0 Hz)) and crosstalk power ρ = - 15 dB.

Joint 4x4 processing

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition (Note 1) | Correlation matrix and antenna config (Note 2) | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fracti- on of max TP (%) | The crosstalk power ρ (dB) | SNR0 with no PN (dB) (Note 3) | SNR1 with PN – No comp (dB) (Note 3) |
| 1-3 Note 4 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-300 | 2x2, XP Low | 70 | -15 | 16.6 | 16.7 |
|  |  |  |  |  |  |  | -12 | 17.0 | 17.1 |
|  |  |  |  |  |  |  | -9 | 17.5 | 17.7 |
|  |  |  |  |  |  |  | -6 | 18.6 | 18.8 |
| 1-4 Note 4 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-75 | 2x2, XP Medium | 70 | -15 | 16.7 | 16.9 |
|  |  |  |  |  |  |  | -12 | 17.0 | 17.2 |
|  |  |  |  |  |  |  | -9 | 17.6 | 17.8 |
|  |  |  |  |  |  |  | -6 | 18.8 | 19.0 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] with scaling factor as 1/sqrt(2) for transmitted signal from each TRxP Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

Observation 10: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), mDCI with full overlapping resource allocation scheme and by applying joint 4x4 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-75 and medium cross polarization under RF impairments (PN, TO (-0.0625 or 0.25 us) and FO (600 Hz)) and crosstalk power ρ = - 12 dB.

## 2.3 Multi-DCI with non-overlapping resource allocation

According to the WF [1], we summarize the general parameters for mDCI with non-overlapping resource allocation scheme in the following table.

| Parameter |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  |  | TRxP #1 |  |
| PDCCH configuration |  | TCI state |  | TCI State #1 | TCI State #2 |
|  |  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  |  | TDD |  |
| Active DL BWP index |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  | Type A |  |
|  | k0 |  |  | 0 |  |
|  | Starting symbol (S) |  |  | 2 |  |
|  | Length (L) |  |  | 12 |  |
|  | PRB bundling type |  |  | Static |  |
|  | PRB bundling size |  |  | 2 |  |
|  | Resource allocation type |  |  | Type 1 |  |
|  | RBG size |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  | {1000,1001} | {1002,1003} |
|  | TCI state |  |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |  |
| Resource allocation |  |  |  | Non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  | Hz | 600 |  |
| Precoding configuration |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |  |

Summary of test cases:

| Metric | Purpose | Test index |
| --- | --- | --- |
| 70% of the peak Throughput | Verify the PDSCH mapping Type A performance for FR2 multi-Rx chain UE using mDCI with non-overlapping resource allocation scheme. Two Rx chain, each using 2 Rx antennas, receiving (2+2) MIMO layers. | 1-1 |

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition (Note 1) | Correlation matrix and antenna config (Note 2) | Reference value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fracti- on of max TP (%) | The crosstalk power ρ (dB) | SNR0 with no PN (dB) (Note 3) | SNR1 with PN – No comp (dB) (Note 3) |
| 1-1 Note 4 | 100/120 | 16QAM, 0.48 | 2+2 | TDLA30-75 | 2x2, XP High | 70 | -15 | 12.9 | 13.1 |
|  |  |  |  |  |  |  | -12 | 13.2 | 13.4 |
|  |  |  |  |  |  |  | -9 | 13.7 | 13.9 |
|  |  |  |  |  |  |  | -6 | 14.6 | 14.8 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix and antenna configuration parameters apply to each of TRxP #1 and TRxP #2 Note 3: SNR corresponds to SNR of TRxP #1 and TRxP #2 as defined in 4.4.2 [2] Note 4: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 and Layer 1 are transmitted from TRxP #1 and PDSCH layer 2 and Layer 3 are transmitted from TRxP #2) |  |  |  |  |  |  |  |  |  |

Observation 11: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), mDCI with non-overlapping resource allocation scheme and by applying independent 2x2 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-75 and high cross polarization under RF impairments (PN, TO (-0.0625 us) and FO (600 Hz)) and crosstalk power ρ up to -6 dB.

# 3	Summary

In this paper, we provided the needed simulation results that should help in the definition of the PDSCH requirements. In the following, our observations that can be further discussed.

Observation 1: Consider the correlation matrices parameters as follow

XP Low:       = 0,  = 0, γ = 0.125

XP Medium:   = 0.3,  = 0.6, γ = 0.125

XP High:      = 0.9,  = 0.9, γ = 0.125

Observation 2: Consider PN effects at 30 GHz and provide SNR requirements

- without PN effects, referred to as SNR0 and

- with PN effects but no compensation, referred to as SNR1.

Observation 3: Consider Tx EVM at 6% since we are considering up to 64QAM modulation.

Observation 4: Consider the maximum achievable SNR for the demodulation requirements given by

Table 2-1: Comparison of maximum SNR between TR38.810 and preliminary extension to band n262

|  | CBW (MHz) | Max SNR (TR38.810) | Max SNR (n262) |
| --- | --- | --- | --- |
| Single band UE | 100 | [19.7 dB] | [15.2] |
|  | 200 | [16.7 dB] | [12.2] |
| Multi-band UE (NOTEs 1,2) | 100 | [17.7 dB] | [14.2] |
|  | 200 | [14.6 dB] | [11.2] |
| NOTE 1: For ∑MBp from TS 38.101-2 [2] Table 6.2.1.3-4 allows up to 2 dB in Rel-15 NOTE 2: From Rel-16 and later ∑MBp can be rounded up to 1.0 dB for all bands |  |  |  |

Observation 5: Considering UE independent processing for sDCI SDM based scheme considering up to 64QAM using either 1+1 layers or 2+2 layers, PN effects did not exceed 0.2 dB.

Observation 6: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), sDCI SDM based scheme can consider MCS17 (64QAM CR = 0.43) for 1+1-layer combination under TDLA30-75 and low cross polarization under RF impairments (PN, TO (-0.0625 or 0.25 us) and FO (600 Hz)) and crosstalk power of up to -6 dB.

Observation 7: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), sDCI SDM based scheme and by applying joint 4x4 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-300 and medium cross polarization under RF impairments (PN, TO (-0.0625 us) and FO (600 Hz)) and crosstalk power ρ = - 12 dB. However, the results under TDLA30-75 and Low cross polarization should be more adequate.

Observation 8: Considering UE independent processing for mDCI with full overlapping resource allocation scheme considering 16QAM using 2+2-layer combination, PN effects did not exceed 0.2 dB.

Observation 9: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), mDCI with full overlapping resource allocation scheme and by applying independent 2x2 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-75 and medium cross polarization under RF impairments (PN, TO (0.25 us) and No FO (0 Hz)) and crosstalk power ρ = - 15 dB.

Observation 10: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), mDCI with full overlapping resource allocation scheme and by applying joint 4x4 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-75 and medium cross polarization under RF impairments (PN, TO (-0.0625 or 0.25 us) and FO (600 Hz)) and crosstalk power ρ = - 12 dB.

Observation 11: Referring to the maximum achievable SNR given by Observation 4 (19.7 dB), mDCI with non-overlapping resource allocation scheme and by applying independent 2x2 processing, can consider MCS13 (16QAM CR = 0.48) for 2+2-layer combination under TDLA30-75 and high cross polarization under RF impairments (PN, TO (-0.0625 us) and FO (600 Hz)) and crosstalk power ρ up to -6 dB.

# References

- R4-2309824, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, Qualcomm Inc.

- 3GPP TR 38.884, “Study on enhanced test methods for Frequency Range 2 (FR2) NR User Equipment (UE)”, v.18.2.0.

# Appendix

## A.1 Table 5.1.3.1-1: MCS index table 1 for PDSCH (TS 38.214)

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |