3GPP TSG-RAN WG4#108	R4-2313352

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.1.1.2

Source: 	Qualcomm France

Title: 	MSD for CA_n71A-n85A

Document for:	Approval

# Introduction

Analysis and proposals on CA_n71A-n85A MSD with 35MHz n71 UL are provided in this contribution.

# WF from RAN4#107

R4-2310308 has the following WF.

<WF1 applicable band n71 cross band MSD due to n85 UL>

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

<WF2 applicable band n85 cross band MSD due to n71 UL applicable to UEs not supporting optional maximum symmetrical CBW >

- Applicable to legacy UEs or UEs supporting optional symmetrical UL/DL CBW for band n71 but not the maximum one

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

<WF3 band n85 cross band MSD due to n71 UL applicable to UEs supporting optional maximum symmetrical CBW to be evaluated for RAN4#108>

- Applicable to UEs supporting the maximum optional symmetrical UL/DL CBW in band n71

- If max CBW is 35MHz

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | TBDX | ACLR1 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

- If max CBW is 30MHz

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 683 | 30 | 15 | 20 (Rbstart=140) | 730.5 | 5 | TBDX | ACLR2 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

In RAN#100, RP-231010 agreed to add 35MHz BW for n71, in which case the following test point must be specified:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | TBDX | ACLR1 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

# Considerations

MSD analysis was conducted for 35MHz cases as 35MHz should be chose according to the WF.

We used the conventional 3GPP assumptions for the TX for the PA noise measurements, as well as in factoring the CIM3/CIM5 contributions.

In case of 35 MHz the IM3 hits the victim DL, dominating the desense.

- 20 MHz 100RB DFT-s-OFDM

- 30dBc ACLR

- 4dB Post-PA loss

- Image -28dB

- LO -28dBc

- CIM3 -60dBc

- CIM5 -70dBc

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 21.5X | ACLR1 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

# Conclusion

MSD analysis was conducted for CA_n71A-n85A with 35MHz n71 UL.

Proposal 1: Use the following analysis results when discussing requirements for CA_n71A-n85A

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 21.5X | ACLR1 |
| NOTE X: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

# Reference