3GPP TSG-RAN WG4 Meeting #108 	R4-2313356

Toulouse, France, August 21-25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures

Agenda item:	5.2.3

Document for:	Approval

Introduction

RAN4 has been discussing the requirements related to SCell activation using tci-ActivatedConfig for a couple of meetings.

The issue at hand is that tci-ActivatedConfig has not been accounted fully into the RAN4 requirements. Exactly how to capture such requirements and when the tci-ActivatedConfig is applicable was left open.

In this paper we discuss the applicability issue of tci-ActivatdConfig and in our CR [1] we have a proposal how to capture the requirements.

Discussion

tci-ActivatedConfig

tci-ActivatedConfig is introduced in Rel-17 by RAN2. It may be configured together with ServingCellConfig [2]:

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

The detailed parameter description [2]:

where

Where:

More specifically, looking at the parts that would be of interest for defining the UE requirements:

tci-ActivatedConfig

- If configured for an SCell, or

- if configured for the PSCell when the SCG is being activated upon the reception of the containing message,

  - the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell.

- If configured for the PSCell when the SCG is indicated as deactivated in the containing message:

- the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD.

When this field is absent for the PSCell and the SCG is being deactivated:

- the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

tci-ActivatedConfig applicability

As we see from the RAN2 description, the tci-ActvtedConfig can be configured for the Direct activated PSCell only.

tci-ActvtedConfig can be configured for the Direct activated PSCell only.

However, there is no such limitation for SCell. The tci-ActivatedConfig can be configured for a deactivated SCell as well as for an SCell being configured as activated upon configuration (direct activated).

tci-ActivatedConfig can be configured for a deactivated SCell and direct activated SCell.

In [1] we have provided a CR capturing the UE requirements accounting tci-ActivatedConfig.

Conclusion

In this paper we discussed the applicability issue of tci-ActivatdConfig and observe:

- tci-ActvtedConfig can be configured for the Direct activated PSCell only.

- tci-ActivatedConfig can be configured for a deactivated SCell and direct activated SCell.

In CR [1] we have a proposal how to capture the requirements.

- References

- R4-2313357, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- 38.331.