3GPP TSG-RAN WG4 #108	R4-2313373

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.30.1.4

Source: 	Qualcomm France

Title: 	Considerations on CA_n5-n105

Document for:	Approval

# Introduction

Considerations on CA_n5-n105 are provided in this contribution.

# Analysis

Compared to previous meeting, we did analysis more analysis for the Two Triplexers RF architecture option. Accounting the fact that n105 is still a very new band, we did update the assumptions a bit after more studies.

The following assumptions in the analysis.

- Cross band TX isolation at n5 RX ≥50dB

- Cross band RX isolation at n5 RX ≥45dB

- Cross band TX rejection at n5 RX ≥42dB

- Cross band TX attenuation at n5 DRX≥40dB

- Cross band TX isolation at n105 RX ≥40dB

- Cross band RX isolation at n105 RX ≥45dB

- Cross band TX rejection at n105 RX ≥40dB

- Cross band TX attenuation at n105 DRX≥40dB

The frequency arrangements were as follows.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | TBD | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | TBD | >ACLR2 |

MSD for n5 DL:

MSD for n105 DL:

# Conclusions

MSD analysis for CA_n5-n105 was provided with the following results.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n105 | n5 | 693 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 1.7 | >ACLR2 |
| n5 | n105 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.9 | >ACLR2 |

# Reference

- RP-230789, “Study on enhancement for sub-1GHz NR band combinations”, Spark NZ, Huawei, HiSilicon