3GPP TSG-RAN WG4 #108	R4-2313378

Toulouse, France, August 21st – 25th, 2023

Agenda item:	8.3.2.1

Source: 	Qualcomm France

Title: 	Considerations on CA_n5-n28-n105

Document for:	Discussion

# Introduction

Considerations on CA_n5-n28-n105 are provided in this contribution.

# SI description

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | DL_n5A-n105A_UL_n5A_BCS0 (New) DL_n5A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n28A-n105A | - | DL_n28A-n105A_UL_n28A_BCS0 (New) DL_n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n26A-n28A | CA_n26A-n28A | DL_n26A-n28A_UL_n28A_BCS0 (Completed in RAN4#106) DL_n26A-n28A_UL_n26A_BCS0 (New) | Telstra |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A | DL_n5A-n28A_UL_n5A-n28A_BCS0 (Ongoing in a separate WI) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n5A-n105A | DL_n5A-n105A_UL_n5A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | single UL in either n28 or n105 or n5 | DL_n28A-n105A_UL_n28A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |

Table 2: Bandwidth combination set for each band combination

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n28A-n105A | - | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

  - The following aspects need be studied

    - UE RF architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna and multiple low band narrowband antennas.

    - Evaluate the MSD requirements if needed.

    - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators.

  - Power class 3 (PC3) is considered in this study

- Identify potential impacts to relevant RAN4 requirements.

# Discussion

Now that the study is completed for CA_n5A-n105A and CA_n28A-n105A it is possible to consider superset combination CA_n5A-n28A-n105A.

The frequency ranges for the superset of this CA combination under study is shown below for illustration.

Figure 1 CA_n5A-n28A-n105A bands and frequency ranges

Frequency-wise there are many similarities to recently completed case CA_n8A-n20A-n28A. Two bands (8, 20 and 28, 105) are very close to each other and one band (8, 5) as a bit further away.

Reference architecture:

Three-antenna reference architecture seems to be best fit for this case:

- n28-n105 MPX

- n5 DPX

- n5-n28-n105 DRX

Notably, in this particular case UL’s are never transmitted from same antenna in UL CA as in CA_n28A-n105A it was agreed to use 1UL only.

Table 1 Three-antenna BW ratios

| Antenna | F_Low | F_High | BW Ratio |
| --- | --- | --- | --- |
| Main TRX | 612 | 803 | 27% |
| Diversity TRX | 824 | 894 | 8.1% |
| Diversity RX | 612 | 894 | 37.5% |

Antenna BW ratios for Main TRX and Diversity RX are very high.

Other options exist as well, such as three-antenna with the following configuration:

- n105 DPX

- n5-n28 MPX

- n5-n28-n105 DRX

Table 1 Three-antenna BW ratios

| Antenna | F_Low | F_High | BW Ratio |
| --- | --- | --- | --- |
| Main TRX | 612 | 703 | 13.8% |
| Diversity TRX | 703 | 894 | 23.9% |
| Diversity RX | 612 | 894 | 37.5% |

Antenna BW ratios for Diversity TRX and Diversity RX are very high.

Four-antenna architecture would, from pure antenna point of view be a good one, however due to practical constraints and so on, we do not propose to use that to specify the requirements.

Observation 1: Three-antenna architecture should be used as a baseline to derive requirements

Observation 2: Antenna BW rations for three-antenna cases are high, but still 3 antenna is preferred over 4 antenna when deriving 3GPP specifications

Cross-band MSD:

Given that 3GPP allows to use any feasible implementation, the cross-band analysis done for 2 band cases can be re-used for 3-band case.

Observation 3: Cross-band analysis done for 2-band cases are sufficient, and no new cross-band analysis are needed for 3-band case

Intermodulations:

IMD3 from n5+n28 UL CA is landing on top of n105DL. The exact impact is to be analysed, but here a lot from CA_n8A-n20A-n28A can be leveraged where similar MSD into non-UL band was in the order of 24..25dB

Observation 4: IMD3 MSD for n105DL must be studied, with assumption that MSD would be close to 25dB

# Conclusions

Considerations for CA_n5A-n28A-n105A were provided with the following observations

Observation 1: Three-antenna architecture should be used as a baseline to derive requirements

Observation 2: Antenna BW rations for three-antenna cases are high, but still 3 antenna is preferred over 4 antenna when deriving 3GPP specifications

Observation 3: Cross-band analysis done for 2-band cases are sufficient, and no new cross-band analysis are needed for 3-band case

Observation 4: IMD3 MSD for n105DL must be studied, with assumption that MSD would be close to 25dB

# Reference