3GPP TSG-RAN WG4 #108	R4-2313380

Toulouse, France, August 21st – 25th, 2023

Agenda item:	7.1.1.1

Source: 	Qualcomm France

Title: 	MSD for UL CA_n5B

Document for:	Approval

# Introduction

Analysis results for CA_n5B are presented in this contribution

# Discussion

UL CA_n5B with max 25MHz aggregated UL BW was recently requested. We did analyze MSD for two different BW combinations, 10MHz+15MHz and 20MHz+5MHz. The following results should be considered when specifying MSD.

Proposal 1: Consider the following MSD evaluations when specifying MSD for UL CA_n5B

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 15MHz | 25 (RBstart = 0) | 20 (RBstart = 59) | 31.6 | 29.8 | FDD |
| CA_n5B | 15/15 | 20MHz + 5MHz | 20 (RBstart = 0) | 25 (RBstart = 0) | 23.6 | 34.8 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

Along with discussing the MSD values, the group should also agree which test point(s) to specify. Earlier, for existing Intra-band contiguous UL CA combinations (CA_n5B, CA_n7B) only one test point is specified. Now that there is BCS0 for CA_n5B with max 20MHz aggregated BW, we cannot remove the existing 10+10MHz case. However, we should strive to specify only one test point for 25MHz aggregated case, if possible.

Proposal 2: RAN4 should consider if CA_n5B with max 25MHz aggregated BW can be specified using one test-point

# Conclusions

Analysis results and considerations on UL CA_n5B were provided with the following proposals.

Proposal 1: Consider the following MSD evaluations when specifying MSD for UL CA_n5B

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 15MHz | 25 (RBstart = 0) | 20 (RBstart = 59) | 31.6 | 29.8 | FDD |
| CA_n5B | 15/15 | 20MHz + 5MHz | 20 (RBstart = 0) | 25 (RBstart = 0) | 23.6 | 34.8 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

Proposal 2: RAN4 should consider if CA_n5B with max 25MHz aggregated BW can be specified using one test-point

# Reference