3GPP TSG-RAN WG4 Meeting #108	R4-2313400

# Toulouse, France, August 21 – 25, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	NR_pos_enh-Core: On per-FR measurement gaps for positioning

Agenda item:	5.2.3

Document for:	Discussion

# 1 	Introduction

At RAN4 #107, in regard to NR positioning enhancements, the following issue raised in [1] was discussed and kept open [2].

### Sub-topic 7-1: MG related proposal - Proposal (OPPO): Proposal 1: Gap patterns #24 and #25 could be configured as per FR type provided UE supports independentGapConfigPRS-r17. Proposal 2: Positioning frequency layers in FR1 should be measured within FR1 gap, and positioning frequency layers in FR2 should be measured within FR2 gap. Proposal 3: If concurrent gaps are configured, - All positioning frequency layers are associated with only one of the measurement gaps, or - All positioning frequency layers in each FR are associated with only one of the per FR measurement gaps in the same FR. - Recommended WF - TBA

This contribution depicts our view on the above issue.

# 2 	Discussion

The above three proposals are discussed in the three subsections below.

## 2.1 Use of MG patterns #24 and #25 for per-FR measurement gaps

The support of per-FR gap for PRS measurements was agreed at RAN4 #106bis-e and an LS sent to RAN2 [3].

1. Overall Description: RAN4 thanks RAN2 for the questions in LS R2-2213350. Based on discussions in RAN4#106-bis-e, RAN4 would like to confirm that PRS to be associated with per FR measurement gap in case of concurrent gaps can be supported from RAN4 perspective, provided that UE supports independentGapConfigPRS-r17. RAN4 respectfully asks RAN2 to take the above information into account.

In the framework of concurrent measurement gaps, support of PRS measurements using per-FR measurement gap has been agreed. This allows to configure per-FR measurement gap in FR1 or FR2 or in both in order to perform PRS measurements, while the use of MG patterns #24 and #25 has been restricted to per-UE MG patterns in Rel-16 and is precluded for per-FR MG. Proposal 1 in section 1 intends to remove this restriction. For the case that UE supports per FR measurement gaps (independent measurement gaps), NW may hence configure MG patterns #1 to #25. In our view, this proposal is reasonable.

For instance, assuming a UE is configured with 1 per-UE MG pattern MG1 (e.g. #24) used for RRM measurements and PRS measurements, but PRS measurements are only performed in FR1. Then MG1 needs also to be used in FR2. A gap length of 10 ms in FR2 cannot be avoided, even if RRM measurements can be performed with a shorter MG. Thus, in case MG1 (e.g. #24 if defined) is selected as per-FR measurement gap being used for PRS and RRM measurements in FR1, and MG2 (e.g. #13) is selected as per-FR gap being used for RRM measurements in FR2, gap length can be shorter in FR2 in case of independent gaps.

Also, in case of concurrent independent measurement gaps in an FR, one per-FR measurement gap is assigned for PRS measurements, where MG patterns #24 and #25 should be made available.

Thus, the restriction of MG patterns #24 and #25 to be used for per-UE MG patterns only should be removed.

RAN4 to remove the restriction for using MG patterns #24 and #25 for per-UE MG only, from Rel-17 for UE supporting independent measurement gaps.

Required changes in TS 38.133, clause 9.1.2 are depicted below.

### 9.1.2 Measurement gap If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply the network must provide a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers. If the UE requires measurement gaps to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply the network must provide either per-FR measurement gap patterns for frequency range where UE requires per-FR measurement gap for concurrent monitoring of all frequency layers of each frequency range independently, or a single per-UE measurement gap pattern for concurrent monitoring of all frequency layers of all frequency ranges. If the UE is configured via LPP [34] to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference measurement and PRS-RSRPP measurement defined in TS 38.215 [4], in order for the requirements in clauses 9.9.2, 9.9.3, 9.9.4 and 9.9.6 to apply, the network must provide - a single per-UE measurement gap pattern for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency and/or inter-RAT frequency layers of all frequency ranges, or - if UE supports independent measurement gap patterns for different frequency ranges for PRS measurement, per-FR measurement gap pattern for the frequency range for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency cells and/or inter-RAT frequency layers in the corresponding frequency range.

And for Table 9.1.2-2 and Table 9.1.2-3, following changes are needed.

Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE Measurement gap pattern configuration; Serving cell; Measurement PurposeNote 5; Applicable Gap Pattern Id. Per-UE; E-UTRA + FR1, or; non-NR RAT Note1,2; 0,1,2,3. Measurement gap; E-UTRA + FR2, or E-UTRA + FR1 + FR2; FR1 and/or FR2 Note 6,7; 0-11, 24, 25. non-NR RATNote1,2 and FR1 and/or FR2 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10, 24. E-UTRA and, FR1 if configured; non-NR RAT Note1,2; 0,1,2,3. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 only Note 6,7; 0-11,24,25. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR2 only Note 6,7; No gap. Per-FR; FR2 if configured; 12-25. measurement gap; E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR1 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 and FR2 Note 6,7; 0-11,24,25. FR2 if configured; 12-25. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR2 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; 12-25. E-UTRA and, FR1 if configured; Non-NR RAT Note1,2 and FR1 and FR2 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; 12-25. Note: In E-UTRA-NR dual connectivity mode, if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitored, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. In NR-E-UTRA dual connectivity mode, if UTRA FDD inter-RAT frequency layer is configured to be monitored for SRVCC, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: In E-UTRA-NR dual connectivity mode, non-NR RAT includes E-UTRA, UTRA and/or GSM. In NR-E-UTRA dual connectivity mode, non-NR RAT means E-UTRA, and UTRA for SRVCC. NOTE 2: Void NOTE 3: When E-UTRA inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 4: For UE supporting supportedGapPattern-NRonly-NEDC or measGapPatterns-NRonly-ENDC-r16 but not supporting supportedGapPattern for the corresponding gap patterns among GP2-11, the corresponding gap patterns are not applicable to measurement of non-NR RATs as defined in NOTE 1. NOTE 5: Inclusion of positioning measurements: Measurement purpose which includes E-UTRA measurements includes also E-UTRA RSRP and E-UTRA RSRQ measurements for E-CID. NOTE 6: Measurement gap patterns #24 and #25 can be requested [2] only when the UE is configured at least with any of RSTD, UE Rx-Tx, or PRS-RSRP measurements requiring such gaps and can only be used during the corresponding positioning measurement period NOTE 7: Inclusion of positioning measurements for measurement gaps: Measurement purpose which includes any of FR1 and FR2 measurements includes also RSTD, UE Rx-Tx, and PRS-RSRP measurements..

Table 9.1.2-3: Applicability for Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA and NR-DC configuration) Measurement gap pattern configuration; Serving cell; Measurement Purpose NOTE 2; Applicable Gap Pattern Id. FR1 NOTE5, or FR1 + FR2; non-NR RAT NOTE3,6; 0,1,2,3. FR1 and/or FR2 NOTE 8,9; 0-11, 24, 25. non-NR RAT and FR1 and/or FR2 NOTE3,6,8,9; 0, 1, 2, 3, 4, 6, 7, 8,10, 24. Per-UE measurement; FR2 NOTE5; non-NR RAT only NOTE3,6; 0,1,2,3. gap; FR1 only NOTE 8.9; 0-11, 24, 25. FR1 and FR2 NOTE 8,9; 0-11, 24, 25. non-NR RAT and FR1 and/or FR2 NOTE3,6,8,9; 0, 1, 2, 3, 4, 6, 7, 8,10, 24. FR2 only NOTE 8,9; 12-25. FR1 if configured; non-NR RAT only; 0,1,2,3. FR2 if configured; NOTE3,6; No gap. FR1 if configured; FR1 only NOTE 8.9; 0-11,24,25. FR2 if configured; No gap. FR1 if configured; FR2 only NOTE 8.9; No gap. Per-FR; FR2 if configured; 12-25. measurement; FR1 if configured; non-NR RAT and; 0, 1, 2, 3, 4, 6, 7, 8,10,24. gap; FR2 if configured; FR1 NOTE3,6,8,9; No gap. FR1 if configured; FR1 and FR2 NOTE 8.9; 0-11, 24,25. FR2 if configured; 12-25. FR1 if configured; non-NR RAT and; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; FR2 NOTE3,6,8,9; 12-25. FR1 if configured; non-NR RAT and FR1 and FR2 NOTE3,6,8,9; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; 12-25. NOTE 1: When E-UTRA inter-RAT RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 2: Measurement purpose which includes E-UTRA measurements includes also inter-RAT E-UTRA RSRP and RSRQ measurements for E-CID; measurement purpose which includes E-UTRA measurements includes also E-UTRA RSRP and E-UTRA RSRQ measurements for E-CID. NOTE 3: Void NOTE4: If per-UE measurement gap is configured with MG timing advance of TMG ms, the measurement gap starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among all serving cells subframes. If per-FR measurement gap for FR1 is configured with MG timing advance of TMG ms, the measurement gap for FR1 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR1. If per-FR measurement gap for FR2 is configured with MG timing advance of TMG ms, the measurement gap for FR2 starts at time TMG ms advanced to the end of the latest subframe occurring immediately before the configured measurement gap among serving cells subframes in FR2. TMG is the MG timing advance value provided in mgta according to [2]. In determining the measurement gap starting point, UE shall use the DL timing of the latest subframe occurring immediately before the configured measurement gap among serving cells. NOTE 5: NR-DC in Rel-15 only includes the scenarios where all serving cells in MCG in FR1 and all serving cells in SCG in FR2. NOTE 6: In NR single carrier, NR CA, and NR-DC mode, non-NR RAT means E-UTRA, and UTRA for SRVCC. In NR single carrier, NR CA, and NR-DC mode, if UTRA FDD inter-RAT frequency layer is configured to be monitored for SRVCC, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 7: For UE only supporting supportedGapPattern-NRonly for any gap patterns among GP2-11, the corresponding gap patterns are not applicable to measurement of non-NR RATs as defined in NOTE 6. NOTE 8: Measurement gap patterns #24 and #25 can be requested [2] only when the UE is configured with any of RSTD, UE Rx-Tx, or PRS-RSRP measurements requiring such gaps and can only be used during the corresponding positioning measurement period. NOTE 9: Inclusion of positioning measurements for measurement gaps: Measurement purpose which includes any of FR1 and FR2 measurements includes also RSTD, UE Rx-Tx, and PRS-RSRP measurements..

Following proposal is made.

RAN4 to agree changes in clause 9.1.2 removing the restriction of selecting GP#24 or #25, in case of independent measurement gaps.

## 2.2	Assignment of PRS measurements in case of independent MGs

Proposal 2 in section 1 suggests, in case the NW configures PRS measurements for per-FR MG in FR1 and FR2, PFLs in FR1 should be assigned to per-FR gap in FR1 and PFLs in FR2 to per-FR gap in FR2 to ensure a unique non-ambiguous assignment of a PFL to a measurement gap. In fact, the same understanding applies for RRM measurements in FR1 and FR2 in case of independent measurement gaps (by MG type definition). Thus, in case of independent measurement gaps used for positioning, no further clarification is needed in our view, as PFLs in FR1 are not subject to be measured within a per-FR MG in FR2 and vice versa. Thus, even that Proposal 2 in section 1 is correct, no further clarification is needed in TS 38.133 on this matter.

Following observation is made.

No further clarification is needed in TS 38.133 for assignment of PRS measurements to a measurement gap in case of non-concurrent independent measurement gaps.

## 2.3 	Assignment of PRS measurements in case of concurrent independent MGs in same FR

Regarding Proposal 3 in section 1, in case of concurrent and independent MGs in the same FR, RAN4 has already dealt with this case, as TS 38.133, Table 9.1.8-1, Note 2 specifies that only one per-FR gap in each FR can be used for PRS measurements, in case per-UE measurement gap is not configured, as replicated below.

Table 9.1.8-1: The number of Gap Combination Configurations by UE supporting both concurrent measurement gap patterns and independent measurement gap patterns Gap Combination Configuration Id; The number of simultaneous configured measurement gap patterns. Per-FR1 measurement gap; Per-FR2 measurement gap; Per-UE measurement gap. 0; 2; 1; 0. 1; 1; 2; 0. 2; 0; 0; 2. 3Note 1; 1; 0; 1. 4Note 1; 0; 1; 1. 5Note 1; 1; 1; 1. 6; 2; 0; 0. 7; 0; 2; 0. Note 1: Gap Combination Configuration Id #3, #4, #5 will be only applied when the per-UE measurement gap is associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4]. Note 2: In Gap Combination Configuration Id #0, #1, #6, #7, one per-FR measurement gap in an FR can be associated to measure PRS for any RSTD, PRS-RSRP, and UE Rx-Tx time difference measurement defined in TS 38.215 [4] provided that UE supports independentGapConfigPRS-r17.

Following observation is made.

No further clarification is needed in TS 38.133 for assignment of PRS measurements to a measurement gap in case of concurrent independent measurement gaps.

# 3	Conclusions

In this contribution, we express our view regarding the open issue on NR positioning enhancements, raised in [1] and discussed in [2], in regard to selection of MG patterns for PRS measurement in case of independent measurement gaps.

The following proposals are made:

- RAN4 to remove the restriction for using MG patterns #24 and #25 for per-UE MG only, from Rel-17 for UE supporting independent measurement gaps.

- RAN4 to agree changes in clause 9.1.2 removing the restriction of selecting GP#24 or #25, in case of independent measurement gaps.

The following observations are made:

- No further clarification is needed in TS 38.133 for assignment of PRS measurements to a measurement gap in case of non-concurrent independent measurement gaps.

No further clarification is needed in TS 38.133 for assignment of PRS measurements to a measurement gap in case of concurrent independent measurement gaps.

For Proposal 2, the companion CR is submitted in [4].

References

- R4-2308460, “Discussion on maintenance Rel-17 positioning”, OPPO

- R4-2310090, “Ad-hoc minutes for [107][202] Maintenance_R17”, Moderator (Apple)

- R4-2306388, “Reply LS on support of per FR PRS gap”, RAN4

- R4-2313401, “NR_pos_enh-Core: Correction to MG patterns for independent measurement gaps”, Nokia, Nokia Shanghai Bell