3GPP TSG-RAN WG4 Meeting #108	R4-2313420

Toulouse, France, August 21 – August 25, 2023

Agenda Item:	8.6.3.3

Source:	MediaTek Inc.

Title: 	On beam correspondence test issues

Document for:	Discussion

- Introduction

In the past meeting, there have been some discussions on test issues for Beam correspondence in initial access [1,2] but no consensus has been reached. The following issues need to be further discussed.

Open issues: Issue 3-1-1: Beam lock function Issue 3-1-2: Holding RAR

In this contribution we discuss these two open issues and propose to leave them to RAN5.

- Discussion

2.1 Beam lock function

According to RAN5’s reply [3], although RAN5 has not started works on Rel-18 FR2 enhancements, it is confirmed that a new UBF-like test function in initial access or update the existing UBF test function to lock the beam during initial access, and it is also confirmed that testing EIRP in initial access requires new test procedure to establish and maintain maximum power/peak EIRP during initial access.

Observation 1: According to RAN5’s reply, for FR2 enhancement requirements, either new UBL-like test function is introduced, or the existing UBF test function is updated to lock the beam during initial access, and new test procedure is required as well to establish and maintain maximum power/peak EIRP during initial access.

With the confirmation from RAN5, RAN4 may assume beam lock function and there is no testability issue on polarization for beam correspondence requirements in initial access.

Proposal 1: RAN4 may assume beam lock function and no testability issue on polarization for beam correspondence requirements in initial access by leaving test design to RAN5.

2.2 Holding RAR

The main intension of holding RAR in the test is to allow UE under test to make power ramping up to its maximum output power, however, according to TS 38.213 [4], a UE may change its spatial domain transmission filter if failing to receive RAR within ra-ResponseWindow, and this will disable power ramping, thus holding RAR is not enough, and UE is not supposed to change its spatial domain transmission filter during the test.

If prior to a PRACH retransmission, a UE changes the spatial domain transmission filter, Layer 1 notifies higher layers to suspend the power ramping counter as described in [11, TS 38.321].

Observation 2: Both holding RAR and disabling change of spatial domain transmission filter are required to guarantee the UE to reach or maintain its maximum output power during the test.

How to achieve this could be left to RAN5 and does not impact directly on the minimum requirements.

Proposal 2: It is up to RAN5 to design tests for holding RAR and disabling change of spatial domain transmission filter, and it does not have direct impact on the minimum requirement.

- Conclusion

In this contribution we have the following observations and proposals for

Observation 1: According to RAN5’s reply, for FR2 enhancement requirements, either new UBL-like test function is introduced, or the existing UBF test function is updated to lock the beam during initial access, and new test procedure is required as well to establish and maintain maximum power/peak EIRP during initial access.

Observation 2: Both holding RAR and disabling change of spatial domain transmission filter are required to guarantee the UE to reach or maintain its maximum output power during the test.

Proposal 1: RAN4 may assume beam lock function and no testability issue on polarization for beam correspondence requirements in initial access by leaving test design to RAN5.

Proposal 2: It is up to RAN5 to design tests for holding RAR and disabling change of spatial domain transmission filter, and it does not have direct impact on the minimum requirement.

References

- R4-2306600, WF on beam correspondence in initial access and RRC_INACTIVE, Nokia

- R4-2310259, WF on beam correspondence in initial access and RRC_INACTIVE, Nokia

- R4-2304021, Response LS on testability for beam correspondence in initial access, Apple

- TS 38.213 v17.3.0