# 3GPP TSG-RAN WG4 Meeting # 108						R4-2313469

# Toulouse, France, 21 - 25 August , 2023

# Source:		Ericsson

Title:			Redcap UE supporting release independent frequency band

Agenda item:		7.25.2

Document for:		Approval

# Introduction

In this contribution, we present our view on the proposal for adding RedCap UE as release independent feature for adding bands for Rel-18 RedCap WI[1][2].

# Discussion

In updated WID [2], the 38.307 is added as impacted specification and intention is to add the RedCap UE as release independent feature for additional supported band in Rel-18. The proposal text is listed below and also a companion CR for it.

To add the RedCap UE as release independent feature, it is proposed in below table for the impact of  38.307.

| 38.307 clause | Proposed change | Note |
| --- | --- | --- |
| 3.3 | Adding abbreviation of “RedCap” |  |
| 5 | Adding new sub-clause 5.9 for Redcap operating bands |  |
| 6, 7,8 | No change | The scope of the WID[1][2] only for FR1 |
| Annex B | Adding B.4.14 |  |

Discuss and agree the above proposed change in 38.307.

# 5	Release independent features for NR frequency range 1

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BW	Bandwidth

CA	Carrier Aggregation

CC	Component carrier

DL	Downlink

EN-DC	Dual connectivity between E-UTRA and NR

FDD	Frequency Division Duplex

FR1	Frequency range 1

FR2	Frequency range 2

NR	New radio

RedCap	Reduced Capability

REL	Release

SDL	Supplementary downlink

SUL	Supplementary uplink

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

## 5.9	Additional operating bands for Redcap for NR frequency range 1

Requirements for a Rel-17 Redcap UE for additional operating bands within FR1 compared to TS 38.101-1 of Rel-17 [2] are introduced via this clause.

Table 5.9-1: NR RedCap UE within FR1

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| RedCap | TDD, FDD and HD-FDD | Rel-17 | Table B.4.14-1 |

## B.4.14	Common UE RF requirements for RedCap

The requirements and test cases listed in Table B.4.14-1 are specified in REL-17 version of TS 38.101-1 [2].

Table B.4.14-1: Common UE RF requirements for RedCap UE in FR1

| Clause | Description |
| --- | --- |
| 6.2I | Transmitter power for RedCap |
| 7.1I | General |
| 7.3I | Reference sensitivity for RedCap |

# Conclusions

In this contribution, we present our view on the RF specification impact with below proposals:

Proposal-1:Discuss and agree the above proposed change in 38.307.

# References

RP-230743, New WID on adding new NR FDD bands for RedCap (Reduced Capability) in Rel-18, Ericsson

RP-231049, updated WID on adding new NR FDD bands for RedCap (Reduced Capability) in Rel-18, Ericsson