TSG-RAN WG4 Meeting #108	R4- 2313485

Toulouse, France, 21 – 25 August 2023

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for NS_17 and NS_43U

Agenda Item:	7.22.2

Document for:	Agreement

# 1	Introduction

In [1] the PC2 A-MPR for NS_18 for band n28 and NS_43 for band n8 has been agreed. For these two bands, the remaining A-MPR work are NS_17 and NS_43U, respectively. In the sequel, we share our analysis and proposals.

# 2	Discussion

## 2.1 A-MPR for NS-17

##### 6.5.3.3.2	Requirement for network signalling value "NS_17"

When "NS_17" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-1: Additional requirements for "NS_17"

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10 |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |

The additional emission requirements as signalled by NS_17 [2] are duplicated below. It can be seen that the guard band is 8MHz in comparison to 5MHz for NS_18.

According to the dual filter assumption [3] used for PC3 A-MPR, 12 dB attenuation is assumed for frequencies below 710MHz. Based on the same assumption, our simulation shows that no A-MPR is needed for NS_17 for PC2.

Proposal 1: Assuming dual type duplexer for band n28A and n28B, no A-MPR is needed for NS_17 for PC2.

## 2.2 A-MPR for NS_43U

The network signalling of NS_43U combines the spectrum emission requirements indicated by NS_43 and NS_100. For NS_100, it indicates protection of adjacent UTRA channels as shown below [2].

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

And the A-MPR for NS_100 is defined as below [2].

Table 6.2.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |

Since a PC2 capable UE needs to meet more stringent NR ACLR requirement than PC3, it’s reasonable to expect that the existing A-MPR requirement for NS_100 is sufficient for PC2 as long as the same UTRA ACLR requirement indicated by NS_100 apply.

Proposal 2: Assuming the UTRA ACLR requirement indicated by NS_100 applies for PC2, the PC2 A-MPR for NS_43U can be set to max{PC2 A-MPR for NS_43, A-MPR for NS_100}.

# 4 Conclusions

Based on our analysis and simulation, the PC2 A-MPR for NS_17 and NS_43U can be defined as follows:

Proposal 1: Assuming dual type duplexer for band n28A and n28B, no A-MPR is needed for NS_17 for PC2.

Proposal 2: Assuming the UTRA ACLR requirement indicated by NS_100 applies for PC2, the PC2 A-MPR for NS_43U can be set to max{PC2 A-MPR for NS_43, A-MPR for NS_100}.

# References

[1] R4-2310245, WF on PC2 A-MPR for band n8 and n28, Huawei, HiSilicon, Apple, China Unicom, RAN4#107

[2] TS 38.101-1 v18.2.0

[3] R4-1813763 Band n28 AMPR, Qualcomm, RAN4#88bis